3GPP RAN WG4 Meeting #114bis	R4-2503408

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on RRM core requirements testing framework for positioning

Source: 	CATT

Agenda item:	7.19.4

Document for:	Discussion

Introduction

Impacts of AI/ML based positioning on RRM requirements were discussed in last meeting and the agreements were captured in WF [1]. In RAN#107, the scope of AI/ML based positioning enhancement was updated and the latest version including change marks is copied below [2]:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

According to the revised WID, case 2a and 2b are deleted and therefore RAN4 do not need to discuss related impacts on RRM requirements anymore. In this contribution, we will continue to discuss open issues summarized in [3], excluding case 2a/2b related issues, and analyse potential impacts on RRM core requirements.

Discussion

Scope of AI/ML positioning

As we mentioned in Introduction part, case 2a/2b are removed from the latest WID. Thus, RAN4 do not need to discuss related impacts on RRM requirements.

Proposal 1: RAN4 do not discussion case 2a/2b related issues in Rel-19.

Requirement for case 1

Whether to define prediction accuracy requirements for case 1 has been discussed for multiple meetings. Proposals for this issue are shown below:

Proposals - Option 1: RAN4 will not define positioning accuracy requirements for case 1(direct UE positioning) - Option 2: RAN4 to further study how to define the requirements, companies are invited to bring concrete proposals on a requirements framework and next steps on how to define - Option 3: others

The main issue is if it is feasible to define performance requirements like other performance requirements, i.e., is it possible to find a relation between the absolute position error and side conditions, e.g., SINR, RS bandwidth, etc. Based on our simulation observations, the position error spans from less than 1 meter to a few meters in different InF scenarios with generalization considered. If we consider more diverse scenarios, such as Uma/Umi, range of position error may be wider. It seems not feasible to define a general performance requirement that can apply for diverse scenarios. Therefore we prefer not to define requirements for case 1.

Proposal 2: RAN4 not to define performance requirements for case 1.

LOS/NLOS indicator

Proposals - Option 1: RAN4 to introduce requirements for LOS/NLOS indicator - Companies should bring detailed proposals on how to define requirements and potential tests - Option 2: It is not feasible to introduce requirements for this indicator - Option 3: Requirements are not needed - Option 4: Others

In regards with the requirements for LOS/NLOS, considering it is not dedicatedly introduced for AI/ML positioning and it is used s assistance information only. In addition, though statistic solutions were proposed by companies, they are more appropriate to be used for performance monitoring. For each LOS/NLOS indicator reporting, we do not see a strong demand to define requirements.

Proposal 3: RAN4 not to define requirements for LOS/NLOS indicator reporting.

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM requirements and following proposals are provided:

Proposal 1: RAN4 do not discussion case 2a/2b related issues in Rel-19.

Proposal 2: RAN4 not to define performance requirements for case 1.

Proposal 3: RAN4 not to define requirements for LOS/NLOS indicator reporting.

References

[1] R4-2502856, WF on the requirements for AI/ML air interface, Qualcomm, RAN4#114.

[2] RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107.

[3] R4-2500683, Topic summary for [114][131] NR_AIML_air_part1, Qualcomm, RAN4#114.