3GPP RAN WG4 Meeting #114bis	R4-2503412

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on impacts of AIML measurement event prediction on RRM requirements

Source: 	CATT

Agenda item:	7.21.2.2

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed the RRM impacts of AI/ML based mobility and the agreements were captured in the WF [1]. It was agreed to start the discussion on AIML measurement event prediction from RAN4#114bis. In this contribution, we will discuss the potential impacts and present our views.

Discussion

General and scope

Measurement event prediction use case

In last meeting, RAN4 agreed to discuss measurement event prediction use case from April meeting [1].

Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

According to RAN2 discussions, there are two types of measurement event prediction: direct and indirect prediction, differences of which are copied below [2]:

RAN2#127bis - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Furthermore, RAN2 has following agreements in RAN2#128 meeting, which further categorizes indirect measurement event prediction into four cases [3]:

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

Based on above RAN2 agreements, we will define measurement event prediction from direct and indirect prediction.

For direct measurement event prediction, RAN2 has agreed that the input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Therefore, from RAN4 perspective, the corresponding achievements on model input discussion can be reused as well as test configurations.

Regarding the model output, since what model outputs is the probability of event occurrence within a time window, a candidate test metric is successful prediction rate, e.g., 90%.

Observation 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

For indirect measurement event prediction, the model input and output, in our understanding, are the same as that in RRM measurement prediction, where the model output is RSRPs of serving/neighbouring cells which are intermediate outputs, and the final result is the probability of event occurrence within a time window, which is derived by UE without AI/ML models. Thus, for both direct and indirect measurement event prediction use cases, RAN4 can reuse the agreements of RRM measurement prediction.

However, test metric for indirect may need more discussion. Regarding the intermediate outputs, the RSRPs of serving/neighbouring cells may not be reported to network, i.e., these results are unknown to TE and therefore it is impossible to check the prediction accuracy. For the final result which is the probability of event occurrence within a time window, the necessity of defining a performance requirement needs justifying. Using successful prediction rate may be more appropriate.

Observation 2: The intermediate output, i.e., RSRP of serving/neighbouring cells, may be unknown to TE and therefore it is impossible to verify its accuracy.

Observation 3: Necessity of defining a performance requirement for a probability needs justifying for indirect measurement event prediction.

Proposal 1: No extra discussion is needed for model input-related aspects for direct and indirect measurement event prediction, including test configurations.

Proposal 2: Successful prediction rate can be used as a test metric for direct and indirect measurement event prediction.

Besides, measurement event prediction also impacts the conditional handover delay. In RAN2#129 meeting [4], RAN2 provides objectives for some measurement event predictions:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

According to both objectives, some components in current conditional handover delay requirements can be updated or removed, e.g., Tprocessing and/or Tmeasure. Details can be further discussed.

Proposal 3: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details.

Conclusions

This contribution discussed the impacts of AI/ML measurement event prediction on RRM requirements, and some observations and proposals are provided:

Observation 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

Observation 2: The intermediate output, i.e., RSRPs of serving/neighbouring cells, may be unknown to TE and therefore it is impossible to verify its accuracy.

Observation 3: Necessity of defining a performance requirement for a probability needs justifying for indirect measurement event prediction.

Proposal 1: No extra discussion is needed for model input-related aspects for direct and indirect measurement event prediction, including test configurations.

Proposal 2: Successful prediction rate can be used as a test metric for direct and indirect measurement event prediction.

Proposal 3: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details.

References

[1] R4-2502594, WF on AI/ML Mobility, Nokia, RAN4#114.

[2] R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis, RAN2#127bis.

[3] Report of 3GPP TSG RAN WG2 meeting #128, RAN2#128.

[4] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.