3GPP RAN WG4 Meeting #114bis	R4-2503414

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on UE to UE CLI handling

Source: 	CATT

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 made fruitful progress of the UE to UE CLI requirements in R19 SBFD WI. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the UE to UE CLI handling.

Discussion

Rx beam assumption

In regards to the assumption of Rx beam, RAN1 has made an agreement in RAN1#118bis and RAN4 agreed to reflect RAN1 agreements in RRM specification. But how RAN1 agreements are reflected is still undetermined.

RAN4#113: - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement.

Considering RAN1 will capture their agreements into RAN1 specification in a detailed way, we prefer that RAN4 uses a reference to RAN1 spec rather than capturing the same contents in RAN4 spec. In addition, since this issue is not a technical discussion anymore, we suggest solving it during CR drafting phase.

Proposal 1: RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements, which can be further discussed in CR drafting phase.

Measurement resources

In RAN4#114 meeting, the impacts of measurement resources on measurement requirements were discussed and some agreements were achieved with an open issue left:

- Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

For DUD case, it is possible that network configures one measurement resource in two discontinuous DL subbands. From network perspective, network has its reasonable motivations when configuring measurement resources in this way, which means that UE is expected to measure both DL subbands and report the results. However, there are different opinions on measurement mechanism when UEs have different capabilities, and the corresponding impacts on RRM requirements should also be analyzed. From the UE point of view, there are three potential measurement behaviors which are illustrated in Figure-1:

(1) Measure both DL subbands simultaneously

(2) Measure both DL subbands sequentially  				            (3) Measure only one DL subband

Figure-1 UE measurement behaviors for DUD case

Measurement behavior (1):

In our opinion, measurement behavior (1) is the default understanding when this case is considered. As discussed above, NW has its justified motivation to configure one measurement resource in both DL subbands. And therefore UE is expected to measure both DL subbands and provide the results.

Regarding the impacts on RRM requirements, the measurement period requirement can be defined based on the period of measurement resource, assuming there is no collision between measurement resources and other signals. And in this case, measurement accuracy requirements should be defined by considering both DL subbands.

Measurement behavior (2):

This measurement mechanism is not efficient from the usage of frequency resource perspective, especially when the measurement resources are configured for a specific UE. Though same principle of defining measurement accuracy requirement in (1) can be reused, performing measurements in such way will lead to a longer measurement time duration than that in (1).

Measurement behavior (3):

Another measurement behavior is that UE always measures only one subband. In this case, the measurement period requirement can be defined based on the period of measurement resource, which is the same as (1). However, this measurement mechanism has the following issues:

(i) Half of frequency resources are wasted, especially when measurement resources are UE-specific.

(ii) May cause misaligned understanding between network and UE, e.g., network does not know on which subband UE performs measurements.

(iii) Measurement accuracy requirements will be defined by considering only one DL subbands.

In summary, from the usage efficiency of frequency resources and measurement delay point of view, we support measurement behavior (1) for DUD case.

Proposal 2: For DUD case, UE measures both DL subbands simultaneously. Measurement period requirements are defined based on the period of measurement resource and measurement accuracy requirements are defined based on BW of both DL subbands.

Side conditions

Timing offset

The assumption of timing offset for L1 CLI measurements depends on specific scenarios and a LS was sent to RAN1 for clarification. In RAN1#119 meeting, the LS was discussed and the following agreement is sent back to RAN4 [2]:

RAN1 Agreement: Send reply LS to RAN4 with following information: When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

According to RAN1 agreement, intra-cell UE-to-UE L1 CLI measurement scenario is prioritized, and therefore the cell phase error can be removed from the Rel-16 SRS-RSRP side condition. From RAN4 perspective, measurement accuracy requirements can be defined without cell phase error, but the requirements apply for both intra-/inter-cell UE-to-UE L1 CLI measurements.

Proposal 3: RAN4 to define measurement accuracy requirements without cell phase error and the requirements apply for both intra-/inter-cell L1 CLI measurements.

SRS Es/Iot and BW

Based on the summary of simulation results [3], we observed that the cell phase error has the most significant impact on L1 SRS-RSRP measurement results. If RAN4 agrees to remove cell phase error, which can significantly reduce the measurement error, especially for large SCS configurations. And then selection of number of measurement samples and SRS Es/Iot will have a limited effect on measurement performance. For SRS Es/Iot, it is acceptable to reuse the Rel-16 value, -1 dB.

In regards to the assumption of number of PRBs of SRS resources for L1 CLI measurements, RAN4 needs to consider the sizes of UL subband and total channel bandwidth. For most of SCS and band combinations, it is reasonable to reuse the assumption of 48 PRBs. However, when SCS equals 60 kHz in FR1 bands, there might be some issues.

According to agreements from RF groups, the channel bandwidths supporting SBFD shall be equal to or larger than 40 MHz except for n39, and the size of UL subbands will be chosen and declared from the Table 5.3.2-1 in TS 38.104 [4]. Hence, the applicable sizes of channel bandwidth for 60 kHz SCS in FR1 bands are highlighted below:

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

If SRS-RSRP measurements apply when the size of SRS BW is not smaller than 48 PRBs, which means the size of UL subband shall not be smaller than 48 PRBs. Furthermore, if the size of DL subbands is also included, assuming it is DU case and the sizes of UL subband and DL subband are the same, at least 96 PRBs are needed. And when guardband is required, larger channel bandwidths should be configured. Based on the above analysis, it is clear that up to 3 options are applicable for channel bandwidths configured with 60 kHz SCS in FR1 bands for DU case, and DUD configuration is not applicable in this case. If a UL subband smaller than 48 PRBs is configured, configuring some small bandwidths and DUD case become applicable from the network perspective, however the SRS-RSRP measurement performance requirement will not apply any more.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

As we discussed above, if cell phase error is removed, SRS-RSRP measurement error will be effectively reduced and the benefit of using a large number of PRBs is limited. Therefore in order to make performance requirements applicable for more channel BW configurations with SCS 60 kHz, we slightly prefer 24 PRBs between two SRS BW assumptions, i.e., 24 and 48 PRBs.

Proposal 4: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot = 1 dB.

- SRS BW: 24 PRBs.

Report mapping

A LS [5] was sent from RAN1 to RAN4 to confirm the following RAN1 working assumption on L1 CLI measurement report mapping:

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value. … RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

From RAN4 perspective, we do not identify any issues to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

In addition, RAN1 agreed to support differential report for L1 CLI measurements, and therefore RAN4 should also define measurement report mapping requirements for differential reporting. According to the RAN1 design, a 4-bit value will be used to report differential results, which means there will be 16 possible entries at most. With 2 dB step size, the maximum range will be around 30 dB. We think the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI are reasonable, and a L1 SRS-RSRP report mapping table is provided below:

Table 1: Differential measurement report mapping for L1 SRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | -30≥ΔRSRP | dB |
| DIFFRSRP_1 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_2 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_3 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_4 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_5 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_6 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_7 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_8 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_9 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_10 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_11 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_12 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_13 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_14 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_15 | 0≥ΔRSRP>-2 | dB |

Proposal 5: It is reasonable to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 6: RAN4 to define measurement report mapping requirements for differential reporting for L1 CLI measurements based on the range and resolution designed by RAN1. An table is provided below:

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | -30≥ΔRSRP | dB |
| DIFFRSRP_1 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_2 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_3 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_4 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_5 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_6 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_7 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_8 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_9 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_10 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_11 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_12 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_13 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_14 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_15 | 0≥ΔRSRP>-2 | dB |

Scheduling restriction

On scheduling restriction for L1-CLI measurements, the following agreements were achieved in last meeting:

- Agreements (from Thu AH session) - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

One open issue for L1-SRS-RSRP measurement is if restricted symbols are needed after L1-SRS-RSRP measurement resource considering UL/DL transition. In our understanding, the UL/DL transition at UE is quite quick and less than the length of one symbol, and in some cases, UE can finish the transition in flexible symbols. Therefore we prefer not to have restriction on symbols after the symbol for measurement resource. Similarly, no additional symbols are needed for L1-CLI-RSSI Methods #1. For L1-CLI-RSSI Method #3, we think the scheduling restriction for Methods #1 can be reused. But we are open to further discuss it.

Proposal 7: For L1-SRS-RSRP measurement, prefer not to have restriction on symbols after the symbols for measurement resource.

Proposal 8: For L1-CLI-RSSI Methods #1, do not have additional restricted symbols. For L1-CLI-RSSI Methods #3, reuse the scheduling restriction for Methods #1.

Measurement restriction

Regarding the measurement restriction, RAN4 agreed to reuse R16 measurement restriction requirements in RAN4#113 with an open issue that was not solved in RAN4#114:

RAN4#113 Agreement: - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

And in RAN4#114, a detailed recommended WF for measurement restrictions for different L1 CLI measurement was provided:

RAN4#114 Recommended WF: - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements - For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, FFS whether to consider exception case - Option 1: do not consider exception case, i.e. measurement requirements for L1-CLI-RSSI do not apply - Option 2: consider exception case, i.e. measurement requirements for L1-CLI-RSSI apply when UE can perform L1-CLI-RSSI measurement and other measurement at same time. - For L1-CLI-RSSI Method #3, - If the measurement timing is same as L1-SRS-RSRP, as baseline, measurement restriction requirements are same as L1-SRS-RSRP. - If the measurement timing is same as L1-CLI-RSSI Method #1, as baseline, measurement restriction requirements are same as L1-CLI-RSSI Method #1. - FFS whether any adaptation is needed on top of baseline. - Recommended WF for collision between different CLI measurements - FFS whether to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. - FFS how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources - Option 1: prioritize L1-SRS-RSRP measurement - Option 2: prioritize L1-CLI-RSSI measurement - Option 3: other

For L1-CLI-RSSI Method #1, the open issue that whether the exception cases need to be considered need to be further discussed. In our understanding, the exception cases mean that UE can receive resources of different types of measurements at the same time. For example, UE could simultaneously receive multiple resources in FR1 or multiple resources QCLed with TypeD in FR2 if frequency configurations are appropriate. If UE supports to perform measurements in parallel, it can significantly reduce measurement delay. Therefore we slightly prefer to consider those exception cases.

Proposal 9: Prefer to consider the exception cases where UE can receive CLI resources and other resources at the same time, e.g., resources QCLed with TypeD in FR2.

For L1-CLI-RSSI Method #3, RAN4 agreed that the measurement timing of L1-CLI-RSSI Method #3 is the same as L1-CLI-RSSI Method #1. Therefore the measurement restriction requirements can also be the same.

Proposal 10: For L1-CLI-RSSI Method #3, the measurement restriction requirements are the same as L1-CLI-RSSI Method #1.

For the two open issues in collisions between different CLI measurements, we prefer to clarify in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. For the 2nd issue, considering that the purpose of CLI measurements is to evaluate the interference level to victim UE, i.e., network cares the channel quality of DL subbands of victim UE. Therefore, when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide, UE shall prioritize L1-CLI-RSSI measurement.

Proposal 11: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Requirement applicability

In order to facilitate the discussion on this issue, two sub-issues are provided for further discussion:

- Recommended WF - The proposals are close to each other, further discuss the following issues and come up with combined the wording - Whether we define L1 CLI measurement resources to be confined within UL or DL subband or just DL active BWP - Whether we define L1 CLI measurement as intra-frequency

Since RAN1 agreed to support CLI-RSSI measurements in UL subband in RAN1#119, then it is reasonable to confine the L1 CLI measurement resources within DL active BWP.

For the second sub-bullet, a definition of L1 CLI measurement similar to that of L1-RSRP measurement can be expected in RAN1 spec, TS 38.215, where the following description is included in SSB-based L1-RSRP definition:

“If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency.”

Hence, we prefer to clarify that the L1 CLI measurement is applicable for RRC_CONNECTED intra-frequency in RRM spec.

Proposal 12: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 CLI measurement resources are confined within BW of DL active BWP.

Spec structure

In order to clearly differentiate L1/L3 CLI measurement requirements, we prefer to use a new clause for CLI measurement of SBFD, i.e., 9.7X.X. And RAN4 try the best to use references as much as possible if the contents in L3 CLI requirements can be reused.

Proposal 13: RAN4 to define the L1 CLI measurement requirements in a new clause, i.e., 9.7X.X and try the best to use references if contents in L3 CLI measurement requirements can be reused.

Conclusions

This contribution discussed the impacts of L1 UE-to-UE CLI on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements, which can be further discussed in CR drafting phase.

Proposal 2: For DUD case, UE measures both DL subbands simultaneously. Measurement period requirements are defined based on the period of measurement resource and measurement accuracy requirements are defined based on BW of both DL subbands.

Proposal 3: RAN4 to define measurement accuracy requirements without cell phase error and the requirements apply for both intra-/inter-cell L1 CLI measurements.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

Proposal 4: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot = 1 dB.

- SRS BW: 24 PRBs.

Proposal 5: It is reasonable to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 6: RAN4 to define measurement report mapping requirements for differential reporting for L1 CLI measurements based on the range and resolution designed by RAN1. An table is provided below:

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | -30≥ΔRSRP | dB |
| DIFFRSRP_1 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_2 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_3 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_4 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_5 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_6 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_7 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_8 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_9 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_10 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_11 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_12 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_13 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_14 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_15 | 0≥ΔRSRP>-2 | dB |

Proposal 7: For L1-SRS-RSRP measurement, prefer not to have restriction on symbols after the symbols for measurement resource.

Proposal 8: For L1-CLI-RSSI Methods #1, do not have additional restricted symbols. For L1-CLI-RSSI Methods #3, reuse the scheduling restriction for Methods #1.

Proposal 9: Prefer to consider the exception cases where UE can receive CLI resources and other resources at the same time, e.g., resources QCLed with TypeD in FR2.

Proposal 10: For L1-CLI-RSSI Method #3, the measurement restriction requirements are the same as L1-CLI-RSSI Method #1.

Proposal 11: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Proposal 12: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 CLI measurement resources are confined within BW of DL active BWP.

Proposal 13: RAN4 to define the L1 CLI measurement requirements in a new clause, i.e., 9.7X.X and try the best to use references if contents in L3 CLI measurement requirements can be reused.

References

[1] R4-2502597, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple, RAN4#114.

[2] R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN1#119.

[3] R4-2420166, Summary of simulation results for L1-SRS-RSRP, Huawei, RAN4#113.

[4] TS 38.104, Base Station (BS) radio transmission and reception, V18.8.0.

[5] R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting, RAN1#120.