3GPP RAN WG4 Meeting #114bis	R4-2503415

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on impacts of SBFD on RRM requirements

Source: 	CATT

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 continued discussing the RRM requirements of R19 SBFD feature. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the impacts of SBFD on RRM requirements.

Discussion

CSI-RS L1 measurement – collision with UL

- Proposals - Proposal 1 (ZTE): - For the collision handling between L1 periodic/semi-persistent CSI-RS resource and semi-statically configured/dynamic UL transmission, wait for RAN1 reply. This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements - Proposal 2 (Samsung): - Dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

Regarding the collision handling between L1 CSI-RS measurement and UL transmission, a reply LS is sent back to RAN4 [2]. According to RAN1 reply, when semi-statically configured DL reception collides with dynamically scheduled UL transmission, dynamically scheduled UL transmission will be prioritized.

Proposal 1: For the collision handling between L1 periodic/semi-persistent CSI-RS resource and semi-statically configured/dynamic UL transmission, dynamically scheduled UL transmission shall be prioritized. And this is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements.

CSI-RS L1 measurement – puncturing in frequency domain

On this issue, some progresses were achieved in last meeting:

Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, depending on UE measurement capability - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

The 1st open issue is essentially the same as the issue in L1 CLI measurements. From network perspective, measurement behavior in option 2 will lead to a low usage efficiency of frequency resources. And doubling existing measurement period will make the new measurement period too long to be useful. Therefore we support option 1. But we are open to further discuss the impacts of UE capabilities on measurement period requirements, if necessary. Scaling the measurement period requirements by considering UE capabilities, e.g., UE CSI processing time, is one solution.

Proposal 2: For DUD case, the existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

For the accuracy requirements for DUD case, we think the existing accuracy requirements can directly apply for case 1 and case 2, considering that bandwidth of at least one DL subband is larger than the assumed BW in the existing accuracy requirements. For case 3 where X1 < X3 and X2 < X3, (X1 + X2) >= X3, RAN4 can use the existing accuracy requirements as starting point and introduce a margin to mitigate the impact of performance degradation due to discontinuous CSI-RS distribution in frequency domain.

Proposal 3: For measurement accuracy requirements for DUD case,

- For case 1 and case 2, the existing accuracy requirements apply.

- For case 3, using the existing accuracy requirements as starting point and introduce a margin to mitigate the impact of discontinuous CSI-RS distribution in frequency domain.

CSI-RS L1 measurement – different symbol types

A reply LS from RAN1 is sent back to RAN4 [2] to clarify the applicability of resource configuration and/or reporting configuration to CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The reply is as follows:

For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

Apparently, for CSI-RS based L1-RSRP/SINR measurement/reporting, the valid symbol type will be configured by network via signalling. Therefore UE follows the indication and measure the corresponding CSI-RS occasions to complete the requested measurements.

For CSI-RS based RLM/BFD/CBD measurements, RAN1 clarify that the configurations that indicate valid symbol type are not applicable. In our understanding, UE is expected to measure CSI-RS resources on both symbol types, if capable, to shorten the measurement delay.

Proposal 4: For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network.

Proposal 5: For CSI-RS based RLM/BFD/CBD measurements, UE is expected to measure CSI-RS resources on both symbol types, if capable, to shorten the measurement delay.

Requirements for CSI-RS-based L3 measurements

Regarding this issue, RAN4 achieved agreements for DU case and FFS on DUD case:

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

In DUD case where one CSI-RS resource is mapping into two discontinuous DL subbands, and a main problem is that the measurement performance will degrade if UE performs neighbour cell measurements in the legacy way. Therefore whether UE is aware of SBFD configuration in neighbour cells is the key issue. We notice that a LS [3] is sent to RAN2 by RAN3 to request RAN2 to define a IE to indicate the SBFD frequency and time configuration of neighbour cells which will be exchanged among gNBs. With this information, RAN4 can assume that UE is able to know the SBFD configuration in neighbour cells, and therefore UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Proposal 6: For CSI-RS-based L3 measurements, UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Anther open issue for both DU and DUD cases is the priority between CSI-RS measurement on SBFD symbols and UL transmission. The candidate options are summarized below:

- Recommended WF - Discuss priority between CSI-RS measurement on SBFD symbols and UL transmission - For intra-frequency, measurement is performed outside gaps, - Option 1: measurement is prioritized over UL (same as legacy scheduling restriction) - Option 2: UL is prioritized over measurement - Option 3: follow same principle as L1 CSI-RS measurement - Option 4: option 1 for neighbour cell and option 2 for serving cell - For inter-frequency, measurement is performed within gaps, and is prioritized over UL.

For inter-frequency measurement which needs gaps in current spec, it is straightforward that inter-frequency measurement should be prioritized over UL transmission.

For intra-frequency measurement, considering that the mobility is more important and the measurements for mobility is always prioritized in current spec, we support option 1, i.e., measurement is prioritized over UL (same as legacy scheduling restriction).

Proposal 7: For CSI-RS-based L3 measurements, measurement is prioritized over UL transmission.

Requirements for scheduling restriction

There are two sub-issues in this section: scheduling restriction requirements for UL/DL transition and for inter-frequency SSB measurement without gap. The recommended WF is reproduced below for reference:

- Recommended WF for UL/DL transition - Do not define new scheduling restriction requirements for DL/UL transition. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

Regarding the scheduling restriction requirements for DL/UL transition, we do not think it is necessary to define new requirements. The impacts of DL/UL transition can be covered by the existing L1/L3 scheduling restrictions.

Proposal 8: Do not define new scheduling restriction requirements for DL/UL transition.

For inter-frequency SSB measurement without gap, when the SSB is fully within DL active BWP but not completely contained in DL sub-band, UE is expected to receive SSB and perform measurements. Therefore UE should not be scheduled by network for UL transmission, i.e., UL scheduling restriction is needed. Since requirements for scheduling restriction for inter-frequency SSB measurement without gap have been defined in TS 38.133 clause 9.3.9, RAN4 can use these existing requirements as a starting point and check if any updates are required.

Proposal 9: RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: For the collision handling between L1 periodic/semi-persistent CSI-RS resource and semi-statically configured/dynamic UL transmission, dynamically scheduled UL transmission shall be prioritized. And this is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements.

Proposal 2: For DUD case, the existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

Proposal 3: For measurement accuracy requirements for DUD case,

- For case 1 and case 2, the existing accuracy requirements apply.

- For case 3, using the existing accuracy requirements as starting point and introduce a margin to mitigate the impact of discontinuous CSI-RS distribution in frequency domain.

Proposal 4: For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network.

Proposal 5: For CSI-RS based RLM/BFD/CBD measurements, UE is expected to measure CSI-RS resources on both symbol types, if capable, to shorten the measurement delay.

Proposal 6: For CSI-RS-based L3 measurements, UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Proposal 7: For CSI-RS-based L3 measurements, measurement is prioritized over UL transmission.

Proposal 8: Do not define new scheduling restriction requirements for DL/UL transition.

Proposal 9: RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point.

References

[1] R4-2502597, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple, RAN4#114.

[2] R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, RAN1#120.

[3] R3-250888, SBFD information exchange among gNBs for CLI mitigation, RAN3#127.