3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503418

Wuhan, CN, April 7th – April 11th, 2025

Title:	Discussion on performance requirements for FR2-1 L3 measurement delay reduction

Source:	CATT

Agenda Item:	7.17.4

Document for:	Discussion

1 Introduction

According to the time budget for this work item, RAN4 will start discussion on performance requirements of FR2-1 L3 measurement delay reduction from April meeting. Therefore we will present our views on how to proceed.

2 Discussion

In RAN4#111 meeting, RAN4 had the following agreements [1]:

Issue 2-1-5: measurement performance requirement when apply L3 measurement delay reduction by optimizing Rx BSF - Agreement: - RAN4 is not to change existing measurement performance requirement when consider optimization of Rx BSF in L3 measurement delay reduction. Issue 2-2-6: measurement performance requirement when apply L3 measurement delay reduction by optimizing CSSF - Agreement: - RAN4 is not to change existing measurement performance requirement when consider optimization of CSSF in L3 measurement delay reduction.

According to the agreements, the existing measurement performance requirement will be reused without any updates. Therefore RAN4 can directly go to test case discussion.

Observation 1: The existing measurement performance requirement will be reused without further discussion needed.

Regarding test tests, the legacy test cases can be used as starting points for both core and performance requirements. In order to limit the number of test cases, we choose some legacy test cases and they are listed in table 1, 2 and 3 [2].

In table 1, we list the existing test cases that are needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and FR2-1 L3 measurement delay reduction by optimizing CSSF. And in table 2 and 3, we list test cases that are needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and FR2-1 L3 measurement delay reduction by optimizing CSSF respectively.

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

Proposal 1: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and CSSF:

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Proposal 2: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF:

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Proposal 3: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing CSSF:

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

Regarding test cases for performance requirements, we also think the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements can be used as a starting point. FFS details.

Proposal 4: Use the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements as starting point. FFS details.

3 Conclusion

In this paper, we discussed performance requirements for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Observation 1: The existing measurement performance requirement will be reused without further discussion needed.

Proposal 1: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and CSSF:

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Proposal 2: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF:

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Proposal 3: Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing CSSF:

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

Proposal 4: Use the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements as starting point. FFS details.

4 References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN4#111.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V18.8.0.