3GPP RAN WG4 Meeting #114bis	R4-2503421

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on the impacts of low band CA via switching on RRM requirements

Source: 	CATT

Agenda item:	7.7.3

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 started to discuss the impacts of low band carrier aggregation via switching on RRM requirements. The achieved agreements are captured in [1]. In this contribution, we will further present our views on the open issues summarized in [2].

Discussion

Interruption requirement

In RAN4#114, RAN4 discussed this issue without agreements. Two alternatives are provided for further discussion:

- Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

Based on the discussion, it seems that companies have different understanding on interruption and/or switching delay. In some scenarios, the time duration of both requirements may be same or comparable. However, they are different for this low-low band CA case, since UE can work only on one band at one time, during which UE is not able to perform data transmission and/or reception in another band. To clearly illustrate our understanding on this issue, a figure is provided below:

Figure 1 Interruption and switching delay for low-low band CA via switching

In our understanding, the interruption should cover both switching delay and the time duration during which UE works on the other carrier. Regarding the switching delay, it is under discussion in RF group. And the time durations of UE working on different carriers depend on switching pattern designed by RAN1. Besides these two aspects, other factors, such as TAE, can also be considered.

In summary, the interruption for PCell CCs is:

X1=2*switching period+TSDL

Where

- switching period is decided by RF group;

- TSDL is the time duration during which UE works on SDL carrier.

The interruption for SDL CC is:

X2=2*switching period+TPCell

Where

- TPCell is the time duration during which UE works on PCell carriers.

Proposal 1: RAN4 to define interruption requirements and the interruption for PCell CCs (X1) is:

X1=2*switching period+TSDL

Where

- switching period is decided by RF group;

- TSDL is the time duration during which UE works on SDL carrier.

The interruption for SDL CC (X2) is:

X2=2*switching period+TPCell

Where

- TPCell is the time duration during which UE works on PCell carriers.

Other aspects can be further considered, e.g., TAE, if necessary.

DL/UL Timing

On this issue, RAN4 discussed the availability of SSB at UE in the last 160 ms in last meeting, and a LS [3] including applicability of UL transmit timing requirements was sent to RAN1 for information. In addition, there are requirements in TS 38.133 [4] to clarify the reference cell which can be used by UE to derive UL transmit timing:

- 7.1 UE transmit timing - 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

According to the highlighted requirements, UE can maintain it UL transmit timing based on the timing of PCell for this low-low band CA vie switching scenario, if UE can receive SSB in PCell in last 160 ms, which has been included in the LS and shared to RAN1. RAN4 can assume that UE’s DL and UL timing will not be impacted in this scenario.

Another solution is that UE is allowed to use SDL cell as reference cell which provide DL timing information. With the following assumption in the WID [5], we think this solution is feasible and does not require much work.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: … - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

Proposal 2: DL/UL timing requirements will not be impacted when either alternative is supported:

- Alt 1: Switching pattern designed by RAN1 can assure the availability of SSB at UE in last 160 ms in PCell.

- Alt 2: UE is allowed to use activated SCell in the pTAG as reference cell which provides DL timing information when there is only one DL cell at one time.

SCell Measurements

Deactivated SCell measurement

In current spec, deactivated SCell measurement is defined based on meaCycleSCell which is configured by higher layer signaling [4]:

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320 ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320 ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: The requirements also apply to deactivated SCG SCell..

However, when UE can measure the SDL cell also depends on switching pattern. In order to simplify the requirements for deactivated SCell measurement, one solution is to replace meaCycleSCell with TSwitchingPattern. But considering that the switching pattern is under design in RAN1 and we do not know the time duration of switching pattern yet. If the length of switching pattern is quite short, which means UE will frequently switch to SDL cell, then a problem with the above solution will be that UE will measure deactivated SCell very frequently. In this case, the TSwitchingPattern can be scaled by constant value(s) which depends on the exact configured length of switching pattern. The constant value(s) can be further discussed after RAN1 finish the design of switching pattern and RAN4 have full knowledge of switching pattern.

Proposal 3: Deactivated SCell measurement period requirements can be updated by replacing meaCycleSCell with TSwitchingPattern. Constant value(s), which need further discussion, can be used to scale the TSwitchingPattern.

SCell BFD/CBD measurement

For SCell BFD/CBD measurement, the evaluation period is derived based on period of reference signals [4]:

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 P) TSSB). DRX cycle ≤ 320 ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle > 320 ms; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length..

The impact of switching pattern can be discussed in two possible scenarios:

(1) Scenario 1: TEvaluate_BFD_SSB ≤ TSDL

In this scenario, single time duration that UE can receive signals in SDL SCell can ensure the completion of BFD/CBD evaluation of SDL SCell, where the switching procedure has no impact on BFD/CBD measurements.

(2) Scenario 2: TEvaluate_BFD_SSB > TSDL

When UE needs longer time to finish BFD/CBD evaluation of SDL SCell, while it has to switch back to PCell, then UE has to continue the BFD/CBD evaluation of SDL SCell after it switches to SDL SCell. In this case, a longer evaluation period should be defined. There are multiple ways to include the impacts of switching procedure on evaluation period. For example, RAN4 can scale the existing evaluation period requirement by a factor derived by taking TSwitchingPattern or TSDL into account. But this method may give a quite long evaluation period, making the requirements meaningless. Another solution is to multiply the TEvaluate_BFD_SSB by a value Pswitching which is given by:

Pswitching=TEvaluate_BFD_SSBTSDL

Then the new evaluation period will be

TEvaluate_BFD_SSB_switching=Pswitching*TSwitchingPattern

Similarly, this principle can also apply to the deactivated SCell L3 measurements which are based on SMTC period and the BFD/CBD measurements of PCell, etc.

Proposal 4: When RRM measurements can be completed within single duration of switching pattern, the existing measurement requirements can be reused. Otherwise, longer measurement period should be defined, which can be given by Pswitching*TSwitchingPattern, where

Pswitching=TexistingTSDL

- Texisting is the corresponding existing measurement period.

- TSDL is the time duration during which UE works on SDL carrier.

This principle applies to the SMTC-based L3 measurements of SCell and the BFD/CBD measurements of PCell, etc.

SCell activation requirement

Issue 1-3-2: SDL SCell activation/deactivation - Proposal 1 (Apple): - If the legacy SSB based inter-band SCell activation procedure is assumed for LB CA, then SCell activation delay shall be defined based on switching pattern periodicity, or if UE is allowed to stay on the SDL SCell during activation time, the interruption can be as large as the whole SCell activation delay after n+1+THARQ. - Proposal 2 (Samsung): - RAN4 to discuss whether SSB-less SCell activation delay requirement can be applied to LB CA switching scenario - Proposal 3 (ZTE): - Normally the UE should execute the AGC, cell search, TCI state switching and fine T/F tracking when switching to the target serving cell. The whole switching procedure could be reduced provided some side condition meets similar as the existing SCell activation procedure.

As summarized above, SDL SCell activation requirements will also be impacted. Firstly, UE should not be allowed to stay on the SDL SCell during activation, because it is a quite long time period during which UE loses connection with PCell and this is not acceptable. However, if RAN4 directly replace SSB periodicity and SMTC period with switching pattern periodicity, which will lead to a longer SCell activation time than the existing one and this is not very efficient. In our opinion, a feasible solution is to scale the SSB periodicity and/or SMTC period by considering the time duration of UE working on SDL SCell, for example,

PSCellActivation_switching=TSDLTSMTC_MAX

Proposal 5: UE is not allowed to stay on the SDL SCell during the SCell activation time.

Proposal 6: RAN4 to update SCell activation requirements by scaling the SSB periodicity and/or SMTC period. The scaling factor, PSCellActivation_switching, can be derived by

PSCellActivation_switching=TSDLTSMTC_MAX

Where

- TSDL is the time duration during which UE works on SDL carrier.

- TSMTC_MAX follows the existing definitions in TS.38.133 clause 8.3.2.

Conclusions

This contribution discussed the impacts of LBCA on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to define interruption requirements and the interruption for PCell CCs (X1) is:

X1=2*switching period+TSDL

Where

- switching period is decided by RF group;

- TSDL is the time duration during which UE works on SDL carrier.

The interruption for SDL CC (X2) is:

X2=2*switching period+TPCell

Where

- TPCell is the time duration during which UE works on PCell carriers.

Other aspects can be further considered, e.g., TAE, if necessary.

Proposal 2: DL/UL timing requirements will not be impacted when either alternative is supported:

- Alt 1: Switching pattern designed by RAN1 can assure the availability of SSB at UE in last 160 ms in PCell.

- Alt 2: UE is allowed to use activated SCell in the pTAG as reference cell which provides DL timing information when there is only one DL cell at one time.

Proposal 3: Deactivated SCell measurement period requirements can be updated by replacing meaCycleSCell with TSwitchingPattern. Constant value(s), which need further discussion, can be used to scale the TSwitchingPattern.

Proposal 4: When RRM measurements can be completed within single duration of switching pattern, the existing measurement requirements can be reused. Otherwise, longer measurement period should be defined, which can be given by Pswitching*TSwitchingPattern, where

Pswitching=TexistingTSDL

- Texisting is the corresponding existing measurement period.

- TSDL is the time duration during which UE works on SDL carrier.

This principle applies to the SMTC-based L3 measurements of SCell and the BFD/CBD measurements of PCell, etc.

Proposal 5: UE is not allowed to stay on the SDL SCell during the SCell activation time.

Proposal 6: RAN4 to update SCell activation requirements by scaling the SSB periodicity and/or SMTC period. The scaling factor, PSCellActivation_switching, can be derived by

PSCellActivation_switching=TSDLTSMTC_MAX

Where

- TSDL is the time duration during which UE works on SDL carrier.

- TSMTC_MAX follows the existing definitions in TS.38.133 clause 8.3.2.

References

[1] R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4#114.

[2] R4-2500527, Topic summary for [114][213] NR_LBCA_Sw, Apple, RAN4#114.

[3] R4-2502664, LS of RAN4 agreements on LB CA via switching, Apple, RAN4#114.

[4] TS 38.133, NR; Requirements for support of radio resource management, V18.8.0.

[5] RP-243317, New WID on low band carrier aggregation via switching, RAN#106.