3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503422

Wuhan, CN, April 7th – April 11th, 2025

Title:	Discussion on the impacts of A-IoT on RRM requirements

Source:	CATT

Agenda Item:	7.24.3

Document for:	Discussion

1 Introduction

In RAN#106, a new WID [1] that requires RAN WGs to define solutions for Ambient IoT in NR was approved. In RAN#107, the WID was updated and the following objectives are as follows [2]:

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. … - RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In this contribution, we will analyse the impacts of A-IoT device 1 deployed in scenario 1 with topology 1 (D1T1-B) on RRM requirements.

2 Discussion

Since this is the first meeting for RRM session to discuss the impacts of A-IoT work item, we would like to provide some backgrounds before jump into the details.

Regarding the topology 1, scenario 1 and device 1

According to the TR 38.848 [3], related descriptions are reproduced below:

Topology 1: BS <->Ambient IoT device Figure 4.2.1.1-1: Topology 1 In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. This topology includes the possibility that the BS transmitting to the Ambient IoT device is a different from the BS receiving from the Ambient IoT device. Deployment scenario 1: Device indoors, basestation indoors With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1. Table 4.2.2.1-1: Characteristics of deployment scenario 1 Applicable representative use cases; Characteristics; Description (NOTE 1). Indoor inventory Indoor sensor Indoor positioning Indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Micro- or pico-cell. Connectivity topology; Topology (1), (2), (3). Spectrum; Licensed FDD, licensed TDD, unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device A or Device B or Device C. NOTE 1: Descriptions may not be applicable for some Devices (A, B). Device 1, i.e., Device B: Relying on these storage capacities, the study considers the following set of Ambient IoT devices: - - Device A: No energy storage, no independent signal generation/amplification, i.e. backscattering transmission. - - Device B: Has energy storage, no independent signal generation, i.e. backscattering transmission. Use of stored energy can include amplification for reflected signals. - - Device C: Has energy storage, has independent signal generation, i.e., active RF components for transmission. A limited energy storage can be different among implementations within Device B or implementations within Device C, and different between Device B and Device C. Such storage is expected to be order(s) of magnitude smaller than an NB-IoT device would typically include. Device A, B, and C are able to demodulate control, data, etc from the relevant entity in RAN according to connectivity topology.

Regarding the functionalities of A-IoT devices

According to TR 38.769 [4], related descriptions are reproduced below:

As to the A-IoT required functionalities, the following functionalities are supported: - A-IoT paging (see clause 6.3.3) - A-IoT random access procedure (see clause 6.3.4) - A-IoT data transmission (see clause 6.3.5) As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): - AS security (The AS layer design assumes no support of AS security, unless the study in [152] further concludes differently.) - RRC states - RRC connection management - RRM L3 measurement reporting - Mobility - ASN.1 encoding/decoding - Periodical system information and MIB - Tracking/RAN area update procedure - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. - Multiple A-IoT logical channels for upper layer data - NR SR, as in TS 38.300 [153] - NR BSR, as in TS 38.300 [153]

Based on the backgrounds above, we find that we are now dealing with a new communication equipment and system which is different from LTE/NR system and is much simpler. We also observe that A-IoT device 1, which will be deployed in indoor scenarios only, has very limited functionalities and many capabilities that NR UEs are capable of, such as synchronization with network, L3 measurement and mobility, are not supported by device 1. Therefore very limited RRM requirements need to be defined for A-IoT.

Observation 1: RAN4 is studying a new communication equipment and system which is much simpler than LTE/NR system.

Observation 2: A-IoT device 1 has very limited functionalities and therefore limited RRM requirements need to be defined for current A-IoT scope.

In the following part, we will analyse the impacts of A-IoT from its working procedures. Overall AS procedures are captured in [4] which include three steps between A-IoT device and reader:

Figure 2: Overall AS procedures between A-IoT device and reader The overall AS procedures can be formulated as: - Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3. - Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See clause 6.3.4 (and 6.3.5).. - Step C1: Possible R2D data transmission (e.g., for sending the command). See clause 6.3.5. - Step C2: Possible D2R data transmission (e.g., the corresponding response to command). See clause 6.3.5.

Based on the quoted contents, both R2D and D2R data transmissions are triggerred by reader via signalling, i.e., paging, and then the triggered device will perform random access with/withour data. For paging, there is no RRM requirement. For A-IoT random access (RA), similar to the RA in NR, there are two types of random access, i.e., contention-free and contention-based random access, and the type that is used by A-IoT device will be controlled by reader. Mor details for both A-IoT RA types can be found in [4]. In current NR RRM spec, RAN4 have captured the correct NR UE behavior duing random access procedure. Thus, RAN4 can also capture the correct A-IoT device behaviour duing random access procedure.

Regarding the necessity of requirements for A-IoT data transmission, no such demand is identified.

Proposal 1: RAN4 not to define requirements for A-IoT paging and data transmission.

Proposal 2: RAN4 to capture the correct A-IoT device behaviour during random access procedure.

One important issue that needs discussion is timing. For D2R, a timing acquisition signal (D-TAS), preceding each PDRCH, is included at least for timing acquisition, indicating the start of the D2R transmission in the time domain, and potentially for SFO estimation, CFO estimation, channel estimation, and interference estimation. Since this signal is used by A-IoT BS, RAN4 do not need to define requirements for D-TAS signal.

For R2D, the structure is designed as follows:

An R2D timing acquisition signal (R-TAS), immediately preceding the transmission of PRDCH, is included at least for timing acquisition and indicating the start of R2D transmission in the time domain. An R-TAS structure using a preamble is studied, in which a start-indicator part provides the start of the R2D transmission, and immediately precedes a clock-acquisition part which is used to determine the OOK chip duration of the subsequent PRDCH transmission. The preamble is not part of PRDCH.

Figure 3: A diagram for R-TAS and PRDCH

According to current RAN1 design, A-IoT device needs to process R-TAS to get the knowledge of start-indicator part (SIP), which indicates the start of PRDCH in time domain, and clock-acquisition part (CAP), which determines the OOK chip duration of PRDCH.  Therefore successfully detecting R-TAS is important for reception of PRDCH. But there are possibilities that A-IoT devices may fail to detect P-TAS and consequently miss the PRDCH, which will cause network performance degradation. Therefore it is necessary to define performance requirement to ensure A-IoT devices are capable of detecting SIP of P-TAS with a high successful rate, and candidate metrics include missed detection rate (MDR), false alarm rate (FAR), etc. The definition of MDR and FAR is:

  - MDR refers to the probability that SIP is not detected when it was actually transmitted

  - FAR probability that the receiver incorrectly detects SIP when SIP was not transmitted

Proposal 3: RAN4 to decide how to define performance requirement to ensure that A-IoT devices are capable of detecting SIP of P-TAS with a high successful rate. Candidate metrics include MDR and FAR:

  - MDR (missed detection rate) refers to the probability that SIP is not detected when it was actually transmitted

  - FAR (false alarm rate) probability that the receiver incorrectly detects SIP when SIP was not transmitted

3 Conclusion

In this paper, we discussed the impacts of A-IoT on RRM requirements and the following observations and proposals are given:

Observation 1: RAN4 is studying a new communication equipment and system which is much simpler than LTE/NR system.

Observation 2: A-IoT device 1 has very limited functionalities and therefore limited RRM requirements need to be defined for current A-IoT scope.

Proposal 1: RAN4 not to define requirements for A-IoT paging and data transmission.

Proposal 2: RAN4 to capture the correct A-IoT device behaviour during random access procedure.

Proposal 3: RAN4 to decide how to define performance requirement to ensure that A-IoT devices are capable of detecting SIP of P-TAS with a high successful rate. Candidate metrics include MDR and FAR:

  - MDR (missed detection rate) refers to the probability that SIP is not detected when it was actually transmitted

  - FAR (false alarm rate) probability that the receiver incorrectly detects SIP when SIP was not transmitted

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106.

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#107.

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, V18.0.0.

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, V19.0.0.