3GPP TSG-RAN WG4 Meeting #114-bis			                                                R4-2503429

# Wuhan , CN, 7th – 11st Apr, 2025

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Source:	CATT

Agenda Item:	7.25.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for OD-SSB SCell operation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

- Discussion

Sub-topic 1-1: OD-SSB based deactivated SCell L3 measurement (Case 1)

OD-SSB based deactivated SCell measurement – Procedure

For deactivated SCell L3 measurement for case 1, the following agreements on the length of FMW were reached in last meeting [1]:

RAN4#114: Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

For the expected minimum interval(T2, sage 4) between multiple FMWs, RAN4 agreed to define measurement requirement based on T2, but what is the definition of T2 and the value of T2 are still controversial, the agreements are copied as below [1]:

Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

In our view, this issue has become too complex compared to the beginning. T2 was initially proposed because the UE side concerned that the network would reactivate the OD-SSB transmission again in a relatively short time period after the network deactivated the first OD-SSB transmission, and if the UE still needed to perform fast measurement, it would be unfriendly to UE power saving.

Observation 1: T2 was initially proposed because the UE side concerned that the network would reactivate the OD-SSB transmission again in a relatively short time period after the network deactivated the first OD-SSB transmission, and if the UE still needed to perform fast measurement, it would be unfriendly to UE power saving.

From the network perspective, the above case is a corner case. If the network wants to reactivate the OD-SSB transmission in a very short time, the operation of deactivating first OD-SSB transmission and then reactivating OD-SSB immediately is unreasonable, and the transmission of OD-SSB should be maintained, otherwise it will not be beneficial to the network as well. On the other hand, since the time window is defined with Tuncertain plus actual measurement duration, and Tidentify in the actual measurement duration has already considered two cases including a detected cell and a newly detectable cell, so this is similar to some companies wanting to differentiate whether to skip the PSS/SSS detection and/or SSB index acquisition in fast measurement based on T2 as a boundary. Therefore, we believe that the agreed UE behaviour in FMW can be reused when reactivating OD-SSB transmission.

In addition, also considering that the workload of this WI is so large, we prefer RAN4 not to spend too much time to further discuss T2 related issues.

Observation 2: If the network wants to reactivate the OD-SSB transmission in a very short time, the operation of deactivating first OD-SSB transmission and then reactivating OD-SSB immediately is unreasonable, and the transmission of OD-SSB should be maintained, otherwise it will not be beneficial to the network as well.

Proposal 1: The network reactivates the OD-SSB transmission again in a relatively short time period after the network deactivated the first OD-SSB transmission, which is a corner case.

Observation 3: Tidentify in the actual measurement duration has already considered two cases including a detected cell and a newly detectable cell, so this is similar to some companies wanting to differentiate whether to skip the PSS/SSS detection and/or SSB index acquisition in fast measurement based on T2 as a boundary.

Proposal 2: The agreed UE behaviour in FMW can be reused when reactivating OD-SSB transmission.

Proposal 3: Prefer RAN4 not to spend too much time to further discuss T2 related issues.

OD-SSB based deactivated SCell measurement – requirement in FMW

For OD-SSB based deactivated SCell measurement requirements in FMW, the agreement son CSSF in the previous meeting are copied as follows [1]:

RAN4#113: Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. RAN4#114: Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

When NW configures single OD-SSB carrier with multiple legacy SSB carriers in FMW, we think that it is reasonable for UE to prioritize measurement on OD-SSB carrier in FMW, which means the CSSF only counts OD-SSB layers for SCell measurement with activated OD-SSB in the FMW, so CSSF=1. For other carriers with legacy AO-SSB, UE shall deprioritize the legacy SCell measurement and longer delay is expected.

Proposal 4: When NW configures single OD-SSB carrier with multiple legacy SSB carriers in FMW,

- UE to prioritize measurement on OD-SSB carrier in FMW, and CSSF = 1 for SCell measurement with activated OD-SSB in the FMW.

- For other carriers with legacy AO-SSB, UE shall deprioritize the legacy SCell measurement and longer delay is expected.

Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 2)

Always-on SSB and OD-SSB time/frequency domain relation

In the last meeting, RAN4 has agreed a new scenario 4 based on RAN1’s Reply LS [3], that is, OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. In scenario 4, RAN4 also agreed that the effective measurement periodicity is half of OD-SSB periodicity. The related background and agreements are copied as below [1]:

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity - The scenarios can be updated based on other WGs’ progress. Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

Regarding the above issues, RAN1 have sent the LS on time location of on-demand SSB for SCell to RAN4 in RAN1#120 meeting, the specific content is as follows [4]:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

From RAN1’s perspective, in Alt Time-C1, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification), so we think that the effective measurement periodicity can be described as ‘the union periodicity of AO-SSB transmission and OD-SSB transmission which is one of the legacy SSB periodicities’ to align with RAN1.

However, the agreement in RAN4 only involves Scenario 4, and Alt Time-C1 involves not only Scenario 4 but also Scenario 2 defined in RAN4. Therefore, the agreement can be revised as follows to align with RAN1:

Proposal 5: In Scenario 2 and Scenario 4 defined in RAN4, the measurement requirement is defined based on the effective measurement periodicity.

- The effective measurement periodicity is the union periodicity of AO-SSB transmission and OD-SSB transmission, which is one of the legacy SSB periodicities.

For Alt Time-C2, if the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, RAN4 also can define the related requirements for this case. In our view, a simple method is that UE follows the OD-SSB periodicity to perform measurement in FMW.

Proposal 6: RAN4 to define requirements for Alt Time-C2 defined by RAN1.

- UE follows the OD-SSB periodicity to perform measurement in FMW.

For Scenario 2 defined in RAN4, the agreements reached in RAN1#120 meeting is copied as follows:

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

According to the above, there is no restriction on the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, and it is corresponds to Scenario 3 defined in RAN4. In this case, we believe that the UE can determine which SSB to use for deactivated SCell measurement based on the network indication.

Proposal 7: For Scenario 3, the UE can determine which SSB to use for deactivated SCell measurement based on the network indication.

Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

For Case #2 where always-on SSB is periodically transmitted on the cell, the UE behaviors before OD-SSB is activated and after OD-SSB is deactivated have been agreed, but the UE behavior when OD-SSB is activated were still FFS. In RAN4#113 meeting, for the UE behavior when OD-SSB is activated, the following options for scenario 1 were proposed [2]:

Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

In our understanding, the most basic intention is to maximize the NES gain and to use a unified approach as much as possible. For case 2, we support option 1 to use the same framework as case 1, that is, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within FMW. After the time window, UE can perform deactivated SCell measurement with a slow mode based on measCycleSCell,  and UE can choose any of SSB to measure after the FMW regardless of NW transmitting OD-SSB or not.

In addition, for option 2, since legacy known/unknown conditions are defined for SCell activation delay not for deactivated SCell measurement, we prefer not to mix known/unknown with deactivated SCell measurement requirements. In scenario 2, even if SCell is known, UE cannot expect SCell to be activated latter, and when to active SCell is depends on the network, and the SCell may also become unknown again when SCell is activated.

Proposal 8: The same approach in Case 1 can be reused.

- UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE will fall back to a slow mode measurement (with DRX and measCycleSCell).

Note: UE can choose any of SSB to measure after the FMW regardless of NW transmitting OD-SSB or not.

LS to RAN2

Meanwhile, we noticed that RAN2 is also discussing the L3 measurement framework and the SSB to be used in case 2 the related issues are copied as follows:

3. L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted 4. Whether always-on SSB and/or OD-SSB are measured in case 2? RAN2#129: Agreements on OD-SSB 1. RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

According to the above, at least RAN2 has already left it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2 in the last meeting. To avoid the duplicated discussion and possible misalignment between different groups, we think RAN4 can send the LS to RAN2 to inform our related agreements if reached.

Proposal 9: Send LS to RAN2 to inform RAN4 agreements on the measurement framework, which includes UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for Case 1 and Case 2.

Sub-topic 1-3: SCell activation part

The known cell condition  – measurement reporting

In RAN4#112 meeting, it was agreed that RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A. And the known cell condition principle was also agreed as below in RAN4#112bis meeting and RAN4#113 meeting:

RAN4#112bis： Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. RAN4#113： Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

For Case 2, RAN4 agreed that known/unknown condition should be defined based on either always-on SSB or OD-SSB, but RAN4 also need to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

In our view, since the measurement before SCell activation is to be used during SCell activation procedure, the measured/reported SSB and the SSB used in SCell activation procedure should be with the spatial relation no matter if the known condition is determined based on always-on SSB and/or OD-SSB. Based on the principle of legacy known/unknown condition, the known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

In addition, RAN1 has considered the case of different frequency location between the always-on SSB and on-demand SSB, we think that the above condition also apply to this case (corresponding to scenario 3 defined in RAN4). In RAN1#120 meeting, the agreement on transmit power of OD-SSB is as follows:

Agreement: - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

According to the above, we don’t think AGC adjustment time is needed in the known SCell activation delay requirement even though OD-SSB and AO-SSB are transmitted on the different frequency.

In addition, for the period of known condition, we agree to reuse the legacy known/unknown cell condition, such as max(5*measCycleSCell,  5*DRX cycles) in FR1.

Proposal 10: The known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and,

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

Proposal 11: For the period of known condition, we agree to reuse the legacy known/unknown cell condition, such as max(5*measCycleSCell,  5*DRX cycles) in FR1.

After defining the known cell condition, the existing requirements framework can be reused by clarifying the signaling processing time, TFirstSSB/TFirstSSB_MAX and the type of SSB to be used in the SCell activation procedure.

SCell activation requirement - uncertainty time

For the definition of TFirstSSB/TFirstSSB_MAX which is the time to the end of the first complete SSB burst after SCell activation command, it depends on the discussion on SSB type used in SCell activation. At least for the case where OD-SSB is used for SCell activation, it can be updated to the end of first actual transmitted SSB to reflect time uncertainty between OD-SSB indication and actual OD-SSB transmission.

Proposal 12: Update the existing definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first complete actual transmitted SSB.

- At least for the case where OD-SSB is used for SCell activation, the time uncertainty between OD-SSB indication and actual OD-SSB transmission should be reflected in TFirstSSB and TFirstSSB_MAX.

- Conclusion

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following observations and proposals are given:

Observation 1: T2 was initially proposed because the UE side concerned that the network would reactivate the OD-SSB transmission again in a relatively short time period after the network deactivated the first OD-SSB transmission, and if the UE still needed to perform fast measurement, it would be unfriendly to UE power saving.

Observation 2: If the network wants to reactivate the OD-SSB transmission in a very short time, the operation of deactivating first OD-SSB transmission and then reactivating OD-SSB immediately is unreasonable, and the transmission of OD-SSB should be maintained, otherwise it will not be beneficial to the network as well.

Proposal 1: The network reactivates the OD-SSB transmission again in a relatively short time period after the network deactivated the first OD-SSB transmission, which is a corner case.

Observation 3: Tidentify in the actual measurement duration has already considered two cases including a detected cell and a newly detectable cell, so this is similar to some companies wanting to differentiate whether to skip the PSS/SSS detection and/or SSB index acquisition in fast measurement based on T2 as a boundary.

Proposal 2: The agreed UE behaviour in FMW can be reused when reactivating OD-SSB transmission.

Proposal 3: Prefer RAN4 not to spend too much time to further discuss T2 related issues.

Proposal 4: When NW configures single OD-SSB carrier with multiple legacy SSB carriers in FMW,

- UE to prioritize measurement on OD-SSB carrier in FMW, and CSSF = 1 for SCell measurement with activated OD-SSB in the FMW.

- For other carriers with legacy AO-SSB, UE shall deprioritize the legacy SCell measurement and longer delay is expected.

Proposal 5: In Scenario 2 and Scenario 4 defined in RAN4, the measurement requirement is defined based on the effective measurement periodicity.

- The effective measurement periodicity is the union periodicity of AO-SSB transmission and OD-SSB transmission, which is one of the legacy SSB periodicities.

Proposal 6: RAN4 to define requirements for Alt Time-C2 defined by RAN1.

- UE follows the OD-SSB periodicity to perform measurement in FMW.

Proposal 7: For Scenario 3, the UE can determine which SSB to use for deactivated SCell measurement based on the network indication.

Proposal 8: The same approach in Case 1 can be reused.

- UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE will fall back to a slow mode measurement (with DRX and measCycleSCell).

Note: UE can choose any of SSB to measure after the FMW regardless of NW transmitting OD-SSB or not.

Proposal 9: Send LS to RAN2 to inform RAN4 agreements on the measurement framework, which includes UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for Case 1 and Case 2.

Proposal 10: The known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and,

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

Proposal 11: For the period of known condition, we agree to reuse the legacy known/unknown cell condition, such as max(5*measCycleSCell,  5*DRX cycles) in FR1.

Proposal 12: Update the existing definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first complete actual transmitted SSB.

- At least for the case where OD-SSB is used for SCell activation, the time uncertainty between OD-SSB indication and actual OD-SSB transmission should be reflected in TFirstSSB and TFirstSSB_MAX.

- Reference

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114

- R4-2500537, Topic summary for [114][223] Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1#119

- R1-2501633, LS on time location of on-demand SSB for SCell, RAN1#120