3GPP TSG-RAN WG4 Meeting #114-bis				                                           R4-2503430

Wuhan , CN, 7th – 11st Apr, 2025

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

Source:	CATT

Agenda Item:	7.25.2.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for adaptation of common signal/channel transmission were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for adaptation of common signal/channel transmission and give our views.

- Discussion

In the last meeting, RAN4 discussed L3 requirements for SSB adaptation and didn’t reach the consensus. The agreement for L3 requirements impacts is copied as follows [1]:

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

RAN4 has agreed to discuss SSB periodicity adaptation in time domain as starting point according to RAN1 agreements. If network only indicate the periodicity adaption in time-domain, the only potential difference is that this may lead to SMTC reconfiguration which may impact L3 measurement requirements.

In RAN4#112-bis meeting, RAN4 sent the LS to RAN2 to inform that L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy and to ask RAN2 that whether there is any SMTC impact due to SSB adaptation [3], and RAN2 replied to LS about the SMTC impact on SSB adaptation in RAN2 #129 meeting, the agreements reached in RAN2 are copied as below [4]:

RAN2 #129 meeting: Agreements on SSB adaptation - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

To follow RAN2’s agreements, for the impact on L3 measurement requirements due to SSB adaptation, we think that L3 neighbour cell measurement can follows legacy SMTC requirement. For L3 RRM measurement on SCell with SSB adaptation, since RAN2 agreed to introduce additional SMTC configuration, RAN4 need to further discuss how to use additional SMTC configuration to define L3 measurement requirements on SCell with SSB adaptation, more details are needed from RAN2.

Proposal 1: The L3 neighbour cell measurement requirements due to SSB adaptation follows legacy SMTC requirement.

Proposal 2: RAN4 need to further discuss how to use additional SMTC configuration to define L3 measurement requirements on SCell with SSB adaptation, more details are needed from RAN2.

- Conclusion

In this paper, we have some further discussions on adaptation of common signal/channel transmission, and the following proposals are given:

Proposal 1: The L3 neighbour cell measurement requirements due to SSB adaptation follows legacy SMTC requirement.

Proposal 2: RAN4 need to further discuss how to use additional SMTC configuration to define L3 measurement requirements on SCell with SSB adaptation, more details are needed from RAN2.

- Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- R4-2500538, Topic summary for [114][224] Netw_Energy_NR_enh_Part2, Huawei

- R4-2416911, LS on SSB adaptation, RAN4#112-bis

- R4-2503117, Reply LS on SSB adaptation, RAN2#129