3GPP TSG-RAN WG4 Meeting #114-bis			                                                R4-2503431

# Wuhan , CN, 7th – 11st Apr, 2025

Title:	Discussion on RRM requirements for On-demand SIB1

Source:	CATT

Agenda Item:	7.25.2.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has agreed to start the discussion on potential RRM impact for On-demand SIB1 from this meeting [1]. In this paper, we discuss the potential RRM impact for On-demand SIB1 and give our views.

- Discussion

OD-SIB1 related requirement

Potential RRM impacts for OD-SIB1

In the last meeting, since this is no dedicated agenda for OD-SIB1, RAN4 didn’t discuss RRM requirements impacts for OD-SIB1, and the proposals are copied as follows [2]:

RAN4#114: Issue 2-1-1: RRM requirements impacts for OD-SIB1 RAN4#112 R4-2413885 Issue 2-1-1: RRM impact of OD-SIB1 - Agreement - Postpone RAN4 work until any RRM impact is identified.. - Proposals - Option 1: RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure. (Vivo) - Option 1a: Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements. (Vivo) - Option 1b: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission. (MTK) - Option 2: For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed. (Vivo) - Option 2a: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that: (CMCC) - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 - Option 3: RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements. (Vivo)

As of now, the progress of RAN1/RAN2 is relatively stable, and the overall process is also clearer. According to the design in RAN1/RAN2, the UL WUS is for wake-up of NES cell. The UE will receive UL WUS configuration from cell A, and then UE shall send UL WUS to the NES cell after UE meets the WUS triggering conditions.

Observation 1: The UE will receive UL WUS configuration from cell A, and then UE shall send UL WUS to the NES cell after UE meets the WUS triggering conditions.

Therefore, we propose that the UL timing requirements of UL-WUS need to be discussed. Since the reference point for the UE initial transmits timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc in legacy, so RAN4 need to clarify downlink reference point of UL-WUS in TS 38.133 for OD-SIB1 related procedure. In our view, the reference cell is the NES cell and NTA for UL-WUS is defined as 0. For NTA offset of UL-WUS, UE identifies NTA offset based on the information n-TimingAdvanceOffset from UL-WUS configuration, and if UE is not provided with the information n-TimingAdvanceOffset, the default value of NTA offset can be reused.

The RAN1/RAN2 progress can be a reference:

### RAN2#126: Triggering condition of WUS transmission - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell. UE request SIB1 to perform initial access in NES cell - RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell. ### RAN1#119: Agreement The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Proposal 1: RAN4 need to clarify downlink reference point of UL-WUS in TS 38.133 for OD-SIB1 related procedure.

- The reference cell is the NES cell.

- NTA for UL-WUS is defined as 0.

- UE identifies NTA offset based on the information n-TimingAdvanceOffset from UL-WUS configuration.

For paging procedure, when UE Performing target cell relevant SIBs reception, paging is interrupted and RAN4 defined the corresponding maximum interruption in paging reception requirements for cell reselection. The legacy paging reception related requirements are copied as below [3]:

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

In our view, legacy interruption time include TSI-NR, which is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 for an NR cell. However, RAN4 didn’t define the exact value for SIB1 receiving in TS 38.133[3]. For OD-SIB1 reception, RAN4 need to discuss whether there are impact on the interruption in paging reception requirements for cell reselection.

Observation 2: In legacy maximum interruption in paging reception requirements, RAN4 didn’t define the exact value for SIB1 receiving in TS 38.133.

Proposal 2: For OD-SIB1 reception, RAN4 need to discuss whether there are impact on the interruption in paging reception requirements for cell reselection in TS 38.133.

- Conclusion

In this paper, we have some further discussions on On-demand SIB1, and the following observations and proposals are given:

Observation 1: The UE will receive UL WUS configuration from cell A, and then UE shall send UL WUS to the NES cell after UE meets the WUS triggering conditions.

Proposal 1: RAN4 need to clarify downlink reference point of UL-WUS in TS 38.133 for OD-SIB1 related procedure.

- The reference cell is the NES cell.

- NTA for UL-WUS is defined as 0.

- UE identifies NTA offset based on the information n-TimingAdvanceOffset from UL-WUS configuration.

Observation 2: In legacy maximum interruption in paging reception requirements, RAN4 didn’t define the exact value for SIB1 receiving in TS 38.133.

Proposal 2: For OD-SIB1 reception, RAN4 need to discuss whether there are impact on the interruption in paging reception requirements for cell reselection in TS 38.133.

- Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- R4-2500538, Topic summary for [114][224] Netw_Energy_NR_enh_Part2, Huawei

- 3GPP TS 38.133, NR Requirements for support of radio resource management