3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503433

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.27.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for event triggered L1 measurement reporting were discussed. The conclusions were captured in the approved WF [1]. In this paper, we further discuss the RRM impact for event triggered L1 measurement for LTM and give our views.

- Discussion

- 2.1 Event triggered reporting delay for L1 reporting

RAN4 agreed the high level event-triggered L1 reporting requirement for SSB based measurements to follow legacy framework of L3 measurement. In the last meeting, RAN4 further discussed the detail, and the agreement for L1 event triggered reporting delay is copied as below [1], there are still two options:

Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

For event triggered reporting delay for L1 reporting, it is necessary to consider both the measurement results of serving and neighboring cells, especially if Event LTM3 is configured, the UE needs to compare the measurement results of serving and neighboring cells. Therefore, the essential issue is how to handle the case that measurement period of the cells corresponding to the event are different. When the event is met, there may be a situation where the measurement results of the serving cell are waiting for the measurement results of the nerghbor cell, or the nerghbor cell is waiting for the measurement results of the serving cell.

For option 1, it emphasizes that the maximum L1-RSRP measurement period on the cells corresponding to the event, it is clearer for us. The motivation for option 2 is not clear, which cell can be regarded as the cell(s) which triggers the event on the cells corresponding to the event need to be further clarified. In addition, what we were originally discussing is the maximum upper bound for event triggered reporting delay for L1 reporting, so we support option 1.

Proposal 1: For the event-triggered L1 reporting requirement

- Except for periodic MR:

- The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

- For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- RAN4 define delay requirement for the first report after condition is met.

- 2.2 whether and how to coordinate with R19 MIMO

In the last meeting, the Candidate options for whether and how to coordinate with R19 MIMO is copied as below [1]:

Candidate options: - Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC)

For this issue, we can agree that RAN4 shall develop the two features independently at least in this stage. When discussing the UE capabilities, the further optimization such as joint consideration of UE capabilities can be discussed.

Proposal 2: RAN4 shall develop the two features independently at least in this stage. When discussing the UE capabilities, the further optimization such as joint consideration of UE capabilities can be discussed.

- Conclusion

This document discussed the issues for event triggered L1 measurement for LTM and the following proposals are presented:

Proposal 1: For the event-triggered L1 reporting requirement

- Except for periodic MR:

- The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

- For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- RAN4 define delay requirement for the first report after condition is met.

Proposal 2: RAN4 shall develop the two features independently at least in this stage. When discussing the UE capabilities, the further optimization such as joint consideration of UE capabilities can be discussed.

- Reference

[1]      R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple