3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503444

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	7.29.2.1

Document for:	Approval

- Introduction

In RAN4 #114 meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Issue 1-1: PC3 output power for (e)RedCap Agreement: - Rel-18 power boosting is applicable for NR NTN including (e)RedCap - No reference sensitivity specification impact (e.g. RSD) from power boosting. - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN - FFS on solutions on additional new output power enhancement - supported modulation orders - potential downselection between HD-FDD and FD-FDD - simultaneous support with rel-18 power boosting - Background: output power enhancement (e.g., FE IL improvement) - Other solutions not precluded Note: No change for (e)RedCap MOP tolerance to enhance the output power does not preclude any solution in other Rel-19 WI.

In the previous meeting, RAN4 has agreed that Rel-18 power boosting feature could be applicable for NR NTN including RedCap. However, for Rel-19 (e) RedCap HD-FDD UE, the applicability of Rel-18 power boosting feature should be discussed as well. Referring to the discussion of RAN4 #114 meeting, the potential implementation of power boosting could be summarized step by step as below:

- Step 1: For NTN RedCap UE operating in HD-FDD mode, load Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18. Now the NTN RedCap UE operating in HD-FDD mode could be boosted 1 dB for PC 3 or 0.5 dB for PC 2.

- Step 2: On top of step 1, load Rel-19 power boosting capabilities (still under discussion), which could boost X dB for PC 3 and Y dB for PC 2 with new schemes (such as reducing the IL).

After step 1 and step 2, for NTN HD-FDD RedCap UE, the final power boosting could be configured as:

- Enable Rel-18 power boosting only

- Enable Rel-19 power boosting only

- Enable both

So far, there does not appear to be any particular problem with the logic of power boosting configuration. However, since the Rel-19 power boosting schemes are still under discussion, it seems not appropriate to take it into account until the final X and Y values have been determined. In addition, Rel-19 power boosting scheme may not be band agnostic, which brings uncertainty for final conclusion.

Observation 1: The Rel-19 power boosting WI is still under discussion.

Considering that the Rel-18 power boosting feature is applicable for NR NTN HD-FDD RedCap UE, the supported configurations should be identified. From our perspective, at least step 1 could be supported, which ensures that NTN HD-FDD RedCap UE can benefit from the power boosting immediately.

Proposal 1: For NTN RedCap UE operating in HD-FDD mode, Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 could be supported.

However, we have to recognise that new solutions such as Rel-19 Power Boosting could further enhance the competitiveness of NTN HD-FDD RedCap UE. Therefore, the discussion on new solutions such as Rel-19 Power boosting should be welcomed, and we believed that it could be further discussed.

Observation 2: The discussion on new solutions such as Rel-19 Power boosting should be welcomed, and it could be further discussed.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Observation 1: The Rel-19 power boosting WI is still under discussion.

Proposal 1: For NTN RedCap UE operating in HD-FDD mode, Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 could be supported.

Observation 2: The discussion on new solutions such as Rel-19 Power boosting should be welcomed, and it could be further discussed.

- References

- R4-2502281, Way Forward for [114][310] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 114.