3GPP TSG-RAN WG4 Meeting #114-bis                                                                                     R4-2503445

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on SAN diagram for Rel-19 NTN SAN

Agenda item:	7.29.3

Document for:	Approval

- Introduction

In RAN4 #114 meeting, some discussion on Rel-19 NR NTN SAN diagram improvement resurfaced, and there are still some different understandings of the approaches of improvement.

In this contribution, we would like to share our views on Rel-19 NR NTN SAN diagram.

- Discussion

The current SAN diagram (Requirement reference points) in TS 38.108 indicates the testing point of SAN, and we would like to take type 1-H SAN as an example, as shown below:

### 4.2.1 SAN type 1-H For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.2.1-1: Radiated and conducted reference points for SAN type 1-H

Since regenerative payloads were introduced for the Rel-19 NTN SAN, and the architecture of regenerative payloads is different from legacy transparent payloads, there is a difference of opinion on how to update the SAN diagram and definition. The conclusions on how to update SAN diagrams and definitions were formed in the RAN4 #111 meeting, as shown below[1]:

Issue 1-2-1: Figures for regenerative payload - Agreements: No update required for TS 38.108 section 4 “requirements reference points diagrams”. Keep SAN type 1-H and for SAN type 1-O figures as agnostic to the regenerative/non-regenerative functionality of the SAN. Issue 1-2-2: SAN definition update - Agreement: “Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN Satellite capable UE, and connected via the NG interface to the 5GC. It encompass a transparent payload on board an NTN platform, a satellite gateway and gNB functions, or a regenerative payload on board an NTN platform and a satellite gateway.”

At this point, the discussion of the SAN diagram has come to an end, and no further modifications should be made to the SAN diagram. However, some companies still believe that SAN diagrams can make some improvements, and the potential directions for improving SAN diagrams are briefly summarised here in two approaches:

- Alt 1: Two different diagrams will be used to represent the transparent payload and the regenerative payload respectively. This is because they have structural conflicts and cannot be represented correctly in one diagram.

- Alt 2: One single diagram will be used to represent the transparent payload and the regenerative payload. Wiping out the components that cause structural conflicts.

In fact, the structural conflict comes from the part circled in red in the figure below, i.e. the non-NTN infrastructure gNB function:

Some companies believed that for SAN supporting regenerative payload, the full gNB will be on board of the satellite, hence the component non-NTN infrastructure gNB function is not necessary and should be removed from the SAN diagram. This is the structural conflict with the existing SAN diagram.

From our understanding, there is no clear conclusion on whether non-NTN infrastructure gNB function is still needed for regenerative payloads. Therefore, we recommend that the non-NTN infrastructure gNB function could be defined as an optional component at this stage, which will ensure maximum flexibility in real deployment.

Proposal 1: The non-NTN infrastructure gNB functions could be optional for Rel-19 NTN SAN that supporting regenerative payloads.

By defining the non-NTN infrastructure gNB functions as optional, the conflicting structures in the SAN diagram will be mitigated. Meanwhile, the non-NTN infrastructure gNB functionality in the current diagram will not need to be removed. Therefore, no further modifications to the SAN diagram are required, as agreed at the RAN4 #111 meeting. In addition, a clarification note can be added in section 4 to declare that the non-NTN infrastructure gNB function is optional for Rel-19 SAN supporting regenerative payload.

Proposal 2: No update required for SAN diagram in TS 38.108/181 section 4 “requirements reference points diagrams”.

Proposal 3: A clarification note can be added in section 4 in TS 38.108/181 to declare that the non-NTN infrastructure gNB function is optional for Rel-19 SAN that supporting regenerative payload.

- Conclusion

This contribution provides our views on Rel-19 NR NTN SAN diagram. The following proposals are concluded as follows:

Proposal 1: The non-NTN infrastructure gNB functions could be optional for Rel-19 NTN SAN that supporting regenerative payloads.

Proposal 2: No update required for SAN diagram in TS 38.108 section 4 “requirements reference points diagrams”.

Proposal 3: A clarification note can be added in section 4 to declare that the non-NTN infrastructure gNB function is optional for Rel-19 SAN that supporting regenerative payload.

- References

- R4-2410009, Way Forward for [111][315] NR_NTN_Ph3, Thales, RAN4# 111.