3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503447

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	7.10.3.2

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz UE RF requirements was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE RF requirements for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on UE RF requirements for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

# WF on UE RF requirements - UE Rx requirements Issue 2-2-1: Reference sensitivity - Agreement - For band n254, [2.2] dB improvement compared with 5 MHz CBW; - For band n256, [1.7]dB improvement compared with 5 MHz CBW; - FFS for band n255

For the UE Rx requirements with NTN UE support 3 MHz channel bandwidth, the REFSENS was discussed during the previous meeting and the REFSENS improvement for band n256 and n254 have been concluded. Compared to the 5 MHz channel bandwidth, the NRB of the NTN operating at 3 MHz channel bandwidth will reduce to 15 RBs, resulting in a 2.2 dB reduction in total received power. Meanwhile, we noticed that the impact of Rx non-linearity may need to be considered as it may degrade the performance of REFSENS (usually 0.5 dB), but this is usually due to the fact that the affected operating bands typically have a small duplex distance. We discussed the duplex distances for n256 and n254 in the last meeting and came to separate conclusions. For band n255, we believe that the duplex distance is still quite large. Therefore, we believe that the 2.2 dB REFSENSE improvement can still be considered for the n255 band.

Proposal 1: For NR NTN UE support 3 MHz channel bandwidth, 2.2 dB REFSENSE improvements could be considered for band n255.

- Conclusion

This contribution provides our views on UE RF requirements for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: For NR NTN UE support 3 MHz channel bandwidth, 2.2 dB REFSENSE improvements could be considered for band n255.

- References

- R4-2502282, WF on [114][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #114