3GPP TSG-RAN WG4 Meeting #114-bis                                                                                       R4-2503449

Wuhan, CN, Apr. 7 –Apr. 11, 2025

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	7.10.2.2.2

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

# Open issues Issue 3-1-3: RSD requirements for PC2 on n255 - Proposals - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; [0.4]; [0.4]; [0.5]; [0.5]. 2Tx; [0.8]; [0.8]; [0.9]; [0.9]. - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei) - PA noise as -117 dBm/Hz. - Option 3: Assuming IIP2 54dBm (Qualcomm) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; 1.3; 1.3; 1.3; 1.3. 2Tx; 2.4; 2.4; 2.4; 2.4. Way forward: - Continue align the assumptions and further discuss above options. Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo) - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek) Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [1.8]; [1.8]; [1.8]; [1.8]. n255; [2.4]; [2.4]; [2.5]; [2.5]. n254; [1.8]; [1.8]; [1.8]. Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [2.2]; [2.2]; [2.2]; [2.2]. n255; [3.2]; [3.2]; [3.3]; [3.3]. n254; [2.2]; [2.2]; [2.2]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung) - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256. Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3.7]; [3.7]; [3.7]; [3.7]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options.

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain.

For PC1 NTN HPUE, the Tx architectures will be non-handheld with 1Tx or [4Tx]. For non-handheld 1Tx PC 1 NTN HPUE, since there is only 1 Tx chain, we believed that the RSD is not required. For non-handheld [4Tx] PC 1 NTN HPUE, it is important to reach a final agreements whether the PC1 NTN HPUE will supports the 4Tx architecture or not, then the corresponding RSD value could be derived.

Proposal 1: For NR NTN 1Tx PC 1 non-handheld HPUE, the RSD value could be 0 dB. Meanwhile, it is necessary to determine if the NR NTN PC 1 non-handheld HPUE supports 4Tx.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For NR NTN 1Tx PC 1 non-handheld HPUE, the RSD value could be 0 dB. Meanwhile, it is necessary to determine if the NR NTN PC 1 non-handheld HPUE supports 4Tx.

- References

- R4-2502954, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #114