3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2503452

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Further discussion on BS demodulation performance for MMSE-IRC

Agenda item:	7.18.3

Document for:	Approval

- Introduction

In RAN4#114 meeting, a WF [1] on NR demodulation performance: Phase 5 was agreed, and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

Number of interferers for 2Rx

As per the WF [1], the remaining open issue concerning number of interferers for 2Rx is shown as below:

Number of interferers for 2Rx - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx. - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following: - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario. - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

The simulation results for number of interferers for 2Rx for HetNet and HomNet form our simulation result contribution [2] are copied in the following Table 2-1.

Table 2-1: Simulation results for number of interferers for 2Rx for HetNet and HomNet

| Propagation condition for target cell | Number of Interferer cells | DIP set (DIP1, DIP2) dB | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE (SNR), ideal | MMSE-IRC(SNR), ideal | gain (SNR)(dB) | gain of 2interferer cells to 1interferer cell (dB) |
| TDLC300-100 | 1 | HomNet set1: (-1.11. N/A) | 14.75 | 11.7 | 3.05 | N/A |
| TDLB100-400 |  |  | 17 | 14.8 | 2.2 | N/A |
| TDLC300-100 | 2 | HomNet set1: (-1.11. -10.91) | 16.9 | 14.1 | 2.8 | -0.25 |
| TDLB100-400 |  |  | 19 | 17.1 | 1.9 | -0.3 |
| TDLC300-100 | 1 | HetNet set2: (-0.43. NA/) | 18.65 | 14.3 | 4.35 | N/A |
| TDLA30-10 |  |  | 19 | 13.1 | 5.9 | N/A |
| TDLC300-100 | 2 | HetNet set2: (-0.43. -13.78) | 21.36 | 17.36 | 4 | -0.35 |
| TDLA30-10 |  |  | 21.6 | 15.8 | 5.8 | -0.1 |

From above Table 2-1, the simulation result shows that the minimum gain of MMSE-IRC to MMSE for HomNet in TDLC300-100, TDLB100-400 with 1 interferer cell, 2 interferer cells is 1.9dB, which indicates that it is reasonable to define the MMSE-IRC for 2Rx for HomNet.

Observation 1: Minimum gain of MMSE-IRC to MMSE for HomNet in TDLC300-100, TDLB100-400 with 1 interferer cell, 2 interferer cells is 1.9dB, which indicates that it is reasonable to define the MMSE-IRC for 2Rx for HomNet.

Proposal 1: To define BS MMSE-IRC requirements with 2Rx for HomNet scenario.

From above Table 2-1, the maximum gain of 2interferer cells to 1interferer cell (dB) for MMSE-IRC for HomNet is -0.3dB, and the maximum gain of 2interferer cells to 1interferer cell (dB) for MMSE-IRC for HomNet is -0.35dB. The above mentioned maximum gain of 2interferer cells to 1interferer cell (dB) for MMSE-IRC for HomNet and HetNet is great than-0.5dB and negative, which shows that the 1interferer cell can achieve more MMSE-IRC gain than 2interferer cells, but if simulation error is considered, the gain of 2interferer cells to 1interferer cell for MMSE-IRC for HomNet and HetNet is not obvious, so to define 1interferer cell or 2interferer cells for MMSE-IRC for HomNet and HetNet is reasonable.  Since the gain of 2interferer cells to 1interferer cell is not obvious, and to keep same number of interferers for 2Rx, 4Rx, and 8Rx, it is reasonable to define 2 interferer cells for 2Rx for HomNet and HetNet.

Proposal 2: To define 2 interferer cells for 2Rx for HomNet and HetNet to keep same number of interferers as 4Rx, and 8Rx.

MCS

As per the WF [1], the remaining open issue concerning MCS is shown as below:

MCS <Agreement> - MCS 16 for 2Rx (if introduced), 4Rx and 8Rx

As above discussion, it is reasonable to define 2 interferer cells for 2Rx for HomNet and HetNet, so it is reasonable to adopt MCS 16 for 2Rx.

Proposal 3: Adopt MCS 16 for 2Rx.

Propagation condition for the serving cell

As per the WF [1], the remaining open issue concerning propagation condition for serving cell is shown as below:

Propagation condition for the serving cell - Select between the following options: - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2) - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

The simulation results for 2 interferer cells for 2Rx,4Rx and 8Rx form our simulation result contribution [2] are copied in the following Table 2-2, Table 2-3 and Table 2-4, respectively.

Table 2-2: Simulation result of gain of MMSE-IRC to MMSE for 2Rx

| MCS for target cell | Propagation condition for target cell | DIP set (DIP1, DIP2) dB | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE (SNR), ideal | MMSE-IRC(SNR), ideal | gain (SNR) |
| 16 | TDLC300-100 | HomNet set1: (-1.11. -10.91) | 16.9 | 14.1 | 2.8 |
|  | TDLB100-400 |  | 19 | 17.1 | 1.9 |
| 16 | TDLC300-100 | HetNet set2: (-0.43. -13.78) | 21.36 | 17.36 | 4 |
|  | TDLA30-10 |  | 21.6 | 15.8 | 5.8 |

Table 2-3: Simulation result of gain of MMSE-IRC to MMSE for 4Rx

| MCS for target cell | Propagation condition for target cell | DIP set (DIP1, DIP2) dB | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE (SNR), ideal | MMSE-IRC(SNR), ideal | gain (SNR) |
| 16 | TDLC300-100 | HomNet set1: (-1.11. -10.91) | 13.6 | 7.74 | 5.86 |
|  | TDLB100-400 |  | 14.8 | 10.08 | 4.72 |
| 16 | TDLC300-100 | HetNet set2: (-0.43. -13.78) | 18.35 | 9.32 | 9.03 |
|  | TDLA30-10 |  | 18 | 8.53 | 9.47 |

Table 2-4: Simulation result of gain of MMSE-IRC to MMSE for 8Rx

| MCS for target cell | Propagation condition for target cell | DIP set (DIP1, DIP2) dB | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MMSE (SNR), ideal | MMSE-IRC(SNR), ideal | gain (SNR) |
| 16 | TDLC300-100 | HomNet set1: (-1.11. -10.91) | 10.9 | 5 | 5.9 |
|  | TDLB100-400 |  | 11.15 | 5.95 | 5.2 |
| 16 | TDLC300-100 | HetNet set2: (-0.43. -13.78) | 15.55 | 5.75 | 9.8 |
|  | TDLA30-10 |  | 15.45 | 5.05 | 10.4 |

From above Table 2-2 to Table 2-4, for HomNet, both TDLC300-100 and TDLB100-400 can achieve obvious MMSE-IRC gain, For HetNet, both TDLC300-100 and TDLA30-10 can achieve obvious MMSE-IRC gain. If only one propagation condition is chosen, the propagation condition with a larger gain of MMSE-IRC to MMSE is preferable. So,

Observation 2: If only one propagation condition is chosen, the propagation condition with a larger gain of MMSE-IRC to MMSE is preferable.

Form above simulation results, the gain of MMSE-IRC to MMSE for HomNet in TDLC300-100 for 2Rx, 4Rx and 8Rx is 2.8, 5.86 and 5.9dB(yellow shadowed), meanwhile, the gain of MMSE-IRC to MMSE for HomNet in TDLB100-400 for 2Rx, 4Rx and 8Rx is 1.9, 4.72 and 5.2dB, the above mentioned gain shows that  the gain of MMSE-IRC to MMSE for HomNet in TDLC300-100 is greater than that in TDLB100-400.  So it is reasonable to adopt TDLC300-100 for HomNet for MMSE-IRC requirement.

Observation3: Gain of MMSE-IRC to MMSE for HomNet in TDLC300-100 is greater than that in TDLB100-400.

Form above simulation results, the gain of MMSE-IRC to MMSE for HetNet in TDLC300-100 for 2Rx, 4Rx and 8Rx is 4, 9.03 and 9.8dB, meanwhile, the gain of MMSE-IRC to MMSE for HetNet in TDLA30-10 for 2Rx, 4Rx and 8Rx is 5.8, 9.47 and 10.4dB (green shadowed), the above mentioned gain shows that the gain of MMSE-IRC to MMSE for HetNet in TDLC300-100 is less than that in TDLA30-10. So it is reasonable to adopt TDLA30-100 for HetNet for MMSE-IRC requirement.

Observation 4: Gain of MMSE-IRC to MMSE for HetNet in TDLC300-100 is less than that in TDLA30-10.

So based on above analysis, we propose,

Proposal 4: Adopt Option 3 for propagation condition for serving cell as below:

  - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Observation 1: Minimum gain of MMSE-IRC to MMSE for HomNet in TDLC300-100, TDLB100-400 with 1 interferer cell, 2 interferer cells is 1.9dB, which indicates that it is reasonable to define the MMSE-IRC for 2Rx for HomNet.

Proposal 1: To define BS MMSE-IRC requirements with 2Rx for HomNet scenario.

Proposal 2: To define 2 interferer cells for 2Rx for HomNet and HetNet to keep same number of interferers as 4Rx, and 8Rx.

Proposal 3: Adopt MCS 16 for 2Rx.

Observation 2: If only one propagation condition is chosen, the propagation condition with a larger gain of MMSE-IRC to MMSE is preferable.

Observation3: Gain of MMSE-IRC to MMSE for HomNet in TDLC300-100 is greater than that in TDLB100-400.

Observation 4: Gain of MMSE-IRC to MMSE for HetNet in TDLC300-100 is less than that in TDLA30-10.

Proposal 4: Adopt Option 3 for propagation condition for serving cell as below:

  - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet

- References

- R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114

- R4-2503453, Simulation results for BS requirement with MMSE_IRC receiver, CATT, RAN4#114bis