3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2503454

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Further discussion on coexistence evaluations for Ku-band for NR NTN

Agenda item:	7.11.2

Document for:	Approval

- Introduction

In RAN4#113 and 114 meeting, a WF [3, 1] on NTN Ku band coexistence study was agreed, and remaining issues on Ku-band coexistence assumptions in WF require further discussion.

In this contribution, we provide our views on coexistence evaluation for Ku-band for NR NTN.

- Discussion

Conducted power (before ohmic loss) per antenna element (dBm) for VSAT phased array antenna

As per the WF [3], the agreement concerning conducted power per antenna element for VSAT phased array antenna for Ku-band NTN is shown as below:

### Sub-topic 2-1: VSAT antenna pattern Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel. Agree to use 38.921 unless there is a need to use sub array then 38.803 The following parameters are agreed: Table 2-1 VSAT phased array antenna characteristics Ku band VSAT phased array antenna. 1; Ku band VSAT phased array antenna Characteristics. 1.1; Antenna pattern; Table 2-2 and 2-3 (TR 38.921). 1.2; Element gain (dBi) (Note 2); 0 or 3.5 ?. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization; Circular（RHCP or LHCP）. 1.6; Antenna array configuration (Y axis × Z axis)(Note 4); 45x45 elements. 1.7; Number of supported polarizations, P; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda. 1.9; Array Ohmic loss (dB) (Note 2); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); [4.44 dBm] TBA. 1.11; maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; normal direction; Toward X+ axis. Note 1: Void Note 2: The element gain in row 1.2 includes the loss given in row 1.9. Note 3: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 4: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Agree 3.5 dB element gain

As above, the Conducted power (before Ohmic loss) per antenna element is [4.44 dBm] TBA, since 40dBm (10W) VSAT max Tx power was agreed in WF in RAN4#112bis [2], and a 45 x 45 = 2025 element antenna[1] was agreed for VSAT phased array (flat-panel) antenna. Then, the conducted power (before Ohmic loss) per antenna element (dBm) is 10*log10(10*1000)-10*log10(45*45) = 6.9357dBm. We propose the conducted power (before Ohmic loss) per antenna element (dBm) = 6.94dBm.

Proposal 1: Adopt 6.94dBm for conducted power (before Ohmic loss) per antenna element (dBm) for VSAT phased array antenna.

VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna

As per the WF [3], the agreement concerning VSAT Max Tx antenna gain for 100cm aperture parabolic antenna for Ku-band NTN is shown as below:

### Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

As above, the 100 cm aperture (41.3dBi) parabolic antenna was agreed for Ku-band NTN coexistence, the VSAT Max TX antenna gain is 41.3 dBi for 14GHz UL, but the VSAT Max RX antenna gain for 11GHz DL is missing. For parabolic antenna, one 100 cm aperture parabolic antenna can support both 14GHz UL and 11GHz DL at the same time, but the antenna gain for DL and UL is different.

3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

So, using Eq.1, for 14GHz,  2θ-3dB14GHz=1.2631 degree, and for 11GHz, 2θ-3dB11GHz=1.6076 degree.

We can use 41.3 dBi for 14GHz and 1.2631degree 3dB beamwidth as reference, the antenna gain for 11GHz can be calculated by following Eq.2.

G11GHz=G14GHz-20log102θ-3dB11GHz2θ-3dB14GHz=41.3-20*log101.60761.2631=39.2                                            (Eq.2)

So, we propose that VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna is 39.2dBi.

Proposal 2: Adopt 39.2dBi for VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna.

The above proposals are captured in Co-existence simulation assumptions for NR NTN Ku band (Running document) in Annex.

- Conclusion

This contribution provides analysis on coexistence assumptions for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Adopt 6.94dBm for conducted power (before Ohmic loss) per antenna element (dBm) for VSAT phased array antenna.

Proposal 2: Adopt 39.2dBi for VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna.

The above proposals are captured in R4-2502315 Co-existence simulation assumptions for NR NTN Ku band (Running document) in Annex.

- References

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), Ericssion, RAN4#114

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

- Annex

The revision is based on R4-2502315 Co-existence simulation assumptions for NR NTN Ku band (Running Document).

<Start of Change 1>

6a.2.2	System parameters

6a.2.2.1	Satellite parameters

Set-1 satellite parameters are listed in Table 6a.2.2.1-2 according to TR 38.821 [6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6a.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration Ku-band | NRB (100MHz BW) |
| --- | --- |
| SCS 30 kHz | 273 |

Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 30.9 dB K-1 | 10.9dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 6a.2.2.1-3: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 14GHz(UL) / 11GHz(DL) |  |
| The number of active UE (UL) | 10 UEs and 27RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6a.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 6a.2.2.1-4 |  |
| Handover margin | 3dB |  |

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 30.9 | 10.9 | 10.9 |
| NF (dB) | 3 | 3 | 3 |

6a.2.2.2	NTN UE parameters

The NTN UE antenna characteristics are given in Table 6a.2.2.2-1 and other NTN UE parameters are given in Table 6a.2.2.2-2.

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| 1 | VSAT Phased array antenna Characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.94 |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). |  |  |

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | Parabolic antenna for GSO, Phased array antenna for NGSO |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 100 cm equivalent aperture diameter for GSO, above Table 6a.2.2.2-1 for NGSO |
| Rx Antenna gain | 39.2 dBi forGSO,34.3 dBi for NGSO |
| Antenna temperature | 150 K |
| Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |
| Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO |
| Tx antenna gain | 41.3 dBi for GSO, 36.6 dBi for NGSO |
| Antenna elevation angle | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

The altitudes of NTN UE for different use case are defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-3: NTN UE altitude

| Use case | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- |
| Altitude | 22.5m | 3-14km | 1.5m |
| Note 1: Co-existence evaluation is only limited to L-ESIM. |  |  |  |

<End of Change 1>