3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2503456

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	7.11.5

Document for:	Approval

- Introduction

In RAN4#114 meeting, the WF on UE and SAN RF [1] was agreed and remaining issues on SAN RF require further discussion. And in RAN#107 meeting, the 15kHz SCS was agreed to introduce for Ku-band NTN for FR1 in revised WID, and some issues for 15kHz SCS also requires discussion.

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

According to WF on UE and SAN RF [1], after removing the agreed reused requirements, the following radiated requirements for SAN type 1-O and SAN type 2-O for NTN Ku-band separately shown in Table 2-1 need to further discussion.

Table 2-1 initial applicability considerations for radiated requirements for Ku-band NTN

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused |
| 9.7 OTA unwanted emissions | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Need to check whether Spurious frequency range is applicable for 11GHz for Ku-band. |
| Radiated Rx Requirements |  |  |  |  |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

EIRP accuracy and TRP accuracy

The existing EIRP accuracy and TRP accuracy in normal conditions for FR1-NTN and FR2-NTN are summarized in the following table.

| Frequency range designation | Corresponding frequency range (MHz) | EIRP accuracy(dB) | TRP accuracy(dB) |
| --- | --- | --- | --- |
| FR1-NTN | 410 – 7125 | 2.2 | 2 |
| FR2-NTN | 17300 – 30000 | 3.4 | 3 |

Referring to TR 37.842 and TR 37.843, the factors [2] which influence EIRP and TRP accuracy are but not limited to the following:

1) Transceiver unit accuracy;

2) Array accuracy, including sub-array gain accuracy, mismatch errors and insertion loss variations;

3) Steer accuracy, including phase shift accuracy among transceiver units, and phase shifter accuracy.

So, EIRP accuracy and TRP accuracy can be expressed as equation 1 and equation 2 respectively.

[dB]                                                                                                       (Equation 1)

[dB]                                                                                                             (Equation 2)

Using Equation 1, the Steer accuracy for FR1-NTN and FR2-NTN can be calculated as 0.9dB and 1.6dB, respectively. From our understanding, it is reasonable to define the Array accuracy for FR1-NTN and FR2-NTN as 0.5dB and 0.8dB respectively. Using Equation 2 with TRP accuracy 2dB for FR1-NTN and 3dB for FR2-NTN in above table, the transceiver unit accuracyfor FR1-NTN and FR2-NTN can be calculated as 1.8dB and 2.9dB respectively. So the transceiver unit accuracy, Array accuracy, and Steer accuracy for FR1-NTN and FR2-NTN are listed in the following table.

| Frequency range designation | Array accuracy(dB) | Transceiver unit accuracy(dB) | Steer accuracy(dB) |
| --- | --- | --- | --- |
| FR1-NTN | 0.5 | 1.8 | 0.9 |
| FR2-NTN | 0.8 | 2.9 | 1.6 |

The frequency range for Ku-band is 10700 MHz – 14500MHz, the DL frequency is about 11GHz, and 11GHz is between higher frequency of FR1-NTN and lower frequency of FR2-NTN, so comparing to FR1-NTN, it is reasonable to improve transceiver unit accuracy, Array accuracy, and Steer accuracy for Ku-band to make Ku-band implementation to unrestrictive. From our understanding, the transceiver unit accuracy, Array accuracy, and Steer accuracy in the following table is applicable for Ku-band.

| Frequency range | Array accuracy(dB) | Transceiver unit accuracy(dB) | Steer accuracy(dB) |
| --- | --- | --- | --- |
| Ku-band 10700 MHz – 14500MHz | [0.7] | [2.1] | [1.2] |

So, using Equation 2 with above transceiver unit accuracy, Array accuracy, the TRP accuracy for Ku-band can be calculated as sqrt(0.7*0.7+2.1*2.1)= 2.2dB. At same time, using Equation 1 with 2.2dB TRP accuracy and 1.2dB steer accuracy, the EIRP accuracy for Ku-band can be calculated as sqrt(2.2*2.2+1.2*1.2)=2.5dB. So for frequency in Ku-band, [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy is more applicable.

Proposal 1:  Adopt [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy for Ku-band.

If we want to introduction of Ku-band don’t impact on existing requirement, we can align the EIRP accuracy and TRP accuracy for FR1-NTN Ku-band and FR2-NTN Ku-band, it is needed to define separate EIRP accuracy and TRP accuracy for Ku-band for alignment as following example:

### 9.2.2 Minimum requirement for SAN type 1-H and SAN type 1-O Except bands in 10700 MHz -14500 MHz, for each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.2 dB of the claimed value. For bands in 10700 MHz -14500 MHz, for each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±[2.5 dB] of the claimed value. Normal conditions are defined in TS 38.181, annex B [3]. In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal. ### 9.2.3 Minimum requirement for SAN type 2-O Except bands in 10700 MHz -14500 MHz, for each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ± 3.4 dB of the claimed value. For bands in 10700 MHz -14500 MHz, for each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±[2.5 dB] of the claimed value. Normal conditions are defined in TS 38.181, annex B [3]. In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

OTA RE power control dynamic range

In TS 38.108, the RE power control dynamic range was defined for FR1-NTN, and not defined for FR2-NTN. From our understanding, the smaller channel bandwidth is needed to define RE power control dynamic range requirement to control RE power boosting/de-boosting. So existing OTA RE power control dynamic range for SAN type 1-O is applicable for FR1-NTN Ku-band, and no OTA RE power control dynamic range for SAN type 2-O is also applicable for FR2-NTN Ku-band. There is not necessary to align it for FR1-NTN Ku-band and FR2-NTN Ku-band.

Proposal 2: Reuse OTA RE power control dynamic range for SAN type 1-O for FR1-NTN Ku-band, and reuse OTA RE power control dynamic range (N/A) for SAN type 2-O for FR2-NTN Ku-band.

OTA total power dynamic range

According to revised WID[3], the channel bandwidth for SCS for Ku-band was approved as following:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE

The OTA total power dynamic range for SAN type 1-O is defined in Table 6.3.3.2-1 of TS 38.108 copied as below:

Table 6.3.3.2-1: Total power dynamic range

| SAN channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.9 | 10.4 | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |

The OTA total power dynamic range for SAN type 2-O is defined in Table 9.4.3.3-1 of TS 38.108 copied as below:

Table 9.4.3.3-1: Minimum requirement for SAN type 2-O total power dynamic range in FR2-NTN

| SCS | OTA total power dynamic range (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | 18.1 | 21.2 | 24.2 | N/A |
| 120 | 15.0 | 18.1 | 21.2 | 24.2 |

As above WID and existing total power dynamic range for SAN type 1-O and SAN type 2-O, we can find that for FR1-NTN Ku-band, it is needed to add OTA total power dynamic range for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. The total power dynamic range is dependent on Transmission bandwidth configuration, it is sufficient to reuse the Transmission bandwidth configuration for above mentioned CBW and SCS for NR BS type 1-O for FR1-NTN Ku-band, So total power dynamic range for above mentioned CBW and SCS for NR BS type 1-O can be reused for FR1-NTN Ku-band. So, we propose to add OTA total power dynamic range for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz CBWs with 30 kHz SCS, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz CBWs with 15 kHz SCS for FR1-NTN Ku-band as following table.

| SAN channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 25 | 21.2 | 18.1 |  |
| 35 | 22.7 | 19.6 |  |
| 50 | 24.3 | 21.2 |  |
| 70 | N/A | 22.7 |  |
| 100 | N/A | 24.3 |  |

Proposal 3: Add OTA total power dynamic range for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz CBWs with 30 kHz SCS, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz CBWs with 15 kHz SCS for FR1-NTN Ku-band as following table.

| SAN channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 25 | 21.2 | 18.1 |  |
| 35 | 22.7 | 19.6 |  |
| 50 | 24.3 | 21.2 |  |
| 70 | N/A | 22.7 |  |
| 100 | N/A | 24.3 |  |

Spurious frequency range

The spurious frequency range for FR1-NTN in Table 9.7.5.2.2-1 of TS 38.108 is 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band, and the spurious frequency range for FR2-NTN in Table 9.7.5.3.2-1 of TS 38.108 is 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band. The difference is maximum spurious frequency, and the maximum spurious frequency for FR1-NTN is 5th harmonic of the upper frequency edge of the DL operating band, the maximum spurious frequency for FR2-NTN is 2nd harmonic of the upper frequency edge of the DL operating band. For 11GHz Ku-band DL frequency, the 5th harmonic is 55GHz, and 2nd harmonic is 22GHz, they are testable, It is reasonable to reuse existing spurious frequency range for SAN type 1-O for FR1-NTN Ku-band, and reuse existing spurious frequency range for SAN type 2-O for FR2-NTN Ku-band.

From our understanding, the maximum spurious frequency is dependent on upper frequency of FR1-NTN and FR2-NTN. It is not necessary to align the spurious range for FR1-NTN Ku-band and FR2-NTN Ku-band.

Proposal 4: Reuse existing spurious frequency range for SAN type 1-O for FR1-NTN Ku-band, and reuse existing spurious frequency range for SAN type 2-O for FR2-NTN Ku-band.

OTA out-of-band blocking

The ΔfOOB for SAN type 1-O is defined in Table 10.6.2.1-2 of TS 38.108 copied as below:

Table 10.6.2.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |

The ΔfOOB for SAN type 2-O is defined in Table 10.6.3.1-2 of TS 38.108 copied as below:

Table 10.6.3.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 4000 MHz | 1500 |

The agreed operating bands for Ku-band in WF[2] are copied as below:

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

Table 5.2.3-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

From above operating bands for Ku-band, the maximum FUL,high – FUL,low is 14500MHz-14000MHz=500MHz, which is greater than the ΔfOOB =100MHz for SAN type 1-O and less than the ΔfOOB =4000MHz for SAN type 2-O, So it is needed to define ΔfOOB for FUL,high – FUL,low =500MHz, referring to ΔfOOB for BS type 1-O in Table 10.5.2.2-0 of TS 38.104 as following, it is reasonable to add ΔfOOB =60MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz for SAN type 1-O to support FR1-NTN Ku-band.

Table 10.5.2.2-0: ΔfOOB offset for NR operating bands in FR1

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

Proposal 5: Add ΔfOOB =60MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz for SAN type 1-O to support FR1-NTN Ku-band.

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1:  Adopt [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy for Ku-band.

Proposal 2: Reuse OTA RE power control dynamic range for SAN type 1-O for FR1-NTN Ku-band, and reuse OTA RE power control dynamic range (N/A) for SAN type 2-O for FR2-NTN Ku-band.

Proposal 3: Add OTA total power dynamic range for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz CBWs with 30 kHz SCS, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz CBWs with 15 kHz SCS for FR1-NTN Ku-band as following table.

| SAN channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 25 | 21.2 | 18.1 |  |
| 35 | 22.7 | 19.6 |  |
| 50 | 24.3 | 21.2 |  |
| 70 | N/A | 22.7 |  |
| 100 | N/A | 24.3 |  |

Proposal 4: Reuse existing spurious frequency range for SAN type 1-O for FR1-NTN Ku-band, and reuse existing spurious frequency range for SAN type 2-O for FR2-NTN Ku-band.

Proposal 5: Add ΔfOOB =60MHz for 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz for SAN type 1-O to support FR1-NTN Ku-band.

- References

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#114

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107