3GPP TSG-RAN WG4 Meeting # 114bis                                                        R4-2503467

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.23.2.2

Document for:	Discussion

- Background

WF [1] of RAN4#114 concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - ACLR and OBUE

We need to decide whether /how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

In [R4-2501026], we agreed with 1dB degradation of victim BS sensitivity and also derived the best UL/DL spectrum configuration for two SBFD-capable BSs.

When two SBFD-capable BSs with adjacent operating bands configured as {DU}, and two DL sub-bands are adjacent, frequency offset between aggressor channel to victim channel is max, and this configuration can optimize the rejection at victim BS UL sub-band by RF BPF used after PA, as shown in figure 1. The best configuration is not a general requirement, but can be used for some scenario.

Figure 1 Interference rejection diagram

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the BPF after PA can provide 70dB suppression.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two co-located SBFD-capable BSs with adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

The UL and DL spectrum configurations between two co-located base stations need to be coordinated, so the best configuration can be achieved by same operator, but not by different operators. There is no need to define requirements for best configurations and scenarios.

Proposal 3: There is no need to define requirements for special configuration and scenario.

- Summary

In this contribution, we have the following proposals.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two co-located SBFD-capable BSs with adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

Proposal 3: There is no need to define requirements for special configuration and scenario.

- Reference

[1] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114

[2] R4-2501026, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT, RAN4#114