3GPP TSG-RAN WG4 Meeting # 114bis                                                     R4-2503468

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on modification of existing Rx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.23.2.3

Document for:	Discussion

- Background

WF [1] of RAN4#114 concluded that further studies are needed for some Rx RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Rx RF requirement

  - ACS requirement

NR BS ACS is dominated by UE operating in adjacent channel, but for SBFD-capable BS, when it is configured as SBFD slot, the interference signal level in adjacent channel is dominated by BS operating in adjacent channel, it is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

Proposal 1: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

  - In-band blocking

WF:

- For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements.

- For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded.

  - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

    - Option A: 20%

    - Option B: 50%

  - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

  - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%.

  - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%.

- For FR1 MR SBFD BS, FFS.

In WF [1], in-band blocking requirements need to be further studied.

For FR1 WA BS，whether the SBFD is configured as {DU} or {DUD}, the in-band interference signal level of the victim SBFD coexisting with the legacy TDD system of the adjacent channel can cover the interference signal level coexisting with the new SBFD system of the adjacent channel, there is no need to define requirements for special scenario.

Proposal 2: For FR1 WA BS, there is no need to define requirements for special scenario.

According to [2], the simulation results of in-band blocking for FR1 and FR2-1 are shown in table 1 and table 2, and the interference are from TDD DL to SBFD UL.

Table 1: SBFD conducted in-band blocking simulation results (FR1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | Conducted in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 1 | FR1 | Urban Macro -> Urban Macro | -25.4 | 0.3 |
| 5 | FR1 | Micro -> Micro | -32.1 | -17.2 |

Table 2: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 6 | FR2-1 | Urban Macro -> Urban Macro | -51.7 | -39.8 |

For FR1 urban macro scenario, the in-band blocking signals power levels are much higher than the existing IBB requirement.

For FR1 micro scenario, the in-band blocking signals power level of 100% grid shift is close to the existing IBB requirement, but the power level of 10% grid shift is much higher than the existing IBB requirement.

For FR2-1 urban macro scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

Observation 1: The simulation result of IBB for FR1 macro scenario is much higher than existing represent.

Observation 2: In the FR1 micro-scenario, the simulation result of IBB is higher than the existing requirements when the grid shift is 10%, and the simulation result of IBB is close to the existing requirements when the grid shift is 100%.

Observation 3: The simulation results of IBB for FR2-1 urban macro scenario is lower than the existing requirements.

Proposal 3: We propose to specify in-band blocking requirements based on data at 100% grid shift.

Proposal 4: The IBB requirements of FR1 micro scenario can reuse existing requirements.

Proposal 5: The IBB requirements of FR2-1 urban macro scenario can reuse existing requirements.

  - Rx intermodulation

In WF [1], the necessity of new RX intermodulation requirement with one interfering signal needs to be further studied.

- FFS on the following options.

  - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. For some SBFD DL and UL spectrum configurations, the single interference signal scenario is implicitly captured by the SBFD RX blocking requirement, even if the frequency range of the single interference signal is from out-of-band to in-band rather than only in-band.

Observation 4: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 6： The single interference signal scenario isn’t needed.

- FFS on the following options.

  - Option 1:

    - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2: new IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3: RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

  - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

For the power level of the receiver intermodulation interference signal, whether BS2BS CLI needs to be considered depends on the power level values of the interference, and the power level can be simulated. The in-band blocking of the SBFD slot has been simulated by RAN4, and the interference signals are from BS of frequency adjacent system, these interference signals can be considered by legacy Rx intermodulation, and new RX intermodulation requirement with two interfering signals is not necessary.

Observation 5: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 7 New RX intermodulation requirement with two interfering signals is not necessary.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

Proposal 2: For FR1 WA BS, there is no need to define requirements for special scenario.

Observation 1: The simulation result of IBB for FR1 macro scenario is much higher than existing represent.

Observation 2: In the FR1 micro-scenario, the simulation result of IBB is higher than the existing requirements when the grid shift is 10%, and the simulation result of IBB is close to the existing requirements when the grid shift is 100%.

Observation 3: The simulation results of IBB for FR2-1 urban macro scenario is lower than the existing requirements.

Proposal 3: We propose to specify in-band blocking requirements based on data at 100% grid shift.

Proposal 4: The IBB requirements of FR1 micro scenario can reuse existing requirements.

Proposal 5: The IBB requirements of FR2-1 urban macro scenario can reuse existing requirements.

Observation 4: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 6： The single interference signal scenario isn’t needed.

Observation 5: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 7 New RX intermodulation requirement with two interfering signals is not necessary.

- Reference

[1] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114

[2] R4-2415106, “SBFD in-band blocking and dynamic range simulation results”, CATT, RAN4#112bis