3GPP TSG-RAN WG4 Meeting # 114bis                                                       R4-2503469

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on potentially new requirements for SBFD operation

Source: 	CATT

Agenda item:	7.23.2.1

Document for:	Discussion

- Background

WF [1] of RAN4#114 concluded that further studies are needed for potentially new RF requirements, in this contribution, we provide our further analysis of them.

- New requirements for SBFD operation

2.1 Transient period

Issue 3-1-1: Location of transient period between SBFD and non-SBFD

Agreement:

- RAN4 reach the conclusion of the location of transient periods:

  - Case-A: in TA_offset in SBFD symbol(s);

  - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols

    - FFS guard symbol(s) in this case is SBFD symbol or not.

  - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission:

    - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design.

    - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall

      - either send LS to RAN1 to ask for clarification,

      - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact.

  - Case-D: in TA_offset in non-SBFD UL symbol(s).

  - Note: The illustrative figure for Case A, B, C and D is provided below:

In WF [1], agreements of transmitter transient period are shown following.

When the NR DL symbol is switched to the SBFD symbol, the frequency bandwidths of the NR DL full-band and the SBFD DL sub-band are different, and the bandwidth related parts of the design need to be reconfigured for some implementations, such as the filters, the digital part of the bandwidth related part, and the link control, etc. and these switches should also need some time. Similar to the switch from NR DL full-band to SBFD DL sub-band, switches from SBFD DL sub-band to NR DL full-band, from NR UL full-band to SBFD UL sub-band and   from SBFD UL sub-band to NR UL full-band all need some time.

Observation 1: It can take time to switch from the NR DL full-band to SBFD DL sub-band, from NR UL full-band to SBFD UL sub-band, from SBFD DL sub-band to NR DL full-band and from SBFD UL sub-band to NR UL full-band.

Proposal 1: DL sub-band switched to DL full-band and UL sub-band switched to UL full-band and vice versa all need to consider the transient period.

Hence, to minimize the impact on the non-SBFD system and simplify the specification, the transition period between SBFD and non-SBFD should be located within the SBFD slot.

Proposal 2: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot.

2.2 In-channel new requirements

In WF [1], some in-channel new requirements need further analysis.

2.2.1 In-channel adjacent sub-band leakage ratio requirement

Issue 3-2-1: In-channel adjacent subband leakage ratio: General

Agreement:

- New requirement for In-channel adjacent subband leakage ratio:

  - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104)

    - FFS the value for absolute PSD limit

    - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration

Way Forward:

- Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline:

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

The ACLR value can be decided based on feasibility analysis of SI stage. If In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline，the Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104 can be reused.

The naming of this requirement can be ASLR absolute limit.

Proposal 3: The Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104 can be reused for absolute PSD limit

Proposal 4: The naming of this requirement can be ASLR absolute limit.

Proposal 5: The in-channel adjacent sub-band leakage ratio limit can be 45dB.

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | 45dB |

2.2.2 In-channel adjacent subband blocking requirement

Issue 3-3-2: Necessity of in-channel adjacent subband blocking requirement

Agreement:

- New requirement for in-channel adjacent subband blocking should be defined

  - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation.

  - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement.

  - The requirement level shall be driven by feasibility consideration.

Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements

Agreement:

- No necessity to introduce the new requirement for in-channel adjacent subband selectivity.

OTA sensitivity only consider self-interference, the interference from inter-sectors BSs and inter-site BSs can be considered by define in-channel adjacent sub-band leakage ratio requirement and 	in-channel adjacent sub-band selectivity requirement.

In-channel adjacent sub-band selectivity requirement is implicitly covered by in-channel adjacent sub-band blocking, so no definition is needed. The in-channel adjacent sub-band blocking requirement should be defined based on feasibility consideration.

When consider interference from inter-sectors BSs, the isolation between two sectors need to be decided, but at last meeting, companies had different recommendations on the value of the sector isolation, and it may be difficult to reach a consensus. We can assume an isolation value between two sectors, and then define the in-channel adjacent sub-band blocking requirement based on the isolation assumption. The isolation between two sectors can be assumed to be 70dB.

Proposal 6: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs

Proposal 7: The in-channel adjacent sub-band blocking requirement should be defined based on feasibility consideration.

Proposal 8: In-channel adjacent sub-band blocking requirement is to be specified under an assumed isolation value between two sectors.

Proposal 9: The isolation between two sectors can be assumed to be 70dB.

- Summary

In this contribution, we have the following proposals.

Observation 1: It can take time to switch from the NR DL full-band to SBFD DL sub-band, from NR UL full-band to SBFD UL sub-band, from SBFD DL sub-band to NR DL full-band and from SBFD UL sub-band to NR UL full-band.

Proposal 1: DL sub-band switched to DL full-band and UL sub-band switched to UL full-band and vice versa all need to consider the transient period.

Proposal 2: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot.

Proposal 3: The Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104 can be reused for absolute PSD limit

Proposal 4: The naming of this requirement can be ASLR absolute limit.

Proposal 5: The in-channel adjacent sub-band leakage ratio limit can be 45dB.

Proposal 6: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs

Proposal 7: The in-channel adjacent sub-band blocking requirement should be defined based on feasibility consideration.

Proposal 8: In-channel adjacent sub-band blocking requirement is to be specified under an assumed isolation value between two sectors.

Proposal 9: The isolation between two sectors can be assumed to be 70dB.

Reference

[1] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114