3GPP TSG RAN WG4 Meeting #114bis		R4-2503479

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	TP to TR 38.755 on Clause 5

Agenda Item:	7.5.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114, a TP was approved for TR 38.755 [1], where there could still be improved in this contribution in particular for Clause 5.

# 2 TP

< Start of TP>

### 5 Study of methods for reducing the number of UE Rx chains

## 5.1 Scenarios of fragmented carriers

An example to present the case of fragmented carriers is shown in the figure below from [6][7].

Figure 5-1: Example of fragmented carriers [8]

Fragmented carriers are not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, proponents of the new SI have chosen the name as “Fragmented Carriers”, since this is how it is perceived mainly from an operator perspective. Adding a specific name also indicates a special way of handling non-contiguous intra-band carrier aggregation where UEs may support downlink reception of the fragmented spectrum block with a single “receive chain”, “receive path” or “receive branch”.

The conventional DL fragmented carriers reception containing more than one non-contiguous CC in the same band consuming multiple Rx RF chains may limit the UE CA capabilities as the maximum supported number of component carriers (CCs) in a CA combination is limited by the number and capabilities of the Rx RF chains as well as the baseband processor capabilities in the UEs.

From [9], a reference RF architecture of non-contiguous CA was initially introduced in LTE Rel-11, and captured in TR 36.823 as below.

Figure 5-2: Reference RF architecture of non-contiguous CA (Figure 6.2.3.1-1 of TR 36.823)

According to this reference RF architecture, the LNA is shared by two non-contiguous carriers, while the mixer, LPF and A/D are independent for each carrier. This reference RF architecture continues to be used in deriving the NR requirements for non-contiguous CA, i.e., the current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on this partially shared architecture.

## 5.2 RF architectures

The primary motivation of current study item is to reduce the number of UE Rx chains that are used to receive the fragmented carriers. To serve this motivation, different RF architectures were mentioned during early discussions, e.g. from separate RF chains per CC to shared RF chains.

Apple has in [10] provided what in their view is meant by a single Rx chain: “A single Rx chain consists of one main Rx and one diversity Rx for a UE with 2Rx”.

In [11] MediaTek shows a block diagram on the definition of a single Rx chain by applying Apple's definition in [13] of a Rx chain to the main and diversity branch in MediaTek’s block diagram, as shown in Fig. 5-3 and labelled as “Type 1” receiver.

It can be observed Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

Type 1 – Single RX chain

| Single RX chain (consists of one main Rx and one diversity Rx) |  |
| --- | --- |
| Main receiver with RF carrier cantered between CCs | Diversity receiver with RF carrier centred between CCs |

Figure 5-3: Single RX chain as presented in [11], where DAC must mean ADCs, since this is at the receiver.

This is also illustrated in Fig. 5-4 with the main and diversity receiver branch.

Figure 5-4: Single Rx Chain for a 2Rx UE consisting of a main and diversity receiver branch (FDD).

It would be beneficial for further discussion to agree on what the underlying assumptions are, as a basis for a common understanding, to align terminologies which can help companies to ensure they discuss the same thing.

Similar to the previous discussion for 2Rx UEs, it becomes even more inconsistent what is meant by a Rx chain for 4Rx UEs when reviewing literature and previous contributions in RAN4. However, it seems the most predominant interpretation is to have 1 main receiver branch and 3 diversity receiver branches. Meaning, when using Fig. 5-4 as a reference, there will be an addition of two diversity receiver branches.

For the current UE implementation, the descriptions in [10] and [11] assume a separate UE RF chain per fragmented carrier which results in the limitations of the maximum number of supported CCs. For the current UE implementation, a separate UE RF chain per fragmented carrier is assumed.

This study item challenges the need for applying a separate UE Rx chain per fragment. In other words, the ambition is to reduce the number of needed Rx chains for fragmented carrier operation.

As previously mentioned, the concept of fragmented carries is in some way similar to non-contiguous intra-band carrier aggregation (NC IB CA). The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx chain will be impacted by interference especially in the “gap” between the fragments of the carrier. The impact on UE requirements is further discussed in R4-2413340 while the following focuses on the UE receiver architectures.

The in-gap interference will, in a single analogue Rx chain system, pass un-attenuated as the UE does not have the ability to suppress/attenuate the in-gap interference via analogue filtering. Suggested solutions are shown in [9] where the UE, by utilizing additional Rx chains, can attend to this presence of an in-gap interferer. In [9] the single Rx chain system, as also shown in Fig. 5-3, is denoted Type 1 while two other multi-chain systems are denoted Type 2 and Type 3. It can be observed Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

There are two ways that the signal can be treated by separate Rx chains. It may be split at the front-end components (eLNA), which is seen as Type 3, and it may also be split inside the transceiver, which is seen as Type 2.

Type 2 – Internal RX path split:

For Type 2 a full picture without the iLNA (internal LNA) split is discussed here. The UE has two Rx chains active, and this allows the UE to set the LO1 and LO2 at the respective centers of each CC, so that the subsequent filtering (channel filters) may attenuate also the in-gap interferer as shown here. In one example it considers non-collocation, where the in-gap interferer is seen as a “worst case” condition. In the collocated case, the wanted to interference ratio will be less.

Figure 5-5: Type 2 – Internal Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1), it will have to accept an in-gap interferer as shown in the next view of Type 2, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-6: Type 2 – Internal Rx path split. Unused Rx chain is toned-out. Type 2 becomes Type 1.

In this state, the UE is without analogue filtering to suppress the interferer, and RAN4 will have to discuss the relaxation of the receiver requirements for the UE to tolerate some level of in-gap interference presence. Since the requirement is based on the presence of an interfering signal, it is closer related to the ACS requirement than to the Reference sensitivity requirement for non-contiguous intra-band carrier aggregation, and a new set of requirements must capture the conditions and the relaxation for the UE state that uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer.

This means that when a Type 2 implementation is using a subset of its Rx chains operating in fragmented carrier mode the new set of requirements will apply.

Type 3 – External Rx path split:

The second way that the signal can be treated by separate Rx chains is a split at the front-end components by multiple external LNAs (eLNA). This can either be within the main/diversity module, or at an extension from the main/diversity module that allows connection to another receive pin at the transceiver. For Type 3 the study would like to show a full picture and without discussion of detail whether front-end modules splits internally at the eLNA or if there’s a separate front-end module that accompanies the main/diversity modules to extend the connection possibilities towards the transceiver(s). Some solutions also have 2 transceivers that allow split of reception in NC IB CA between the two transceivers. The full system view is that the UE has two Rx chains active, and this allows the UE to set the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here.

Figure 5-7: Type 3 – External Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

Details on the differences between the Type 2 and Type 3 UE architectures may be found in [10], where there is a split at the eLNA (external transceiver LNA) towards two iLNA (internal transceiver LNA) (Type2) and a split only at the DACs (Edit: it should mean ADCs, since this is at the receiver) for CC0 and CC1 (Type 3). However, it is not explaining how the UE actually may receive the two carriers individually.

To extend the understanding of the different choices for transceiver implementation, it would have been chosen to follow the principal presented by other companies and also split into three different types with emphasis on Type 2 and Type 3 in the following. This as Type 1 is the hardware state it is wanted to free up hardware resources i.e. Rx chains as described in the introduction.

It can be noted that there is also the possibility to suppress some of the in-gap interference in the digital domain. However, this does not solve the dynamic range issues given by the near-far problem. It can mitigate some of the adjacent channel interference but only when within proper dynamic range provided from the analog and mixed signal Rx chain. Hence, the focus of this work is on solutions related to the analog Rx path. Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up Rx resources hence the focus is on the analog Rx chains.

If this UE is to transition to a single Rx chain state (Type 1) it will have to tolerate an in-gap interferer as shown in the next view of Type 3, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-8: Type 3 – External Rx path split. Unused Rx chain is toned-out. Type 3 becomes Type 1.

In this state, the UE is in the same conditions as in Type 2, bringing both Type 3 and Type 2 into the state of Type 1 single Rx chain reception, so the relaxation of the requirements is the same for Type 2 and Type 3, and the same set of requirements can apply regardless of how the UE supports non-contiguous intra-band carrier aggregation.

As presented previously it has been shown that with both a Type 2 and a Type 3 transceiver implementation it is possible to separate the Rx chains (i.e. operate as Type 1). When separating or freeing up Rx chains in the Type 2 and Type implementations it is possible to apply these now idle Rx chains for reception of other carries if new RAN4 requirements for this type of operation is introduced to the specification.

## 5.3 Reference architecture and RF performance

In the RAN4#112bis meeting, it was agreed the reference architecture is based on fully shared RF chain [12].

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

  - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

Figure 5-9: Reference architecture of one Rx RF chain

  - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

There are also some discussion on baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study as well as “single FFT” or “separate FFT” assumption evaluation.

Referring to [13], there may be different digital BB chain approaches:

o	Separate digital BB chains per CC after digital down conversion which would enable a dedicated digital BB filter and thus good ACS.

o	Common digital BB chain assuming that CCs are SCS aligned such that a shared FFT/digital filter are used. In this case it may be difficult to maintain good ACS and blocking performance, especially in the gap.

•	Note that if a single FFT is used to receive two non-contiguous CCs/cluster that are SCS aligned, if the CCs are 15kHz SCS the maximum BW is 50MHz and if 30kHz it is 100MHz.

Figure 5-9 illustrates some of the above aspects and terminology. The CCs in the illustration are the co-located operator non-contiguous CCs/clusters to be received.

Figure 5-10 illustration of shared Analog path and shared/separate digital BB paths

For the baseband signal processing, referring to[14], separate FFT is legacy implement to handle DL NCCA carriers since it provides better rejection on interference and unwanted signal. Single FFT that can receive both NCCA carriers including in-gap interference (with poor rejection) at a time. There are some constraint for single FFT. The channel raster of PCC and SCC shall align to orthogonal grid that can avoid unnecessary ICI after FFT. This would need to be guaranteed by network scheduling. And due to the max FFT size is 4096, for the fragment carriers that span larger than 50MHz, only 30KHz or 60KHz SCS would be applicable for the case.

With above assumptions, the evaluated RF performance when there exists in-gap interference applying single FFT together is shown below. The fc of interference was assumed in the middle of two FFT grid as worst-case simulation.

Figure 5-11: SCC: 5M, PCC: 5M + ACS single FFT simulated spectrum plot

Figure 5-12: SCC: 5M, PCC: 5M + IBB1 single FFT simulated spectrum plot

Figure 5-13: SCC: 5M, PCC: 5M + IBB2 single FFT simulated spectrum plot

It can be seen from simulated results, the spectrum of interference was spread out due to allocated on non-orthogonal FFT grid. UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2. And due to wide frequency span for a single FFT, the baseband power consumption could be higher. It is expected single FFT may have worse rejection ratio to interference signal and this is not common baseband implementation for DL NCCA carriers.

If UE apply separate FFT baseband, it is foreseen the digital filter can provide better rejection on the in-gap interference signal so that performance of wanted CCs would not be degraded except when the interference falls in the Rx image range of wanted signal. Separate FFT for DL NCCA carriers has been widely used for modern modem design.

It has been agreed in [15] single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the technical report.

<End of TP>

# Reference

[1] R4-2502925, “TP for TR38.755”, MediaTek Inc., et. al.