3GPP TSG RAN WG4 Meeting #114bis		R4-2503487

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on BS RF requirements for LP-WUS

Agenda Item:	7.26.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#114 has made some progress on BS RF requirements for LP-WUS with the following agreements [1]:

# 1. On dynamic range for LP-WUS ## 1.1 Concept of LP-WUS power boosting and associated detail WF: - Adopt following content as the concept of LP-WUS power boosting. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. - The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement. - FFS on the detail of the minimum power boosting level, e.g. - Whether it can be different for different BS CBW ## 1.2 Consideration on EVM requirement for LP-WUS WF: - For RAN4 simulation campaign and discussion in next meeting, the latest RAN1 agreements on waveform design are expected to be used. - FFS on the following options, others are not precluded. - Option 1: No need to specify transmit signal quality requirement for LP-WUS. - Option 2: Instead of defining transmit signal quality requirement for LP-WUS, just do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

In this contribution, we present our views on open issues.

# 2 Discussion

Declaration on minimum power boosting level

Since it was agreed that the minimum power boosting level for LP-WUS is declared by BS manufacturer and not as a minimum requirement, and considering that different BS channel bandwidth may have different minimum power boosting level, it is informative to declare the pairs of (minimum power boosting level, BS channel bandwidth) for all BS channel bandwidths supporting LP-WUS. However, providing such detailed declarations is not essential, and it should suffice to declare only the pair with the lowest minimum power boosting level among all.

Proposal 1: Among all minimum power boosting levels across different BS channel bandwidths, only the lowest value and its corresponding BS channel bandwidth need to be declared.

EVM requirement

As the LP-WUS signal design in RAN1 approaches completion, it is now time for RAN4 to make necessary decisions on specifying requirements, such as EVM for LP-WUS modulation quality.

First of all, it is obvious that EVM requirement is only applicable for “ON” chips of OOK symbols, and “OFF” chips do not have such requirements since no signal is transmitted.

Proposal 2: RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

Secondly, two scenarios should be differentiated: one is without overlaid OFDM sequence, and the other is with overlaid OFDM sequence. Different scenarios may have different requirements.

Proposal 3: RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

For the case where no OFDM sequence is overlaid, EVM requirement for BPSK can be reused, i.e, 17.5%.

Proposal 4: RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

For the overlaid sequence, since RAN1 is still working on it, RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for this case.

Proposal 5: RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

# 3 Conclusion

In this contribution, we have the following proposals on BS RF requirements for LP-WUS:

Proposal 1: Among all minimum power boosting levels across different BS channel bandwidths, only the lowest value and its corresponding BS channel bandwidth need to be declared.

Proposal 2: RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

Proposal 3: RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

Proposal 4: RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

Proposal 5: RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

# Reference

[1] R4-2502305, “Way Forward for [114][308] NR_LPWUS”, Huawei