3GPP TSG-RAN WG4 Meeting #114bis												R4-2503495

Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.14.2.3

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:	Approval

# Introduction

RAN4 #114 has the following agreement [1].

Issue 1-2: Dynamic MIMO OTA test and UE performance metric

Agreement:

- All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test.  FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF.

- Use the following as a starting point:

  - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

### Sub-topic 2-1 General issues on Rel-19 static MIMO OTA requirements

Agreement:

- Down select the following four bands as a starting point: n3, n7, n77, (n8 or n20) and choose either n8 or n20 to represent low band.

This contribution discuss the issues further.

# Discussion

The frequency range of band n8 and n20 from Table 5.2-1 in 38.101-1 are listed below.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |

From smartphone antenna design point of view, the lower the frequency, the higher the degree of the design difficulty due to lack of space in handheld devices. In addition, antennas covering a larger bandwidth present more challenges than those covering narrower bandwidth. The bandwidth of band n8 and n20 are 35MHz and 30MHz, respectively. In other words, their bandwidth values are similar. Given the above two factors, it would be prudent to select the lower frequency band of n20 for MIMO OTA tests. Because the performance of band n20 should give a good indication of the performance in band n8. Typically band n8 performance should be at par with, if not better than, that of band n20.

Proposal 1: select band n3, n7, n77, n20 for measurement campaign as n20 frequency range is lower than n8 with a similar bandwidth.

It was agreed in the last meeting to combine data from UMa and UMi channels with separate measurement for UMa and UMi channels. The associated data processing is straightforward. The remaining detail could be the bin size to create CDF curves.

Proposal 2: use bin size of 0.5 dB to create CDF curves for the combined data from UMa and UMi channels.

# Conclusion

This contribution provides the following proposals.

Proposal 1: select band n3, n7, n77, n20 for measurement campaign as n20 frequency range is lower than n8 with a similar bandwidth.

Proposal 2: use bin size of 0.5 dB to create CDF curves for the combined data from UMa and UMi channels.

# Reference

- R4-2502296 WF for [114][328] MIMO_OTA_Ph3