3GPP TSG-RAN WG4 Meeting #114bis												R4-2503496

Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.14.2.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA tests

Document for:	approval

# Introduction

The WF [1] from the last RAN4 meeting captures the following issue. This contribution addresses the issue.

Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2

Agreements:

  - RAN4 further discuss the performance metric for UE declared only support scenario 2.

  - Details on performance metric data processing should also be discussed.

# Discussion

Scenario 2 is browsing mode and could be considered impacting only a hemisphere as far as performance is concerned. The positioning guideline in clause 6.2 of 38.870 states that device main display is tilted 45 degrees from vertical. Under those circumstances, OTA performance over a hemisphere containing the satellite should be representative enough for scenario 2. In other words, the same test procedure as in 38.870 would be used with the range of  changed from (0,) to (0, /2) and the factor from 4 to 2 as shown below for TRP and TRS equations.

TRP= 12πθ=0π/2ϕ=02πEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ

TRS=2πθ=0π/2ϕ=02π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ

Proposal 1: performance metric for NTN UE supporting only scenario 2 should be defined over a hemisphere containing the satellite.

# Conclusion

This contribution provides the following proposal.

Proposal 1: performance metric for NTN UE supporting only scenario 2 should be defined over a hemisphere containing the satellite.

# Reference

- R4-2502370 Way Forward for [114][327] TRP_TRS_Ph3