3GPP TSG-RAN WG4 Meeting #114bis	R4-2503501

Wuhan, China, Apr. 7-11, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made [1]:

### Sub-topic 1-2: RRM impacts of SBFD operation #### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). #### Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

Additionally, the following topics have been captured in the topic summary [2]:

### Sub-topic 2-1: RRM impacts of SBFD operation #### Issue 2-1-1a: CSI-RS L1 measurement – collision with UL #### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types #### Issue 2-1-3: Requirements for scheduling restriction #### Issue 2-1-4: Requirements for RACH requirements #### Issue 2-1-5: Requirements for UL PC and or spatial relation update #### Issue 2-1-6: Requirements for MG and BWP switch #### Issue 2-1-7: Requirements for UL resource muting

Discussion

Requirements for CSI-RS based L1 measurement

CSI-RS colliding with dynamically scheduled UL transmission

In the RAN4 #113 meeting, an LS was sent to RAN1 about CSI-RS based measurements distributed over SBFD and non-SBFD symbols, and collision between CSI-RS measurement and dynamic UL transmissions on SBFD symbols [R4-2420165]. RAN1 has answered the questions in [R1-2501560], referring to clauses 11.1 and 11.1.1 in TS 38.213 copied below:

TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”

According to the RAN1 LS response, the UE does not receive the CSI-RS if the UE is dynamically scheduled to transmit PUSCH, PUCCH, SRS or PRACH in the set of symbols.

Therefore, we see that the measurement period may be impacted if the CSI-RS transmission overlaps with a dynamically scheduled UL transmission on SBFD symbols. In order to take this fact into account in the requirements, we propose:

Longer CSI-RS based L1 measurement period would be expected if the CSI-RS transmission overlaps with dynamically scheduled UL transmission on SBFD symbols.

CSI-RS puncturing in frequency domain in SBFD symbols

The following other issues related to CSI-RS based L1 measurement were discussed in RAN4 #114:

Sub-topic 2-1: RRM impacts of SBFD operation Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, depending on UE measurement capability - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

RAN4 has agreed that existing accuracy requirements for CSI-RS based measurements can be reused for DU operation in case the number of consecutive CSI-RS PRBs is no less than the existing threshold.

For DUD operation, three cases were discussed in RAN4#114 meeting with different assumptions regarding the number of CSI-RS RBs in the first (X1) and second (X2) DL subband. One of the main concerns is that splitting the RBs in the two subbands for DUD configuration could potentially lead to worse accuracy performance than having all 48 RBs (or 24 for RLM/BFD/CBD) contained in one of the subbands. Moreover, it was also raised that worse accuracy could be obtained when measuring the 40+8 RBs across the two subbands, as compared to measuring only 40 RBs on one of the subbands. This could be a consequence of additional noise captured by the UE DL receiver in case the UL subband is included as part of the measurement bandwidth.

To address these concerns, we think that RAN4 needs to specify additional conditions for which existing accuracy requirements for CSI-RS based measurement should apply in case of SBFD DUD operation, for instance:

- In the simplest case, existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

- Existing requirement should also apply for Case 1 and Case 2, i.e. if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the current threshold.

  - In other words, distributing the PRBs as 48PRB + 8 PRBs should not bring worse performance than the case with 48PRB + 0 PRBs.

- For Case 3, if the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, RAN4 may agree on a new threshold or condition on the minimum number of CSI-RS RBs that shall be present in each DL subband. For instance, it can be specified that  (X1 + X2) >= X3, while both X1>X4 and X2>X4 i.e. the number of CSI-RS RBs in each subband shall be larger than a new threshold, e.g. X4=20.

On the accuracy requirements for CSI-RS based L1 measurements, RAN4 should specify one or more conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation. Consider one or more of the following options:

  - Existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

  - Existing requirement should also apply if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the existing threshold (Case 1 and Case 2 in RAN4#114 agreement).

  - If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, a new threshold X4 can be introduced on the minimum number of CSI-RS RBs that shall be present in each DL subband, i.e. X1>X4 and X2>X4.

- In case the new specified conditions are not fulfilled, the existing accuracy requirements can be relaxed to account the negative impact of the measurement on SBFD symbols.

CSI-RS measurement period when CSI-RS is configured across DL subbands

In the last RAN4 meeting, the CSI-RS measurement period was discussed if the CSI-RS resources are split in two DL subbands. Two options were proposed. In the first option, the existing measurement period is kept since it is assumed that the UE can measure both DL subbands at the same time. In the second option, the existing measurement period is doubled, considering that the UE cannot measure both subbands at the same time.

Our view is that the UE is able to measure both DL subbands with a single FFT and puncturing of the UL subband could be done through removal of the subcarriers in the UL subband. Similar UE behaviour has been applied in the subcarrier blanking techniques. Therefore, there is no reason to double the CSI-RS measurement period in case the CSI-RS configuration spans two DL subbands.

The CSI-RS based L1 measurement period shall not be scaled because CSI-RS resource is configured across DL subbands.

The measurement period for CSI-RS based L1 measurement shall be defined with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

CSI-RS distributed over SBFD and non-SBFD symbols

The RAN4 measurement requirements for CSI-RS L1-RSRP and L1-SINR, and for CSI-RS based RLM/BFD/CBD are based on the periodicity of the CSI-RS resources. In the RAN1 LS response, RAN1 replied that the agreement below is applicable to CSI-RS based L1-RSRP/SINR:

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Currently, the L1-RSRP and L1-SINR measurement period is derived based on the periodicity of CSI-RS resource. If only the CSI-RS transmission occasions that overlap with a given symbol type are used to measure the L1-RSRP or the L1-SINR, the measurement period needs to be discussed.

According to RAN1 agreement, a valid symbol type (SBFD or non-SBFD symbol) is configured by the network and only the CSI-RS transmission occasions that overlap with the given symbol type are used to measure the L1-RSRP or the L1-SINR.

RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period assuming the valid symbol type (SBFD or non-SBFD symbols) is indicated.

One possible solution is to redefine the measurement period requirement for L1-RSRP/SINR measurements as a function of the overlapping between the configured CSI-RS resources and the configured symbol type. For instance, the measurement period may be scaled by a factor 2 if only every second CSI-RS resource takes place in the configured symbol type.

The L1-RSRP and L1-SINR measurement period is defined considering the overlapping between the configured CSI-RS resources and the configured symbol type.

In addition, as the measurement on SBFD symbols are expected to be subject to higher interference levels. RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in order to fulfill the same accuracy requirements.

RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in the CSI-RS based L1-RSRP performance requirements.

Requirements for CSI-RS based L3 measurement

The problem of CSI-RS being punctured in SBFD symbols is also relevant for CSI-RS L3 measurements. For instance, we expect that CSI-RS that is transmitted from a neighbour cell for the purposes of inter-cell mobility would need to be punctured in case the CSI-RS resources are located in a SBFD symbol (of the neighbour cell) and overlap with the UL sub band (of the neighbour cell). This was discussed by RAN2 in RAN2#128 where the following agreement was reached:

- RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In RAN4 #114, it has been agreed that:

- For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

The agreement from the last RAN4 meeting consider only the DU case. In this case, we do not expect that the configured resources of the neighbor cell or the serving cell will overlap with the UL subband hence puncturing is not needed. For the DUD case, there is the possibility that the CSI-RS resources are configured with an overlap with the UL subband. RAN4 is still discussing the applicability of requirements depending on the CSI-RS puncturing and the number of resources available within one or two DL subbands.

We believe that this discussion of the puncturing of the CSI-RS resources (discussed in the context of L1 measurements) is also relevant for the CSI-RS layer 3 measurement case. In our contribution to the last RAN4 meeting, we discussed the following options:

- The UE has information about the neighbour cell SBFD configuration

- For the intra-frequency measurement, the UE assumes that the neighbour cell valid CSI-RS resources are the same as the serving cell.  There are three possible cases:

  - The neighbour cell SBFD configuration is the same as the serving cell.

  - The neighbour cell SBFD configuration is different from the serving cell,

  - The neighbour cell is not configured with SBFD.

In our view, in order to measure CSI-RS from neighbor cell, the UE would need to be aware of the SBFD configuration from the neighbour cell, e.g. at least whether the neighbouring cell operates in SBFD mode and the potential UL-DL RB split in SBFD symbols. Without being aware of the neighbour cell SBFD configuration, in case the SBFD configuration is different from the serving cells, as in cases 2b and 2c above, the UE might puncture incorrectly the CSI-RS for L3 measurements, impacting the measurement performance.

Without being aware of the SBFD configuration of the neighbor cell, the UE cannot correctly derive the valid neighbor cell CSI-RS resources, which might impact the measurement performance.

To avoid blocking the progress of other working groups on this issue, we think RAN4 should discuss requirements for CSI-RS based L3 measurement under the assumption that the UE is aware or provided with information of the SBFD configuration of neighbouring cells. RAN4 may inform RAN2 on this assumption which may then trigger RAN2 to discuss the signaling details.

For CSI-RS based L3 measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells.

If the UE is provided with the SBFD configuration of the neighbor cell, it can derive the resources available for measurement also in the DUD scenario both in intra-frequency measurements and inter-frequency measurements. Then for the intra-frequency case, the requirements are applicable if at least the same resources as the serving cell are available in DL subband of SBFD symbols on the neighbor cell.

Radio Link Monitoring and Beam Failure Detection

RAN1 has sent the LS R1-2501560 to RAN4, addressing the questions from RAN4 related to CSI-RS based RLM and BFD in RAN4 #113. RAN1 has informed RAN4 that Configuration 1 and Configuration 2 are not applicable to RLM/BFD resource configuration. Furthermore, the configuration of a valid symbol type is not applicable to CSI-RS configured for RLM/BFD.

RAN1 replied to RAN4 that the agreement concerning the configuration of valid symbol type for CSI derivation is applicable to L1-RSRP/SINR, but not to RLM/BFD.

- RLM/BFD can be based on SSBs and/or CSI-RS. Both CSI-RS and SSB can occur in DL symbols and SBFD symbols. CSI-RSs, however, can be punctured in case the configured CSI-RS resources overlap with UL subbands or guardbands. SSBs, on the other hand, are expected to be confined into the DL subband. Additionally, no UL transmission is expected in SSB symbols, i.e. no CLI is expected.

We envision the following cases:

- The configuration of CSI-RS resource for RLM/BFD (i.e. RLM/BFD-RS) is limited to one symbol type:

  - In this option, one RLM-RS is configured in only one symbol type and the UE does not expect to be configured with RLM-RS that occurs in different symbol types. There is no ambiguity on the UE behavior and the existing requirements can be reused for RLM. There is no need to update signaling to inform the valid symbol type at the UE.

  - It is up to the network to ensure that the RLM-RS is configured in the same symbol type, either SBFD or DL symbols. But this would restrict network configuration of CSI-RS resources and there is no such limitation for CSI-RS resource for L1 measurement.

- The configuration of CSI-RS resource for RLM/BFD (i.e. RLM/BFD-RS) can be across different symbol types. In this case, there are two possible approaches for the RLM/BFD evaluation:

  - The evaluation is done in only one symbol type by the UE. The valid symbol type can be configured by the network similarly as for CSI-RS L1 measurement. In this case, the RAN4 requirements need to be adapted for example, to consider in the evaluation period only the RLM-RS occasions that overlap with a given symbol type

  - The evaluation is done in both symbol types. This option is not optimal, because the interference conditions and valid resources can be different in different symbol types. Furthermore, it is unclear whether the RLM procedure, thresholds, counters needs to be updated.

Considering the analysis above, we propose that:

RAN4 to assume the CSI-RS configuration for RLM/BFD is limited to one symbol type, or the configuration is not limited to one symbol type but the network indicates the valid symbol type for CSI-RS based RLM/BFD.

Requirements for scheduling restriction

DL/UL transition

During the RAN4#113 meeting, it has been agreed to check RAN1 agreements and potential impact to RAN4 scheduling restrictions due to DL/UL transition.

In RAN1 #116, there was an agreement regarding the collisions between receptions in DL subband, and transmissions in UL subband captured below:

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

According to the scheduling availability requirements in TS 38.133, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured and on K data symbols before and after each consecutive SSB symbols to be measured within SMTC window duration. In both FR1 and FR2-1, K equals to 1 or 2 (60 kHz SCS).

The Tx/Rx transition time is specified in TS 38.211 as summarized below. RAN1 has not had any discussions regarding transition time in the current Rel-19 work item, so we assume that these existing requirements should also apply for SBFD operation.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3. … Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx Transition time; FR1; FR2. NTx-Rx; 25600; 13792. NRx-Tx; 25600; 13792.

Tc is approximately 5.09 ns, so the transition time in FR1 is approximately 13 µs, and 7 µs in FR2. The duration of 1 OFDM symbol for different subcarrier spacings is given in the table below. Note that in all cases, the duration of an K OFDM symbos are longer than the transition time.

| Subcarrier spacing [kHz] | OFDM symbol duration [µs] |
| --- | --- |
| 15 | 66.73 |
| 30 | 33.37 |
| 60 | 16.68 |
| 120 | 8.35 |

In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

The existing RAN4 scheduling restriction requirements for SSB measurements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Inter-frequency SSB measurement without gap

In the last RAN4 meeting, it has been discussed if new scheduling restrictions are needed for the case in which the SBFD UE is capable of performing inter-frequency measurements without gaps and is configured to do so. The proposal discussed in RAN4 #113 was to define DL scheduling restrictions for the case in which the neighbor cell SSBs are at least partially overlapping with the UL subband of the serving cell as in the example in the Figure below:

Figure 1 – Neighbour Cell SSBs partially overlapping with Serving Cell UL subband.

Some of the conditions for an inter-frequency measurement without gap are copied below from TS 38.133:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

Additionally, it is important to discuss the impact of some relevant RAN1 agreements and working assumptions:

Working Assumption (RAN1#118b) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

The working assumption is that for SSBs symbols configured with SBFD subbands, the SSB is assumed to be within the DL subband. For the SBFD aware UE, only DL receptions within DL usable PRBs are allowed. Our understanding is that the working assumption applies to Serving cells configured with SBFD. The scenario we are discussing is the one in which the neighbor cell either does not support SBFD or has a different SBFD configuration from the serving cell.

Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

In meeting RAN#117, it has been agreed that:

Agreement (RAN1#117) Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

If a UE is configured to measure an SSB, let’s say the neighbor cell SSB, we assume that the downlink reception is prioritized over configured UL transmission as per RAN1 agreement copied above. So, while configured to perform inter-frequency measurements without gaps, there should be UL scheduling restrictions. In FR1, According to TS 38.133, there are already UL scheduling restrictions specified in clause 9.3.9.3.1. There are also DL scheduling restrictions specified if the subcarrier spacing from the SSB and the data are different. In FR2, there are already DL and UL scheduling restrictions in place.

Therefore, for the scenario in which the neighbor cell SSB overlaps with the UL subband, if the UE is capable of performing inter-frequency measurements without gaps, and the neighbor cell SSB is overlapping with the serving cell UL subband, the UE should still be able to receive DL signals from the serving cell within the DL subband, so there is no need to define new DL scheduling restrictions.

There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

RACH requirements

In RAN4 meeting #113, it has been proposed to update the RACH behavior due to SBFD operation [2]:

- Proposals - Proposal 1 (LGE): - RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check. - Proposal 2 (vivo, E///, MTK): - No impact on RACH requirements due to SBFD operation.

In TS 38.133, the correct behaviour when not receiving Random Access Response is:

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

Currently, the correct UE behavior is to update the PRACH transmission power if the Random Access Response is not received. In the RAN2 #127bis[3], RAN2 has agreed that:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

According to the agreement above, the UE is allowed to switch to legacy RACH occasions. Therefore there will be two different behaviours depending on whether the configured number of times of RACH attempts in SBFD RACH occasions has been reached or not. Therefore, our view is that RAN4 migth need to update the correct behaviour when not receiving Random Access Response after a certain number of attempts in SBFD symbols, depending on how the agreement above is captured in RAN2 specification.  However, there is still one open aspect about the case when the UE select legacy ROs first. This issue has not been finalized yet in RAN2, so we propose to wait for the agreement in RAN2 before RAN4 works on updating the RACH requirements.

RAN4 to wait for RAN2 conclusions before updating the correct behavior when the UE does not receive the random access response in SBFD symbols.

Conclusion

The following Observations and Proposals were made:

- According to the RAN1 LS response, the UE does not receive the CSI-RS if the UE is dynamically scheduled to transmit PUSCH, PUCCH, SRS or PRACH in the set of symbols.

- Longer CSI-RS based L1 measurement period would be expected if the CSI-RS transmission overlaps with dynamically scheduled UL transmission on SBFD symbols.

On the accuracy requirements for CSI-RS based L1 measurements, RAN4 should specify one or more conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation. Consider one or more of the following options:

  - Existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

  - Existing requirement should also apply if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the existing threshold (Case 1 and Case 2 in RAN4#114 agreement).

  - If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, a new threshold X4 can be introduced on the minimum number of CSI-RS RBs that shall be present in each DL subband, i.e. X1>X4 and X2>X4.

- In case the new specified conditions are not fulfilled, the existing accuracy requirements can be relaxed to account the negative impact of the measurement on SBFD symbols.

The CSI-RS based L1 measurement period shall not be scaled because CSI-RS resource is configured across DL subbands.

The measurement period for CSI-RS based L1 measurement shall be defined with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

According to RAN1 agreement, a valid symbol type (SBFD or non-SBFD symbol) is configured by the network and only the CSI-RS transmission occasions that overlap with the given symbol type are used to measure the L1-RSRP or the L1-SINR.

RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period assuming the valid symbol type (SBFD or non-SBFD symbols) is indicated.

The L1-RSRP and L1-SINR measurement period is defined considering the overlapping between the configured CSI-RS resources and the configured symbol type.

RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in the CSI-RS based L1-RSRP performance requirements.

Without being aware of the SBFD configuration of the neighbor cell, the UE cannot correctly derive the valid neighbor cell CSI-RS resources, which might impact the measurement performance.

For CSI-RS based L3 measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells.

RAN1 replied to RAN4 that the agreement concerning the configuration of valid symbol type for CSI derivation is applicable to L1-RSRP/SINR, but not to RLM/BFD.

RAN4 to assume the CSI-RS configuration for RLM/BFD is limited to one symbol type, or the configuration is not limited to one symbol type but the network indicates the valid symbol type for CSI-RS based RLM/BFD.

In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

The existing RAN4 scheduling restriction requirements for SSB measurements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

RAN4 to wait for RAN2 conclusions before updating the correct behavior when the UE does not receive the random access response in SBFD symbols.

- References

- R4-2502699, WF on SBFD RRM, Huawei

- R4-2500536, summary [114][222] NR_duplex_evo, Huawei