3GPP TSG-RAN WG4 Meeting #114bis	   R4-2503508

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda Item:	7.11.6

Source:	Eutelsat Group

Title: 	Ku Band RRM updates required to support FR1-NTN and FR2-NTN 	numerology

Document for:	Discussion and Decision

# 1	Background

The WID "Introduction of Ku bands for NR NTN" comprises provision for RRM work and was updated at RAN#106 [1] to include support for mobile VSAT connectivity to NGSO:

- Specify RRM requirements including timing requirements for mobile VSAT served by NGSO

  - NOTE:	RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

This contribution provides some further detail on the RRM specifications required to support the Ku bands being introduced for NR-NTN to support VSAT UE operation using both FR1-NTN and FR2-NTN numerologies.

For the purposes of this document, we adopt the definition of ‘numerology’ from [2]:

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

The WID [1] was further updated at RAN#107 [3] to include the option of 15 kHz SCS in addition to 30 kHz and 120 kHz.

- Discussion

## Summary of the comparison with existing RRM requirements applicable to Ka band (“FR2-NTN”)

Most of the existing Rel-18 Ka band RRM requirements [4] are expected to apply to Rel-19 Ku operating bands with the following additional aspects:

- Ku operating bands are to be defined for both FR1-NTN and FR2-NTN frequency ranges which, therefore, overlap in the Ku band.

- In order to accommodate Ku operating bands in NR-NTN for the considered numerologies, specifically 15 kHz, 30kHz SCS in FR1-NTN and 120kHz SCS in FR2-NTN, FR1-NTN frequency range needs to be extended up to 14.5 GHz and FR2-NTN frequency range needs to be extended down to 10.7 GHz.

- FR1-NTN upper bound as specified in Rel-18 is 7.125 GHz, and RRM procedures are based upon the assumption of an omnidirectional antenna for UEs operating in this frequency range. VSAT UEs operating in the 10.7 GHz – 14.5 GHz require the use of a directional antenna. Due to the extension up to 14.5GHz, the FR1-NTN RRM requirements for Ku bands need to adopt the directional UE antenna requirements already specified for FR2-NTN.

- A Ku band UE is expected to support operation using either FR1-NTN numerology or FR2-NTN numerology; there is no requirement for a Ku band UE to support both numerologies.

- In addition to the existing RRM requirements specified for NTN in Rel-18, Ku bands operation also requires the support for mobile VSATs served by NGSO.

Consideration of corrections to Rel-18 FR2-NTN numerology related requirements (if any) are outside the scope of this contribution. Any essential corrections should be treated as maintenance of Rel-18 as they would apply to Ka band; Rel-19 Ku band, using FR2-NTN numerology would be in addition to any Rel-18 corrections.

  - Ku impact taking TS 38.133 v18.8.0 [4] as baseline

A preliminary analysis of the changes necessary (or desirable for reasons of clarity) to the RRM requirements to support Ku band operation using either FR1-NTN or FR2-NTN numerology are presented in Table 1. Where applicable, proposals for changes have been made.

Note: This list is presented for discussion and may not be a complete list of the changes required (i.e., there may be omissions).

For this stage of the work only the Core Requirements (sections 1- 7) are considered; Performance Requirements and test cases are not considered in this contribution.

Table 1: Selected Core requirement sections of TS 38.133

| Section Number | Section/ sub-section title | Ku band Discussion, Observations, Proposals |
| --- | --- | --- |
| 3.3 | Abbreviations (as used throughout the specification) | Discussion: GEO, GSO, LEO and NGSO are defined in TS 38.133 but their use in the body of the specification is not always consistent. This may lead to confusion when comparing requirements within the specification and with other RAN4 specifications referenced (e.g., TS 38.101-5). The following definitions are generally accepted: GEO: Geostationary Earth Orbit (a subset of GSO where the satellite appears stationary to a user on earth) GSO: Geosynchronous Orbit (the orbital duration matches that of the earth - 23 hours, 56 minutes, and 4 seconds - but not all GSO are geostationary) LEO: Low Earth Orbit (a subset of NGSO) NGSO: Non-Geosynchronous Orbit (LEO is an example of NGSO) Observation 1: GSO/GEO and LEO/NGSO are used interchangeably throughout the specification. Proposal 1: Align orbital descriptors used in the body of the text in all sections; unless critical to the interpretation of a clause or requirement: - Replace LEO with NGSO - Replace GEO with GSO |
| 4.2.2.12 | Measurements of inter-frequency NR cells with NTN carrier | Discussion: The FR1-NTN measurement requirements imply an omni-directional antenna, or VSAT capable of supporting dual [multiple?] reception. For NTN operation, two or more frequencies could be used in adjacent spot beams from the same SAN. Inter-frequency measurements by the UE could be used to identify the optimal spot beam which may vary even with Earth Fixed Cell (EFC) due to movement of the UE. Note: Some NR NTN VSAT UE could support more than one Rx chain (some proprietary VSAT, using either Electronically Steered Antenna (ESA) or more than one mechanically steered antenna, have this capability today). Observation 2: FR2-NTN VSAT UE and FR2-NTN (FR2-NTN is always assumed to be a VSAT UE in Rel-18) requirements are not mentioned in clause 4.2.2.12. However, some FR1-NTN requirements may be relevant depending upon UE capability (to be discussed). Proposal 2: The differentiation of requirements in clause 4.2.2.12 should not be based on the Rel-18 frequency range definition but on the UE/ antenna type and capability. |
| 4.2C | Cell Re-selection for NR UE for Satellite Access |  |
| 4.2C.1 | Introduction | Discussion: Baseline VSAT UE are assumed to support only one directional receiver beam ('see' one satellite at any instance). See Note in 4.2.2.12. Observation 3: Ku band UE operating in FR1-NTN or FR2-NTN will employ directional antennas (VSAT). Proposal 3: modify text to generalise the term FR2-NTN to use VSAT: The requirements specified in clause 4.2C apply to FR2-NTN [single receive beam] VSAT UE with the following assumption: - no inter-satellite measurement is configured; - single SAN Tx beam per radio cell in DL; - same UE Rx beam is used for both serving and neighbouring cells which belong to the same satellite. |
| 4.2C.2 | Requirements | No change (performance depends on VSAT UE capabilities, single or multiple receive beamformer). |
| 4.2C.2.1 | UE measurement capability | Discussion: Satellite operators using Ku band service links typically have access to 2 GHz of downlink spectrum. Today, typical downlink channelization is in 250 MHz segments (a bandwidth not natively supported in Rel-18) hence 8 channels. Future systems based on 5G NR NTN may retain the same channelization. Observation 4: In Ku band, with 2 GHz of downlink spectrum used per satellite network operator, there may be more than 7 active RF carriers per satellite network. Proposal 4: For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least: - Intra-frequency carrier [in the case of VSAT UE with single receive beam, non-simultaneous monitoring intra-frequency carriers from more than one SAN], and - Depending on UE capability, FR1-NTN UE 7 NR inter-frequency carriers and for all VSAT UE up to [8] inter frequency carriers. |
| 4.2C.2.2 | Measurement and evaluation of serving cell | Discussion: FR1 scaling factor can be used for FR1-NTN once the definition of FR1-NTN is extended in to the Ku band (no change required). |
| 4.2C.2.3 | Measurements of intra-frequency NR cells | Discussion: FR1 measurements require the UE to be able to 'see' multiple satellites simultaneously. This requirement is not aligned with the present VSAT UE limitation in 4.2C.1. Measurement accuracy in Ku band using FR1-NTN may be less accurate than for FR1 TN operating at lower frequencies (to be confirmed). One reasonable approach would be for Ku band requirements to reuse FR2-NTN UE measurement tolerances. Practical VSAT implementations may support more than the baseline assumption of a single Rx chain/ single receive beam at any instant. Observation 5: The FR1 requirements in clause 4.2C.2.3 do not apply to VSAT with a single receive beam/ Rx chain. The FR2-NTN requirements apply to single Rx chain VSAT UE in FR1-NTN or FR2-NTN. Proposal 5: For [single receive beam] VSAT UE FR2-NTN, Kmulti_SMTC,i = 1. |
| 4.2C.2.4 | Measurements of inter-frequency NR cells | Discussion: The FR1 requirements in section 4.2C.2.4 do not apply to VSAT UE with one single RX chain. The FR2-NTN requirements would apply to a baseline assumption single Rx chain VSAT. Observation 6: The FR2-NTN requirements in clause 4.2C.2.4 apply to VSAT UE in FR1-NTN or FR2-NTN. Proposal 6: For [single receive beam] VSAT UE FR2-NTN, Kmulti_SMTC,i = 1. |
| 6.1C | Handover for SAN |  |
| 6.1C.1.2 | NR SAN FR1 – NR SAN FR1 Handover | This clause is not applicable to VSAT UE. See clause 6.1C.1.3. |
| 6.1C.1.3 | NR SAN FR2-NTN – NR SAN FR2-NTN Handover | Discussion: The requirements would also apply to Ku band VSAT UE [with single receive beam] in both FR1-NTN and FR2-NTN. |
| 6.1C.2.4 | NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover | Discussion: The requirements would also apply to Ku band VSAT UE [with single receive beam] in both FR1-NTN and FR2-NTN. |
| 6.1C.3 | NR SAN Satellite switching with re-synchronization |  |
| 6.1C.3.2 | NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization | Discussion: The requirements could also apply to Ku band VSAT UE [with multiple receive beam capability] operating in either FR1-NTN or FR2-NTN numerology. No specific proposal at this time. |
| 7.1C | UE transmit timing for Satellite Access | See R4-2503139 [5] for detailed changes proposed for this section and sub-sections. |
| 7.1C.2 | Requirements |  |
|  | Table 7.1C.2-1: Te_NTN Timing Error Limit | FR1-NTN will apply to Ku band using FR1-NTN numerology. |
|  | Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO | FR2-NTN will apply to Ku band using FR2-NTN numerology. |
|  | Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO | FR2-NTN will apply to Ku band using FR2-NTN numerology. |
|  | Table 7.1C.2-3: The Value of NTAoffset for VSAT in FR2-NTN | Discussion: Section 7.1.2 does not explicitly identify a value for NTAoffset for FR1-NTN (see Table 7.1.2-2: The Value of NTAoffset ), Observation 7: The table in Section 7.1C (Table 7.1C2-3) could be generalized from just FR2-NTN to include both FR1-NTN and FR2-NTN VSAT UE implementations. Proposal 7 a): Modify title to "The Value of NTAoffset for VSAT". Modify part of Note 1: "If UE is not provided with the information n-TimingAdvanceOffset, the default value of N_"TA offset" is set as 0 for FR1-NTN and FR2-NTN bands." Alternative Proposal 7 b): FR1-NTN value for Ku band either [0] or [25600] – appropriate value to be discussed in RAN4. |
| 7.1C.2.1 | Gradual timing adjustment |  |
|  | Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate | Applicability depends upon SCS - no change required due to Ku band numerology alone. Change may be required to support mobile VSAT served by NGSO to support very high-speed cases (aircraft). |
| 7.2C | UE timer accuracy for satellite access | Discussion: No technical change required in section 7.2C; values corresponding to the SCS in use apply. Observation 8: Editorial modification only necessary / desirable to clarify the requirement in clause 7.2C. Proposal 8: The requirements in clause 7.2C are applicable for both UE served by SAN in FR1 and all VSAT UE types.served by SAN in FR2-NTN. |
| 7.3C | Timing advance for satellite access |  |
| 7.3C.2.2 | Table 7.3C.2.2-1: UE Timing Advance adjustment accuracy | Discussion: No technical change required to section 7.3C; values corresponding to the SCS in use apply. Observation 9: Editorial modification only necessary/ desirable clarify the requirement in clause 7.3C.2.2. Proposal 9: Modify text to read: "table 7.3C.2.2-1, for FR1-NTN (all UE types)…" |
|  | Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN | No change required. |

# 3	Conclusion

The Ku band WID requires RAN4 to update the RRM requirements. Consideration of changes to FR2-NTN requirements (if any) for Rel-18 (Ka band) are outside the scope of this contribution.

The following observations and proposals are made based on our analysis of [4].

Observation 1: GSO/GEO and LEO/NGSO are used interchangeably throughout the specification.

Proposal 1: Align orbital descriptors used in the body of the text in all sections; unless critical to the interpretation of a clause or requirement:

•	Replace LEO with NGSO

•	Replace GEO with GSO

Observation 2: FR2-NTN VSAT UE and FR2-NTN (FR2-NTN is always assumed to be a VSAT UE in Rel-18) requirements are not mentioned in clause 4.2.2.12. However, some FR1-NTN requirements may be relevant depending upon UE capability (to be discussed).

Proposal 2: The differentiation of requirements in clause 4.2.2.12 should not be based on the Rel-18 frequency range definition but on UE/Antenna type and capabilities.

Observation 3: Ku band UE operating in FR1-NTN or FR2-NTN will employ directional antennas (VSAT). 

Proposal 3: modify text to generalise FR2-NTN to VSAT:
The requirements specified in clause 4.2C apply to FR2-NTN single receive beam VSAT UE with the following assumption:
	- no inter-satellite measurement is configured; 
	- single SAN Tx beam per radio cell in DL; 
	- same UE Rx beam is used for both serving and neighbouring cells which belong to the same 	satellite.

Observation 4: In Ku band, with 2 GHz of downlink spectrum used per satellite network operator, there may be more than 7 active RF carriers per satellite network. 

Proposal 4: Proposal 4: For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:
- Intra-frequency carrier [in the case of VSAT UE with single receive beam, non-simultaneous monitoring intra-frequency carriers from more than one SAN], and
- Depending on UE capability, FR1-NTN UE 7 NR inter-frequency carriers and for all VSAT UE up to [8] inter frequency carriers.

Observation 5: The FR1 requirements in clause 4.2C.2.3 do not apply to VSAT with a single receive beam/ Rx chain. The FR2-NTN requirements apply to single Rx chain VSAT UE in FR1-NTN or FR2-NTN.

Proposal 5: For [single receive beam] VSAT UE  FR2-NTN,  Kmulti_SMTC,i = 1.

Observation 6: The FR2-NTN requirements in clause 4.2C.2.4 apply to VSAT UE in FR1-NTN or FR2-NTN. 

Proposal 6: For [single receive beam] VSAT UE  FR2-NTN,  Kmulti_SMTC,i = 1.

Observation 7: The table in Section 7.1C (Table 7.1C2-3) could be generalized from just FR2-NTN to include bot FR1-NTN and FR2-NTN VSAT UE implementations.

Proposal 7 a): Modify title to "The Value of NTAoffset for VSAT". Modify part of Note 1: "If UE is not provided with the information n-TimingAdvanceOffset, the default value of N_"TA offset"  is set as 0 for FR1-NTN and FR2-NTN bands." 
Alternative

Proposal 7 b): FR1-NTN value for Ku band either [0] or [25600] – appropriate value to be discussed in RAN4.

Observation 8: Editorial modification only necessary/ desirable to clarify the requirement in clause 7.2C.

Proposal 8: The requirements in clause 7.2C are applicable for both UE served by SAN in FR1 and all VSAT UE types. served by SAN in FR2-NTN.

Observation 9: Editorial modification only necessary/ desirable clarify the requirement in clause 7.3C.2.2.
Proposal 9: Modify text to read: "table 7.3C.2.2-1, for FR1-NTN (all UE types)…"

# 4	References

[1] RP-243314 “Revised WID: Introduction of Ku bands for NR NTN”, RAN#106 Madrid (December 2024)

[2] TS 38.300, "NR; Overall description; Stage 2" - v18.5.0 (March 2025)

[3] RP-250799 “Revised WID: Introduction of Ku Bands for NR NTN”, RAN #107, Incheon (March 2025)

[4] TS 38.133 "NR; Requirements for support of radio resource management" - v18.8.0 (December 2024)

[5] R4-2503139 “Draft CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO”, Eutelsat Group, RAN4 #114bis (April 2025)