3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503511

Wuhan, China 7th – 11th April, 2025

Source:	Qualcomm Incorporated

Title:	MPR and A-MPR for Intra-band contiguous UL CA for PC1.5

Agenda item:	7.1.1.1.2

Document for:	Approval

# 1.	Introduction

For this WI companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In RAN4#114 tentative MPR numbers were presented for contiguous CCs with contiguous RBs [1].  In this paper we present further views on the MPR/A-MPR for contiguous CCs with contiguous and non-contiguous RBs.

# 2. 	Discussion

In RAN4#114 a WF was created that stated the following [1]:

## 1.1 MPR values for intra-band contiguous ULCA with contiguous RB allocation (with TxD)

Agreement:

- Define separate MPR/A-MPR requirements for handheld and FWA

- Companies are encouraged to bring A-MPR values in following meetings

- The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2

- Having the following PC1.5(2Tx) MPR values for equal PSD as starting point to be further checked in upcoming meetings

  - Companies can further evaluate the unequal PSD (6dB) impacts in next meeting.

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [7] | [9] | [6.5] | [8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [9] | [9.5] | [8.5] | [9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [6-6.5] | [9.5] | [5.5-6] | [9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [8.5] | [9.5] | [8.5] | [9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

Based on our latest measurements we suggest the following MPRs for PC1.5 contiguous CC with contiguous RBs

| Modulation |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 9 | 4 | 8.5 |
|  | QPSK | 4.5 | 9 | 4 | 8.5 |
|  | 16QAM | 4.5 | 9 | 4 | 8.5 |
|  | 64QAM | 7 | 9 | 6.5 | 8.5 |
|  | 256QAM | 9 | 9.5 | 8.5 | 9 |
| CP-OFDM | QPSK | 5 | 9.5 | 4.5 | 9 |
|  | 16QAM | 5 | 9.5 | 4.5 | 9 |
|  | 64QAM | 6 | 9.5 | 5.5 | 9 |
|  | 256QAM | 8.5 | 9.5 | 8.5 | 9 |
|  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

Table1: MPRs for contiguous CC/contiguous RBs for PC1.5 with 2Tx

Proposal 1: For contiguous CC/contiguous RBs adopt the following MPR for handheld and FWA devices

| Modulation |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 9 | 4 | 8.5 |
|  | QPSK | 4.5 | 9 | 4 | 8.5 |
|  | 16QAM | 4.5 | 9 | 4 | 8.5 |
|  | 64QAM | 7 | 9 | 6.5 | 8.5 |
|  | 256QAM | 9 | 9.5 | 8.5 | 9 |
| CP-OFDM | QPSK | 5 | 9.5 | 4.5 | 9 |
|  | 16QAM | 5 | 9.5 | 4.5 | 9 |
|  | 64QAM | 6 | 9.5 | 5.5 | 9 |
|  | 256QAM | 8.5 | 9.5 | 8.5 | 9 |
|  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

We also performed A-MPR measurements for n41 CA_NS_04 for contiguous CC/contiguous RBs. Based on the measured results it was seen that for cases where RBstart ≤ 0.33*BWchannel_CA/0.18MHz the A-MPR required for PC1.5 was 1.5dB higher than for PC3 and PC2 for UE handhelds and 1.5dB higher than for PC3 and PC2 for FWA devices. Therefore, it is recommended to adopt the following wording for PC1.5 in section 6.2A.3.1.1.1 of TS38.101-1 [2].

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPR+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPR+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Proposal 2: PC1.5 A-MPR for n41 CA_NS_04 for contiguous CC/contiguous RBs should be revised as follows in section 6.2A.3.1.1.1 of TS38.101-1 [2]:

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPR+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPR+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Measurements were also done for PC1.5 MPR for contiguous CC/non-contiguous RBs. Measurements indicate that for the inner region about 0.5dB more MPR is required, for outer1 about 3dB more MPR is required and for outer2 about 1dB to 1.5dB of MPR is required compared to PC2 2Tx. Our proposal is given in table 2 below.

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 4.5 | 10.5 |  |
|  | QPSK | 4.5 | 10.5 |  |
|  | 16QAM | 4.5 | 10.5 | 16 |
|  | 64QAM | 6.5 | 10.5 |  |
|  | 256QAM | 8 | 10.5 |  |
| CP-OFDM | QPSK | 5 | 11 | 16.5 |
|  | 16QAM | 5 | 11 |  |
|  | 64QAM | 6.5 | 11 |  |
|  | 256QAM | 9 | 11 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |

Table2: MPRs for contiguous CC/non-contiguous RBs for PC1.5 with 2Tx

Proposal 3: The following table should be adopted for PC1.5 MPR for contiguous CC/non-contiguous RBs

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 4.5 | 10.5 |  |
|  | QPSK | 4.5 | 10.5 |  |
|  | 16QAM | 4.5 | 10.5 | 16 |
|  | 64QAM | 6.5 | 10.5 |  |
|  | 256QAM | 8 | 10.5 |  |
| CP-OFDM | QPSK | 5 | 11 | 16.5 |
|  | 16QAM | 5 | 11 |  |
|  | 64QAM | 6.5 | 11 |  |
|  | 256QAM | 9 | 11 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |

# Conclusion

In this paper we present our measurements for contiguous CC with contiguous RB MPRs for various modulations and waveform types.

Proposal 1: For contiguous CC/contiguous RBs adopt the following MPR for handheld and FWA devices

| Modulation |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 9 | 4 | 8.5 |
|  | QPSK | 4.5 | 9 | 4 | 8.5 |
|  | 16QAM | 4.5 | 9 | 4 | 8.5 |
|  | 64QAM | 7 | 9 | 6.5 | 8.5 |
|  | 256QAM | 9 | 9.5 | 8.5 | 9 |
| CP-OFDM | QPSK | 5 | 9.5 | 4.5 | 9 |
|  | 16QAM | 5 | 9.5 | 4.5 | 9 |
|  | 64QAM | 6 | 9.5 | 5.5 | 9 |
|  | 256QAM | 8.5 | 9.5 | 8.5 | 9 |
|  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

Proposal 2: PC1.5 A-MPR for n41 CA_NS_04 for contiguous CC/contiguous RBs should be revised as follows in section 6.2A.3.1.1.1 of TS38.101-1 [2]:

###### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPR+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPR+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Proposal 3: The following table should be adopted for PC1.5 MPR for contiguous CC/non-contiguous RBs

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 4.5 | 10.5 |  |
|  | QPSK | 4.5 | 10.5 |  |
|  | 16QAM | 4.5 | 10.5 | 16 |
|  | 64QAM | 6.5 | 10.5 |  |
|  | 256QAM | 8 | 10.5 |  |
| CP-OFDM | QPSK | 5 | 11 | 16.5 |
|  | 16QAM | 5 | 11 |  |
|  | 64QAM | 6.5 | 11 |  |
|  | 256QAM | 9 | 11 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |

# References

[1] R4-2502863, WF on TN HPUE, 17-21 February 2025

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.0.0