3GPP TSG-RAN WG4 Meeting #114bis                          R4-2503517

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.2

Source:	Apple

Title:	Discussion on CSI Prediction: CSI reporting requirement framework for CSI prediction

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Metric for RAN4 Requirements

In the previous meeting RAN4#112, the following agreements was reached for CSI prediction.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

using relative throughput as the test metric for inference, the baseline denominator for relative throughput still requires further clarification.

In the current PMI test case, RAN4 uses random PMI as the baseline to evaluate whether the UE-reported PMI is accurate enough. This approach is analogous to a scenario where no UE reporting occurs.

Issue 2-1: Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Issue 2-2: Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Issue 2-3: Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

Issue 2-5: Prediction delay requirements

Agreement:

- Prediction delay includes:

  - Measurement for prediction

    - FFS on whether the legacy measurement delay can be reused

  - Inference delay

  - Other components are FFS

The following issues/agreements were discussed in RAN4#114:

Issue 1-2: Reference model for CSI prediction

Agreement:

- Assume no reference AI model for CSI prediction as baseline

  - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Issue 1-3: Training and test data consistency

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Issue 1-4: Generalization

Agreement:

- Postpone this discussion until the requirement and test definition is clear

According to the agreement, relative throughput should be tested for prediction accuracy. The γ value comparing predicted PMI versus random PMI can be used in the test. To demonstrate the performance gain of AI-based CSI prediction, the γ value from the AI-based prediction would differ from the γ value observed in previous non-AI tests.

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

During RAN4#112bis it was discussed whether the new requirements on AI/ML-based functionality should be worse or better than the requirements on the legacy CSI feedback:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Given that the requirements for legacy non-AI/ML CSI feedback mechanisms are well-established, it would be advantageous and practical to evaluate the performance of AI/ML-based CSI prediction using the same test parameters that have already been defined.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

For CSI monitoring, since SGCS is the direct KPI of the AI/ML model, SGCS may be considered as a monitoring KPI metric. The relevant KPIs and evaluation metrics for CSI prediction from RAN1, as outlined in TR 38.843 section 7.1.2, are noted below.

7.1.2 CSI feedback enhancement Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: … - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

As outlined above, for NW-side performance monitoring (Type 2) in functionality-based LCM, predicted CSI along with its corresponding ground truth is sent to the network. The same mechanism can be applied for testing, where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Proposal 3: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

- On Rel-19 AIML-based CSI reporting

Since time domain prediction reporting mechanism is already introduced in Rel-18, RAN1 at 116bis meeting agreed to assume R18 eType II doppler codebook for AI/ML-based prediction:

Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression). Conclusion For Rel-19 study on CSI prediction only, consider UE-sided model only. Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

- Proposal 4: RAN4 to assume R18 eType II Doppler codebook for reporting AI/ML based CSI predictions.

- Simulation Results for AI/ML based CSI prediction

Simulation assumptions for AIML based CSI prediction are defined in Table 1.

Simulation assumptions:

Training Data: We generated TDLA30 channels with Nrx=4, Ntx =8, cross-pol antennas, rank 1. We generate CSI samples every 5ms, each CSI sample is a tensor of dimension: [52(tones) 2(Re,Im) 4 8]. We generated 200 sequences of CSI samples, each sequence had 400 time samples for a total of 80,000 samples. We used the 180 sequences for training and the other 20 sequences for testing.

AI/ML model: We employed a spatiotemporal LSTM model, with a sliding window of observations =5. A fully connected layer was used after LSTM. The output size to the fully connected network was equal to the spatial flattened size and the input size was determined by the hidden size of the LSTM. The LSTM also has a number of layers.  The model is trained with MSE loss and Adam optimizer in Pytorch.

Simulation methodology: We have the following variables in our simulations: the genie CSI sample every 5ms, the predicted sample, and the sample and hold sample (SH) which is the last sample from the observation window. The baseline prediction is the SH scheme and we aim at evaluating the gains of AI/ML prediction compared to SH. The comparisons are done with 2 metrics: NMSE and SGCS. For SGCS, we compute the covariance matrix across tones for each CSI sample (predicted, genie, and SH) , and we compute the principal eigenvector.

Fig 1. Sliding window based prediction

Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |

Simulation results with 20 Hz Doppler spread:

LSTM has a hidden size of 512 units and 3 layers. The training curve across epochs is shown in Fig2.

Fig 2. Sliding window AI/ML prediction Doppler =20Hz

After training is completed, we obtain the following results with the testing data:

(1) Genie and SH: NMSE= -6.1dB, SGCS= 0.8

(2) Genie and AI/ML predicted: NMSE=-11.5dB, SGCS =0.92

Therefore, we observe a 5.4dB gain in NMSE and a 14% increase in SGCS with AI/ML prediction.

Simulation results with 50 Hz Doppler (With overfitting):

LSTM has a hidden size of 512 units and 3 layers. The training curve across epochs is shown in Fig 3. We observe an overfitting issue.

After training is completed, we obtain the following results with the testing data:

(1) Genie and SH: NMSE= -0.37dB, SGCS= 0.458

(2) Genie and AI/ML predicted: NMSE=-4.73dB, SGCS =0.67

Fig 3. Sliding window AI/ML prediction Doppler spread =50Hz (overfitting)

To remedy the overfitting, we trained a less complicated LSTM network.  This is shown in Fig 4.

Fig 4. Sliding window AI/ML prediction Doppler spread =50Hz (less overfitting)

After training is completed, we obtain the following results with the testing data:

(1) Genie and SH: NMSE= -0.37dB, SGCS= 0.458

(2) Genie and AI/ML predicted: NMSE=-6.75dB, SGCS =0.77

Therefore, we observe a 6.3dB gain in NMSE and a 68% increase in SGCS with AI/ML prediction.

We tried another configuration of LSTM as shown in Fig 5 with less overfitting. However, the testing results were not better. This demonstrates that there is a tradeoff between training and testing loss and the optimum tradeoff needs to be obtained.

Fig 5. Sliding window AI/ML prediction Doppler spread =50Hz (1 layer, 128 units)

Simulation results with 100 Hz Doppler spread:

Simulation results with 100Hz doppler spread are under current investigation, but based on initial results the performance of both SH and AI/ML are poor.

Fig 6. SGCS comparison between Sample and Hold and LSTM

Fig 7. NMSE comparison between Sample and Hold and LSTM

Observation 2: Significant gains are observed with AI/ML prediction over Sample and Hold for Doppler spreads up to 50 Hz. Further investigations are needed for higher Doppler spreads

Proposal 5: RAN4 to perform simulation results alignment among companies. If simulation results don’t match, companies should be encouraged to share CSI prediction datasets and/or and AI/ML model architectures

Proposal 6: Simulation results show that there is a tradeoff between training and testing losses (overfitting). RAN4 to discuss the need for optimal models as a function of Doppler spread.

Proposal 7: For generalization purposes investigate the granularity of Doppler spread where a new AI/ML model is needed to maintain performance.

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Observation 2: Significant gains are observed with AI/ML prediction over Sample and Hold for Doppler spreads up to 50 Hz. Further investigations are needed for higher Doppler spreads

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

Proposal 3: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

- Proposal 4: RAN4 to assume R18 eType II Doppler codebook for reporting AI/ML based CSI predictions.

Proposal 5: RAN4 to perform simulation results alignment among companies. If simulation results don’t match, companies should be encouraged to share CSI prediction datasets and/or AI/ML model architectures.

Proposal 6: Simulation results show that there is a tradeoff between training and testing losses (overfitting). RAN4 to discuss the need for optimal models as a function of Doppler spread.

Proposal 7: For generalization purposes investigate the granularity of Doppler spread where a new AI/ML model is needed to maintain performance.

# Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”