3GPP TSG-RAN WG4 Meeting #114                                                       R4-2503523

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Study of RAN4 impacts for measurement event prediction for AIML Mobility in NR

Source:	Apple

Agenda item:	7.21.2.2

Document for:	Discussion

Introduction

In the last RAN2 meeting (RAN2 #128 [1]), the simulation framework, assumptions, and parameters for Measurement Event Prediction for handover events were discussed. Among the agreements captured in the Chair notes [1], definitions were agreed for 4 indirect measurement event prediction cases:

- Indirect measurement event prediction for temporal domain case A: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for temporal domain case B: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for frequency domain: Measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at a time instance occurs, without further involvement of an AI/ML model​

- Indirect measurement event prediction for spatial domain: Measurement result(s) is predicted by an RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Discussion

In the TP for TR 38.744 [4], the following is captured:

There are two methods to predict measurement event, namely indirect and direct measurement event prediction as illustrated in Figure 1-1 and Figure 1-2 respectively. Figure 1-1 Indirect measurement event prediction In indirect measurement event prediction for intra-frequency temporal domain case A, temporal domain case B or spatial domain, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. In indirect measurement event prediction for frequency domain, measurement result(s) is predicted by a RRM measurement prediction model for frequency domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Figure 1-2 Direct measurement event prediction As illustrated in Figure 1-2, the input of the model with direct prediction is the same as indirect prediction as illustrated in Figure 1-1 and additional other input is also allowed. Measurement event is predicted directly without any intermediate measurement results.

The RAN2 agreements from RAN2#127bis meeting validates the need for performance requirement for event prediction from RAN4.

Agreements on Inputs/outputs and KPIs for event prediction from RAN2#127bis [3]

- For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighbouring cells.  The final output is the expected occurrence time of a certain measurement event (ex. event A3).

- For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

- A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT).

For the indirect measurement event prediction problem in the temporal domain Case A, a Radio Resource Management (RRM) measurement prediction model generates predictions of future measurement results for all cells under consideration.

The process operates as follows:

- Input: A set of historical RRM measurements collected across all cells within a sliding observation window (OW).

- Prediction: The model generates predicted RRM values for all cells at future measurement instances within a sliding prediction window (PW).

- Event Prediction: These predicted values are then analyzed to determine whether a measurement event (e.g., Event A3) will occur at a future time instance.

Figure 2 illustrates the sliding observation and prediction windows used in this process.

Figure 2 Sliding observation/prediction windows in temporal domain case A prediction

The following was agreed in the previous RAN2 meeting (RAN2 #128 [1]):

- For both temporal domain RRM measurement prediction Case A and Case B, upon receiving a Measurement Event prediction report from the UE, e.g., containing a prediction that an event A3 will occur at a future time, the network initiates handover preparation

Temporal prediction Case A is related to the case where the UE or network is trying to predict RSRP values into the future, e.g. to help with mobility decisions.

In the indirect approach Case A, the cells predictions (e.g. predicted RSRP) are used in the A3 event inequality formulas (which are defined in TS 38.331), instead of the actual cell quality measurements. Therefore, we have the following phases: observation window => measurement predictions => evaluation of events.

Based on RAN 2 simulation results [2] the current simulation and setting result in a minimal error for AI/ML and sample and hold algorithms (comparing to the total number of A3 events) and if we would like to show the gain of the AI/ML based algorithm more challenging scenarios by considering measurement error and different UE mobility patterns are needed for evaluation. Moreover, the false detection of mobility events could impose higher costs on the network by leading to incorrect mobility decisions, whereas missing a mobility event might be mitigated by detecting it in subsequent predictions. In light of the simulation observation, we have the following proposal:

Proposal 1: RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (which incurs a higher cost)

The following figure shows how indirect Event A3 works in a simulation [3].

Figure 3. Indirect measurement event prediction

In Figure 1, at the current time T1, the UE uses the Case A prediction model to predict future L3 RSRP values within the prediction window (PW, T1–T4) based on historical L3 RSRP measurements from the serving and neighboring cells during the observation window (OW, T0–T1). Subsequently, we evaluate whether an Event A3 report would be triggered by analyzing the predicted L3 RSRP values within the PW.

In the example provided, the Event A3 entering condition is predicted to be first satisfied at T2 and sustained throughout the time-to-trigger (TTT) period (T2–T3). Consequently, the output of the Event A3 prediction is labeled ‘predicted at T3’. If no Event A3 condition is predicted within the PW, the output defaults to ‘not predicted’.

For indirect measurement event prediction, the process involves two steps:

- A Radio Resource Management (RRM) measurement prediction model (intra-frequency temporal domain Case A) generates future measurement results.

- These predicted results, optionally combined with actual historical measurements, are used to determine whether a measurement event will occur at a future time instance. Notably, this step does not require additional involvement of an AI/ML model.

The accuracy of event prediction is evaluated using F1 score, precision, and recall, as defined in RAN2#128. Simulation results for F1-score as a function of UE speed are presented in [3]. Since F1 scores depend on RSRP prediction accuracy, which itself relies on RSRP measurement accuracy, the impact of measurement errors must be further investigated.

Proposal 2: For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score)

Proposal 3:  RAN4 to investigate modeling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

Another aspect of measurement prediction is the enhancement of mobility and handover (HO) performance when the time and spatial density of measurements remain unchanged, meaning there is no reduction in measurements. In this use case, future measurements predicted using AI/ML are leveraged to optimize mobility decisions.

The measurements may be predicted by the UE and either reported to the network or utilized within the UE to derive further predictions, such as forecasting handovers, radio link failures, or measurement events. The specific use cases and decisions regarding how the predicted measurements are employed to optimize mobility performance, along with their signaling impact, are still under further study in RAN2

Ultimately, the interplay between achievable prediction accuracy and mobility performance is crucial. The evaluation of the benefits provided by AI/ML-assisted mobility should consider KPIs related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will shape the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures

Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

Proposal 5: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization.

For RRM measurement prediction (e.g., L3-RSRP), metrics such as prediction accuracy (RMSE, MAE) and statistical reliability are typically used. For event prediction (e.g., Event A3), additional considerations are necessary, such as the temporal alignment of predicted events with ground truth occurrences and probabilistic confidence in predictions.

Proposal 6: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

- Baseline Metrics from L3-RSRP Prediction

  - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

- Event-Specific Temporal Accuracy

  - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

  - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

- Combined or Down-Selected Metrics

  - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

    - Jointly used for comprehensive performance assessment, or

    - Independently selected depending on use-case priorities.

The final selection and weighting of metrics should account for the following aspects, to be resolved in RAN2 discussions:

- AI/ML Model Type

  - Whether the model performs indirect prediction (e.g., event derived from predicted RSRP) or direct prediction (e.g., event forecasted directly by the AI/ML model).

- Reporting Requirements

  - Whether metrics should include probabilistic confidence (e.g., likelihood of an event occurring within a time window) or focus solely on deterministic predictions.

- Network Signaling Constraints

  - Practical limitations on reporting granularity (e.g., time-domain resolution) to the serving cell.

Conclusion

This paper we discuss RAN4 impacts on measurement accuracy for measurement event prediction for AIML Mobility in NR.

Proposal 1: RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost)

Proposal 2: For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score)

Proposal 3: RAN4 to discuss modeling measurements errors as a function of SINR. The propagation environment varies across serving/neighbouring cells. Measurement errors depend on the cell-specific SINR

Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

Proposal 5: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization.

Proposal 6: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

- Baseline Metrics from L3-RSRP Prediction

  - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

- Event-Specific Temporal Accuracy

  - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

  - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

- Combined or Down-Selected Metrics

  - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

    - Jointly used for comprehensive performance assessment, or

    - Independently selected depending on use-case priorities.

- References

[1] Chair notes, RAN2 #128 meeting.

[2] R2-2500919, Evaluation of measurement event prediction

[3] R2-2500640, Simulation results for measurement event prediction and system level performance

[4] R2-2500287 Text Proposal of 38.744 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025