3GPP TSG-RAN WG4 Meeting #114bis                                 	                        R4-2503524

# Wuhan, China, April 7 – April 11, 2025

Agenda item:	7.1.1.3.2

Source:	Apple Inc.

Title:	On Core Requirements for MIMO Layer Support for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

  - During the last RAN4#114 meeting it was agreed that RAN4 is unable to conclude on simulation assumptions to determine feasibility for handheld UEs. Because of this, it was also agreed that 6-layer performance requirements will be only defined for FWA, and 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. For this discussion paper, we will focus on the agreement that the fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications. Finally, another open item is still the discussion of optionality of the 6 layers feature. The agreement in place is that RAN4 should further discuss if 6-Layer support should be considered an optional feature per the following. In this context, the idea is to introduce 6 MIMO layers support as an optional feature only for FWA.

# Discussion

During RAN4#114 companies debated largely about the feasibility of 6 MIMO layers for 6Rx handheld UEs. After many months of discussions, it was agreed that RAN4 was “unable to conclude on simulation assumptions to determine feasibility for handheld UEs”. In this regard, further conclusions are relatively straightforward:

- 6-layer performance requirements will be only defined for FWA.

- 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

  - Observation #1: Differentiated performance requirements for handheld UEs and FWA devices can captured by UE applicability rules in 38.101-4.

In addition, during RAN4#114 several companies were interested in the possibility of including RAN2 signalling to capture the fact that 6-layer is only supported for FWA. On the other hand, there were companies not in favour of this approach, arguing that applicability rules already serve this purpose.

  - Observation #2: During RAN4#114 companies debated the possibility of including RAN2 signalling to capture the fact that 6-layer is only supported for FWA.

Since this was a contentious topic, we believe it’s worth revisiting during this upcoming meeting.

Proposal #1: RAN4 to discuss the potential need of RAN2 signalling to capture the outcome of Core Requirements discussion of number of MIMO layers for 6Rx UEs.

In order to capture not only in RAN4 specification, but also in others, a few changes might be needed. First, at the component carrier level, the IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding component carrier of one band entry of a band combination. Here, one of the existing fields is maxNumberMIMO-LayersPDSCH, and current specification only indicates 2, 4 or 8 MIMO layers MIMO-Layers enumerator that defines it.

  - Observation #3: Information Element FeatureSetDownlinkPerCC only enumerates values of twoLayers, fourLayers and eightLayers for maxNumberMIMO-LayersPDSCH.

Proposal #2: Enhance signalling by adding value sixLayers to information element maxNumberMIMO-LayersPDSCH in FeatureSetDownlinkPerCC.

  - Another complementary possibility is to capture the outcome of the number of MIMO layers discussion at the cell level configuration. The PDSCH-ServingCellConfig IE is used to configure PDSCH parameters that are common across UE’s bandwidth parts (BWPs) of one serving cell. The field maxMIMO-Layers in PDSCH-ServingCellConfig IE indicates the maximum number of MIMO layers to be used for PDSCH in all BWP’s of a serving cell. RAN4 could also discuss whether is worth including additional text to RAN1/RAN2 specifications that clarifies the use of maxMIMO-Layers in PDSCH-ServingCellConfig in case there is a limitation at the UE level via FeatureSetDownlinkPerCC is decided.

  - Observation #4: Enhancing signalling in maxNumberMIMO-LayersPDSCH in FeatureSetDownlinkPerCC could also be paired with clarifying text in 38.331 specification.

Proposal #3: Include in description of the description of maxNumberMIMO-LayersPDSCH that value sixLayers apply to FWA devices only -- text to be discussed during online session.

In any case, if the outcome of this discussion agrees to recommend any modification to UE signalling, involvement of RAN2 group should take place and RAN4 should send an appropriate LS will be needed.

Proposal #4: RAN4 to send an LS to RAN2 to capture the outcome of the RAN4#114bis discussion.

# Conclusions

Our observations and proposals are summarized below:

  - Observation #1: Differentiated performance requirements for handheld UEs and FWA devices can captured by UE applicability rules in 38.101-4.

  - Observation #2: During RAN4#114 companies debated the possibility of including RAN2 signalling to capture the fact that 6-layer is only supported for FWA.

Proposal #1: RAN4 to discuss the potential need of RAN2 signalling to capture the outcome of Core Requirements discussion of number of MIMO layers for 6Rx UEs.

  - Observation #3: Information Element FeatureSetDownlinkPerCC only enumerates values of twoLayers, fourLayers and eightLayers for maxNumberMIMO-LayersPDSCH.

Proposal #2: Enhance signalling by adding value sixLayers to information element maxNumberMIMO-LayersPDSCH in FeatureSetDownlinkPerCC.

  - Observation #4: Enhancing signalling in maxNumberMIMO-LayersPDSCH in FeatureSetDownlinkPerCC could also be paired with clarifying text in 38.331 specification.

Proposal #3: Include in description of the description of maxNumberMIMO-LayersPDSCH that value sixLayers apply to FWA devices only -- text to be discussed during online session.

Proposal #4: RAN4 to send an LS to RAN2 to capture the outcome of the RAN4#114bis discussion.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024