3GPP TSG-RAN WG4 Meeting #114bis                                       	                    R4-2503525

# Wuhan, China, April 7 – April 11, 2025

Agenda item:	7.1.1.3.2

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# Discussion

This contribution addresses the next steps after deciding the discussion on maximum number of MIMO layers during the core requirements phase.

Observation #1: Since RAN4 was unable to conclude on simulation assumptions to determine feasibility for handheld UEs, 6 MIMO layers will only be supported in FWA devices.

  - Proposal #1: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will be only apply for FWA devices, and 2) for up to 4 MIMO layers, performance requirements will be defined and will apply to both FWA and handheld UE devices.

Based on this, defining the number of layers to be configured for each tests implied by the above proposal should be a core decision. We believe that based on previous experience during Rel-18 8Rx discussion, only a single PDSCH test should suffice for each case.

Observation #2: The use of 6 MIMO layers in 6Rx FWA devices is targeted for high throughput scenarios. For 6Rx FWA and Hanheld devices, performance requirements for 6Rx and 4 MIMO layer is sufficient.

Proposal #2: For PDSCH requirements, consider a single test for 6 MIMO layers for FWA devices, and for 4 MIMO layers, consider a single test applicable to both FWA devices and handheld UEs. Use test parameters in Table 5.2.4.2.1-2 in 38.101-4 as a starting point.

During Rel-18 discussion on 8Rx it was raised the proposal that for 2CW test cases, the scenario to test should be based on CWs optimized with different MCS. Part of this comes from concerns that if a simple TDL model is used with no spatial correlation between layers, then no spatial component is introduced, and each layer exhibits the same average post equalization SINR. From a UE point of view, what MCS are configured to each CW is nothing critical and the UE signal processing is exactly the same for each CW.

  - Observation #3: For the PDSCH test with 2CWs, channel estimation and MIMO detection per CW is the same regardless of the number of CWs in the transmission.

  - Proposal #3: For PDSCH requirements for 6 MIMO layers, consider same MCS value and same propagation conditions for both codewords (CW).

Moving forward, in terms of configuration setup, we collect a number of important parameters in the following observations and proposals below:

Observation #4: For testing and performance requirement purposes, there’s no justification to increase the number of transmit antennas beyond 8 when considering 6Rx.

Proposal #4: Consider a transmit/receive antenna configuration of 8Tx/6Rx for Rank 6, and a transmit receive antenna configuration of 4Tx/6Rx for Rank 4.

In terms of MCS configuration, since feasibility study is concluded, the need to operate a very high SNR is not the most important focus. Since performance requirements are mostly targeted to test correct UE implementation at this point, the SNR operating point and the configured MCS can be of lower value.

Observation #5: As opposed to previous feasibility study, MCS Table 2 is not really needed since 64QAM can provide a good balance in terms of operating SNR and throughput.

Proposal #5: Consider up to 64QAM (MCS Table 1) for setting PDSCH performance requirements.

Regarding the assumption on TxEVM impairments, we continue proposing a typical value based on prior experience and demodulation performance requirements discussions. Specifically, we suggest 6% for 64QAM and 3% for 256QAM, which already represent lower values than the minimum base station (BS) requirements.

Observation #6: During Rel-18 8Rx performance requirements discussions, TxEVM assumptions were an important consideration. Same for Rel-19 6Rx number of MIMO layers discussion.

  - Proposal #6: For PDSCH consider a TxEVM value of 3% for 256QAM and 6% for 64QAM.

Regarding the DMRS configuration, up to four options were considered during the previous 6 MIMO layers feasibility study. However, for performance purposes, a typical configuration is enough, which can be either Type-1 DMRS or Type-2 DMRS.

Observation #7: Type-1 and Type-2 DMRS are typical configurations observed in the field.

  - Proposal #7: For both 4 and 6 MIMO layers requirements consider Type-1 DMRS.

Observation #8: As opposed to handheld devices, FWA devices typically experience no mobility.

  - Proposal #8: For up to 4 MIMO layers consider the use of 1 additional symbol DMRS to target mobility scenarios. For 6 MIMO layers consider no additional DMRS symbol to specifically target the FWA deployment scenario.

The purpose of the SDR requirements is to verify that layers 1 and 2 correctly process in a sustained manner the received transport blocks corresponding to the maximum data rates indicated by UE capabilities.

Observation #9: SDR requirements were defined for 8Rx UEs.

Proposal #9: Define SDR requirements for 6Rx UEs.

During 8Rx discussion in Rel-18, PDSCH was initially prioritized over PDCCH for initial simulation work. Afterwards, it was agreed not to define new PDCCH requirements since the WID prioritized the scope into higher throughput scenarios where PDCCH is not the bottleneck. However, during 6Rx discussion the focus is on both coverage and throughput, so we agree that a new PDCCH requirement is necessary. However, a different case applies for PBCH.

  - Observation #10: Current WID focuses on both coverage and high throughput conditions.

  - Proposal #10: Define PDCCH requirements for 6Rx UE. A single test case with AL1 is enough to quantify for 6Rx vs 4Rx coverage gain.

  - Observation #11: Defining PBCH requirements, even though the channel is not testable in terms of conformance, would consume time that can be dedicated to other 6Rx topics of interest.

  - Proposal #11: Do not define new demodulation requirements for PBCH, since PBCH is not testable in RAN5.

Finally, we propose to based the CSI requirements discussion again on previous experience with Rel-18 8Rx.

Observation #12: Rel-18 scope for CSI requirements focused on CQI reporting only.

Proposal #12: RAN4 to discuss whether to consider CQI reporting for 6Rx, but do not define PMI requirements and do not define RI requirements.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: Since RAN4 was unable to conclude on simulation assumptions to determine feasibility for handheld UEs, 6 MIMO layers will only be supported in FWA devices.

Proposal #1: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will be only apply for FWA devices, and 2) for up to 4 MIMO layers, performance requirements will be defined and will apply to both FWA and handheld UE devices.

Observation #2: The use of 6 MIMO layers in 6Rx FWA devices is targeted for high throughput scenarios. For 6Rx FWA and Hanheld devices, performance requirements for 6Rx and 4 MIMO layer is sufficient.

Proposal #2: For PDSCH requirements, consider a single test for 6 MIMO layers for FWA devices, and for 4 MIMO layers, consider a single test applicable to both FWA devices and handheld UEs.

  - Observation #3: For the PDSCH test with 2CWs, channel estimation and MIMO detection per CW is the same regardless of the number of CWs in the transmission.

Proposal #3: For PDSCH requirements for 6 MIMO layers, consider same MCS value and same propagation conditions for both codewords (CW).

Observation #4: For testing and performance requirement purposes, there’s no justification to increase the number of transmit antennas beyond 8 when considering 6Rx.

Proposal #4: Consider a transmit/receive antenna configuration of 8Tx/6Rx for Rank 6, and a transmit receive antenna configuration of 4Tx/6Rx for Rank 4.

Observation #5: As opposed to previous feasibility study, MCS Table 2 is not really needed since 64QAM can provide a good balance in terms of operating SNR and throughput.

Proposal #5: Consider up to 64QAM (MCS Table 1) for setting PDSCH performance requirements.

Observation #6: During Rel-18 8Rx performance requirements discussions, TxEVM assumptions were an important consideration. Same for Rel-19 6Rx number of MIMO layers discussion.

Proposal #6: For PDSCH consider a TxEVM value of 3% for 256QAM and 6% for 64QAM.

Observation #7: Type-1 and Type-2 DMRS are typical configurations observed in the field.

  - Proposal #7: For both 4 and 6 MIMO layers requirements consider Type-1 DMRS.

Observation #8: As opposed to handheld devices, FWA devices typically experience no mobility.

Proposal #8: For up to 4 MIMO layers consider the use of 1 additional symbol DMRS to target mobility scenarios. For 6 MIMO layers consider no additional DMRS symbol to specifically target the FWA deployment scenario.

Observation #9: SDR requirements were defined for 8Rx UEs.

Proposal #9: Define SDR requirements for 6Rx UEs.

  - Observation #10: Current WID focuses on both coverage and high throughput conditions.

  - Proposal #10: Define PDCCH requirements for 6Rx UE. A single test case with AL1 is enough to quantify for 6Rx vs 4Rx coverage gain.

  - Observation #11: Defining PBCH requirements, even though the channel is not testable in terms of conformance, would consume time that can be dedicated to other 6Rx topics of interest.

Proposal #11: Do not define new demodulation requirements for PBCH, since PBCH is not testable in RAN5.

Observation #12: Rel-18 scope for CSI requirements focused on CQI reporting only.

Proposal #12: RAN4 to discuss whether to consider CQI reporting for 6Rx, but do not define PMI requirements and do not define RI requirements.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024