3GPP TSG-RAN WG4 Meeting #114bis 	R4-2503526

# Wuhan, China, April 7 – April 11, 2025

Agenda item:	7.3.4

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for NR Channel BW less than 5MHz for FR1 Phase 2

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

3GPP Rel-8 TS 36.101 specified various channel bandwidths for 4G LTE systems including 1.4, 3, 5, 10, 15 and 20 MHz channel sizes. Global regulators and mobile network operators allocated and deployed networks occupying these various channel bandwidths that are still operational today. Allocation of additional spectrum is prohibited in nearly all cases due to existing use of the adjacent spectrum. Most of these networks do not utilize NB-IoT but are fully mobile networks that utilize CAT-4 and higher devices. Certain licensees operate regionally and nationally in the USA, internationally across Europe and in parts of SE Asia and Africa. These networks support the family of 3GPP technologies including GSM and LTE for mission critical applications. These operational networks are now looking to invest in the future of 3GPP by deploying 5G services. This includes the use of NR to support multiple low latency use cases (see 3GPP TR 38.825) resulting from various vertical industry domains, e.g., electrical power distribution grid & rail communication. These uses cases often require less than the latency offered by LTE technology and thus need NR to fully support them. During Rel-18 the feature Dedicated Spectrum of Less than 5MHz was finalized, that considered all these previous scenarios to define its value proposition to the 3GPP ecosystem.

# 2. Discussion

During the past Release 18, more specifically in RAN4#109, it was agreed that for the purpose of demodulation performance requirements and coverage, it is assumed there will be no UEs that exclusively support a maximum a channel bandwidth of 5MHz. Hence, the conclusion of this work item was that all previous PDSCH requirements of past releases can be applied to a UE that supports both Less than 5MHz channel bandwidth and legacy bandwidths of larger size.

Observation 1: For the purposes of Rel-18 demodulation performance requirements, it was assumed there will be no UEs that exclusively support a maximum channel bandwidth of 5 MHz. A UE that supports <5MHz will also support 10MHz or more.

Observation #2: The PDSCH requirements implication is that since in such UE PDSCH is already exercised for larger CBWs, there is no need for additional tests for smaller CBWs — any small difference in performance is already expected due to reduced different transport block size and different number of code blocks.

Hence, we believe that for this Phase 2, nothing has change from what has been discussed in Rel-18.

Proposal #1: Maintain current assumption that no product is expected to launch that supports only <5MHz channels.

Proposal #2: Do not introduce new PDSCH requirements for <5MHz channels.

In order to give some more context, from 38.101-4, section 5.5, we can quote the following: “The requirements and procedure defined in Clause 5.5A.1 apply using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.”. Hence, since the SDR test is defined by the bandwidth that provides the largest data rate, a UE that support higher bandwidths in addition to 3MHz will always use a CBW other than 3MHz to perform this test.

Observation #3: For CA requirements, in Rel-18 we test the "Largest aggregated CA bandwidth combination”. For UEs supporting 10MHz or more, it’s not the case that the largest aggregated CA bandwidth will be obtained by one or two carriers being of the type 'Less than 5MHz”.

Proposal #3: Do not define new SDR requirements for <5MHz channels.

Proposal #4: Do not define new PDSCH CA requirements for <5MHz channels.

# 3. Conclusion

Our observations and proposals are stated below:

Observation 1: For the purposes of Rel-18 demodulation performance requirements, it was assumed there will be no UEs that exclusively support a maximum channel bandwidth of 5 MHz. A UE that supports <5MHz will also support 10MHz or more.

Observation #2: The PDSCH requirements implication is that since in such UE PDSCH is already exercised for larger CBWs, there is no need for additional tests for smaller CBWs — any small difference in performance is already expected due to reduced different transport block size and different number of code blocks.

Proposal #1: Maintain current assumption that no product is expected to launch that supports only <5MHz channels.

Proposal #2: Do not introduce new PDSCH requirements for <5MHz channels.

Observation #3: For CA requirements, in Rel-18 we test the "Largest aggregated CA bandwidth combination”. For UEs supporting 10MHz or more, it’s not the case that the largest aggregated CA bandwidth will be obtained by one or two carriers being of the type 'Less than 5MHz”.

Proposal #3: Do not define new SDR requirements for <5MHz channels.

Proposal #4: Do not define new PDSCH CA requirements for <5MHz channels.

# References

- R4-2321064, “WF on NR_FR1_lessthan_5MHz_BW_demod”, Nokia.