# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503536

# Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.29.4.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114][230] NR_NTN_Ph3_Part2 [2].

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# Topic #1: SSB periodicity larger than 20 ms

## Issue 1-1: Cell Reselection

Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed

In last RAN1 and RAN2 meetings, some agreements for DL coverage have been made as followings:

Agreement in RAN1 #120: For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms. Agreements in RAN2#129: 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further Agreement in RAN2 #128: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) 1. As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) 2. RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells 1. For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states. Agreement in RAN2 #127bis 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

The SSB periodicity extension for DL coverage enhancement can probably impact UE initial access procedure compared with the legacy assumption of 20ms. In the current initial access assumption for cell selection, it was defined in RAN1 that,

In the current NTN network (R17, and R18), by default NTN UE will try the initial access to network assuming SSB periodicity as 20ms, however, if the R19 NTN network changes the SSB periodicity to be 160ms, it will cause the initial access failure for the R17/R18 NTN UEs. Even for the R19 NTN UE, it has no clear idea if the network behavior is following R17/R18 or R19, and the UE behavior is unclear for implementation during the initial access. However, cell reselection requirement is defined based on the DRX cycle and SMTC periodicity(SMTC periodicity is mainly used to decide scaling factor), and it shall not be impacted by using 160ms SSB periodicity. Additionally, there is no additional SMTC introduced for DL coverage enhancement. Thus, we agree that no update is needed for cell reselection requirement for NTN DL coverage enhancement.

Proposal 1: no update is needed for cell reselection requirement for NTN DL coverage enhancement, i.e., for the case of 160ms SSB periodicity.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms. In the following agreements in RAN2 #128, there is UE capability of “UEs supporting DL-CE” for RAN2 discussion. Thus, we can also define the new by-default TSMTC,I for UEs supporting DL-CE.

If the SMTC for the inter-frequency carrier is not provided by network, UEs who supporting DL-CE can blindly search the potential carriers/cells by using 20ms or 160ms SSB periodicity, however, UE may fail the re-establishment by using 20ms SSB periodicity assumption when network is using 160ms SSB periodicity. In our view, since beam hopping or DL coverage enhancement is a very practical deployment for NTN system, it would be safer for UE to assume 160ms SSB periodicity if SMTC is not explicitly configured by network. If network would like UE to speed up the re-establishment, it could choose to configure SMTC to UE explicitly; otherwise, it shall allow UE to take the most conservative way, i.e., assuming 160ms SSB periodicity by default.

Agreement in RAN2 #128:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

In addition, since RAN2 agreed that in in RAN2#129 that “it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer”. The same by-default assumption above shall also applied for intra-frequency as well if network does not configure the SMTC on intra-frequency carrier to UE explicitly. Using 160ms SSB periodicity as by-default assumption won’t cause re-establishment failure if there is a detectable cell on intra-frequency layer.

Proposal 2: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for both intra-frequency and inter-frequency cases, where the SMTC for the target carrier is not provided.

- The above will apply only for UEs capable of coverage enhancement feature.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

Regarding the RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB, we have one note in the requirement to say: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. We understand this note was based on an assumption that initial access SSB periodicity is 20ms and SMTC will be configured in line with the SSB periodicity. The re-establishment delay requirement will be missing for DL coverage enhancement case (SSB periodicity =160ms) if SMTC is not configured and RAN4 clarifies the new by-default SSB periodicity for re-establishment. Serving cell SINR<-8dB means the timing information from serving cell is not reliable for UE to detect target cell, and UE needs to perform blind detection within SSB periodicity. In our view, such kind of blind detection is quite similar as SFTD measurements without measurement gaps and with additional SS-RSRP reporting, i.e., serving cell timing information cannot help UE to locate the SMTC window for target cell detection. In the legacy SFTD measurement without measurement gaps and with additional SS-RSRP reporting, the delay is defined as: 19 SMTC periods.

Thus, we think the RRC Re-establishment SSB Ês/Iot < -8dB and SSB periodicity =160ms, shall be specified as 19*160ms= 3040ms.

Proposal 3: for both intra-frequency and inter-frequency RRC re-establishment of R19 NTN, in condition of “serving cell SSB Ês/Iot < -8dB and unknown NR Cell”:

- if configured SMTC periodicity=160ms or no SMTC is configured on target carrier, both Tidentify_inter_NR,i and Tidentify_intra_NR equals 3040ms,

- if configured SMTC periodicity=20ms, both Tidentify_inter_NR,i and Tidentify_intra_NR equals 800ms.

# Topic #2: Satellite Beam Hopping (e.g. Cell DTX/DRX)

In addition, we identified a new issue about scheduling restriction with DL coverage enhancement. In the current TN FR1 scheduling restriction for L1 or L3 measurement, it has two main kinds:

- TDD based scheduling restriction because UE cannot do UL transmission when UE is operation on DL for neighbor cell measurement.

- Other scheduling restriction applies because UE cannot do e.g., mixed numerology Rx or Tx when UE is performing SSB or CSI-RS measurement.

R17 NTN has new scheduling restriction if UE does’t support parallelMeasurementWithoutRestriction, when UE is performing SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

The above scheduling restriction happens during the SSB measurement occasions of the target cells, e.g., UE is doing SSB measurement but cannot receive the data/control or transmit data/control/RACH to the serving satellite. However, in this special case for DL coverage enhancement with Tx beam hopping, even on other time period outside the configured SSB occasions, NW may be still not possible to schedule or receive any channel/signal to or from UE, as shown in following figure 1. To the example UE in cell 2, outside the cell 2 SSB occasions, before T2 and after T2+40ms, NW hopped Tx beams to other cells (cell 1/3/4), and therefore serving satellite cannot transmit/receive anything to or from UE in cell 2. Therefore, we need to reconsider the scheduling restriction applicability in this scenario of DL coverage enhancement.

Figure 1. cell 2 UE(s) may not transmit or receive signaling from satellite when beam hopped away

If the RO periodicity configuration from serving cell is not greater than the SSB periodicity (less than 160ms in the example figure 2), the scheduling restriction applies to UE RACH and RACH related channels transmission outside the time period of serving Tx beam availability, time period of serving Tx beam availability means, e.g., the time period between the beginning of slot containing available SSB (i.e., T2 in example figure 2) and (original SSB periodicity + T2). Thus, the scheduling restriction for RACH could be: If DL coverage enhancement or satellite beam hopping is enabled, the UE is not expected to transmit PRACH on symbols outside the serving satellite Tx/Rx beam availability durations.

Figure 2. example for RO in SSB periodicity

Proposal 4: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: no update is needed for cell reselection requirement for NTN DL coverage enhancement, i.e., for the case of 160ms SSB periodicity.

Proposal 2: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for both intra-frequency and inter-frequency cases, where the SMTC for the target carrier is not provided.

- The above will apply only for UEs capable of coverage enhancement feature.

Proposal 3: for both intra-frequency and inter-frequency RRC re-establishment of R19 NTN, in condition of “serving cell SSB Ês/Iot < -8dB and unknown NR Cell”:

- if configured SMTC periodicity=160ms or no SMTC is configured on target carrier, both Tidentify_inter_NR,i and Tidentify_intra_NR equals 3040ms,

- if configured SMTC periodicity=20ms, both Tidentify_inter_NR,i and Tidentify_intra_NR equals 800ms.

Proposal 4: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# References

[1] R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#114

[2] R4-2500544, Topic summary for [114][230] NR_NTN_Ph3_Part2, Qualcomm, RAN4#114