# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503546

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.22.3.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114 the RRM requirements for UE initiated beam management was discussed and WF [1] was approved. In this contribution we present our views on RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] the following agreements were made for RRM core requirements with UE initiated beam management:

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting? Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting. Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not precluded Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM Agreement: - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? Agreement - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

Issue 1-1: assumption or applicability of event triggered L1 measurement:

<Way forward >

- Option 1: (vivo)

  - Follow the assumption:

    - the condition which triggers the event is remained fixed during the time UE performs L1 measurements.

    - the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting

    - during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell

    - during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam.

- Option 2: others are not precluded

The assumption or applicability for event triggered L1 measurement needs to be discussed. The proposals from RAN4#114 seems very restrictive for the feature to be useful, as the conditions cannot be guaranteed in real environments.

- Applicability for event triggered L1-RSRP reporting should be discussed.

- Options for applicability from RAN4#114 seem restrictive and hard to guarantee in practice

We propose the following conditions for the reporting delay requirements to be applicable:

- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Issue 1-3: Same or different periodicities of current beam and new beam(s)?

<Way forward >

- Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo)

  - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities.

  - According to RAN1 LS (R1-2410914)

- Option 2: (Huawei)

  - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1

- Option 3: (vivo)

  - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

Based on RAN1 LS, only same periodicity of current beam and new beam are considered. Unless there is further discussion and agreement in RAN1 on considering RS with different periodicities, RAN4 shall only discuss requirements based on same periodicity.

RAN4 only discuss/define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1.

Issue 1-4: Components of Event triggered measurement reporting delay?

<Way forward >

- Option 1: (Apple, vivo, Samsung, E///, MTK)

  - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel

    - FFS on whether other components are needed.

    - FFS the definition of Tfirst UL channel

- Option 2: (CMCC, Xiaomi)

  - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

- Option 3: (Nokia)

  - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

It has been agreed to use the L1-RSRP measurement period for SSB and CSI-RS as the starting point for the measurement period for current beam and new beam. The current L1-RSRP measurement period already considers the required resource sharing factors to account for overlap with MG, SMTC or other resources.

Figure 1: Measurement reporting delay for UEIBM

Components of the measurement reporting delay for UEIBM are –

- Measurement period

- Time to PUCCH

Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH.

The measurement period for UEIBM would be max of measurement period for current beam and new beam – max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }

In FR1 and FR2 when current beam and new beam are not adjacent or overlapping:

TL1-RSRP_currBeam = Mc*N*Pc*TcurrBeam

TL1-RSRP_newBeam = MN*N*PN*TnewBeam

In FR2 when current beam and new beam are adjacent or overlapping:

TL1-RSRP_currBeam = Mc*N*2*Pc*TcurrBeam

TL1-RSRP_newBeam = MN*N*2*PN*TnewBeam

Where M is the number of samples =1 if timeRestrictionForChannelMeasurement is configured, 3 otherwise

N: RX beam sweeping factor in FR2

P: Resource sharing factor for the beam (SSB-intra-cell, SSB-inter-cell, CSI-RS)

TcurrBeam , TnewBeam periodicity of the current and new beams (both are equal)

Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

The time to PUCCH is time to first PUCCH after measurement delay or TPUCCH – periodicity of PUCCH resource for UEIBM.

Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH.

Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH.

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

<Way forward >

- Option 1: (Qualcomm, Apple, Ericsson)

  - Only define requirements for the number of events is met at once

- Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable M

- Option 2a: (ZTE)

  - Event1&Event 2: Case 2: multiple samples M and timer X should be considered

  - Event1&Event 7: FFS

- Option 2b (Samsung)

  - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

- Option 2c (MediaTek)

  - For multiple events is met by counter:

    - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

    - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

    - C: event instance counter is configured by NW.

It is not feasible to define delay requirements based on evaluation window and can only be defined based on no time window configured or M=1.

- It is not feasible to define core requirement for reporting delay with time window based triggering

Measurement reporting delay requirement for UE initiated beam report is defined for M=1.

To extend the requirements to window based evaluation, we can add a note that longer reporting delay is expected if M> 1 is configured.

To extend the requirements for case when M>1 is configured, add note longer delay is expected if M>1 is configured.

Issue 1-7: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

<Way forward >

- Option 1: (Xiaomi)

  - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Option 2: (LGE, Nokia)

  - Need to wait RAN1 progress

- Option 3: (vivo)

  - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

The network should configure resources for measurement and PUCCH accordingly for UEIBR. The UE is only expected to measure until the trigger condition is met at the UE.

- The NW is expected to configure resources for measurement and PUCCH accordingly to ensure timely indication/ request when event is triggered at the UE.

The UE is expected to make measurements until event trigger condition is met at the UE.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, MediaTek(descoped))

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case

- Option 2: (Apple, Huawei, Ericsson)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

  - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP

  - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement.

- Option 3: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 4: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

The CSI_RS resources configured for new beam measurement cannot be identified as from serving cell or cell with different PCI. For event triggered reporting based on CSI-RS, the requirements are defined for CSI-RS without specifying inter-cell or intra-cell.

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

RAN4 defines UEIBR reporting delay for CSI-RS based measurement without specifying intra-cell or inter-cell.

Issue 1-11: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Xiaomi, Nokia)

  - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Option 3: (LGE)

  - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

- Option 4: (ZTE)

  - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc

- Option 5: (Samsung)

After the UE sends event triggered beam report, the network could send a TCI state switch command to the new beam. The existing TCI state switching delay requirements for unified TCI state or legacy DL active TCI state would still be appliable. The TCI state switching delay components are

MAC CE decoding time

L1-RSRP measurement period for unknown TCI state

TO/FO tracking if TCI state is not in active list

UEIBR doesn’t impact any of the delay components.

- UE initiated beam reporting doesn’t impact any of the delay components for TCI state switching delay.

There is no need to specify new unified TCI state switching delay requirements.

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Apple, Ericsson)

  - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 2: (China Telecom)

  - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

- Option 3: (CMCC, MediaTek)

  - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

For RAN1 led WIs and features developed RAN1 would typically define associated capabilities. It would be premature to define any UE capabilities related to supported number of UE-initiated/event-driven beam reporting in RAN4.

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Issue 1-15: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia, Samsung)

  - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately.

- Option 4: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

In parallel WI in R19 for L1 based mobility, UE initiated beam reporting for neighbor cell is being discussed. It is very early in the WI stage to discuss UE capabilities. The RRM requirements for the respective WIs should proceed independently and while UE features are discussed, revisit discussion on compatibility of capabilities with LTM.

Discuss later in the WI on compatibility of capabilities with LTM.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

Reporting Delay Requirements

- Applicability for event triggered L1-RSRP reporting should be discussed.

- Options for applicability from RAN4#114 seem restrictive and hard to guarantee in practice

- Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

RAN4 only discuss/define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1.

Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH.

Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH.

Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH.

- It is not feasible to define core requirement for reporting delay with time window based triggering

Measurement reporting delay requirement for UE initiated beam report is defined for M=1.

To extend the requirements for case when M>1 is configured, add note longer delay is expected if M>1 is configured.

- The NW is expected to configure resources for measurement and PUCCH accordingly to ensure timely indication/ request when event is triggered at the UE.

The UE is expected to make measurements until event trigger condition is met at the UE.

CSI-RS Based measurements

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

From UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

RAN4 defines UEIBR reporting delay for CSI-RS based measurement without specifying intra-cell or inter-cell.

TCI State Switching Requirements

- UE initiated beam reporting doesn’t impact any of the delay components for TCI state switching delay.

There is no need to specify new unified TCI state switching delay requirements.

Capabilities

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Discuss later in the WI on compatibility of capabilities with LTM.

# Reference

- R4-2502595, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.