# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503547

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.22.3.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114 the other RRM requirements impact for MIMO phase 5 was discussed and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

UE Reporting enhancement for CJT calibration

For CJT calibration the following agreements were made in RAN4#114:

Issue 1-1: How to decide on the number of samples used for CJT calibration? Agreement: - RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters - Report resolution (MD = 128, MFO = 128). - Offset range (AD and AFO) - AD = CP - AFO = 0.2 ppm - Time offset between the two TRPs: 0.5 CP - Frequency offset between the two TRPs: 0.1 ppm - Note that report resolution and offset range are network configuration via RRC signalling. - Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

In the past few meetings, for CJT calibration, the issue of deciding the number of samples has been discussed. Simulation parameters have been agreed for further evaluation. The CJT calibration reporting is aperiodic, and the UE should be expected to make a report based on a one-shot measurement based on the aperiodic report trigger. The delay/phase/frequency offset report resolution is very small. It is obvious that more averaging would give us better results in terms or accuracy. But it is hard to understand the impact of error when the resolution is so small.

CJT calibration reporting is aperiodically triggered. UE could be expected to make a one-shot measurement based on the trigger.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

Based on some results presented in RAN4#114, we observe that the improvement in accuracy for delay offset with 3 samples vs 1 sample is about 0.01 CP to 0.04 CP depending on the SNR. If it is looked in terms of number of levels based on a given resolution, it could seem large, but the error is still very small, in 1/100ths of CP. It is hard to understand the overall improvement in the system with such small accuracy improvement.

Based on results from RAN4#114 the accuracy improvement is very small 1/100ths of CP with 1 vs 3 samples.

Impact of such small accuracy improvement on overall system performance is unclear.

Based on the above observations, we propose that no core requirements are defined for CJT calibration with assumption of one-shot measurement reporting.

- Do not define core requirements for CJT calibration reporting enhancement for delay/frequency/phase offset reporting based on multiple samples.

Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/U: mTRP  the following agreements were made in RAN4#114:

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured. Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement.

On the timing requirements for 2TA, RAN4 has been discussing based on scenario 1 and scenario 2 –

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

Scenario 1, 2 only concerns PL-offset configuration and has no implication to DL timing reference. In our understanding the agreement in RAN1#118 – “One downlink reference timing is supported and applied to both TAGs.” is applicable to both scenario 1and scenario 2.

Scenario 1, 2 only concerns PL-offset configuration and not reference timing.

RAN1#118bis agreement on reference timing is applicable to both scenario 1 and scenario 2.

The timing reference is derived based on the DL/Joint TCI state, since the UE is not configured with DL RS for UL only TRP, the reference timing should be same for both TAGs, irrespective of PL-offset configuration.

The DL timing reference is based on DL/Joint TCI state configured for TRP receiving on DL.

Since RAN4 has sent LS to RAN1 seeking clarification, we can wait for RAN1’s reply to discuss how the timing requirements/ reference timing for UL only TRP when PL-offset is not configured is defined.

- Discuss DL timing reference for UL only TRP for scenario 2 after receiving reply from RAN1.

In Rel-18 MIMO evo, for timing requirements with 2 TA, the gradual timing adjustment requirements were applicable to each TAG. In case of asymmetrical DL sTRP/ UL mTRP, at least for scenario 1 there is only 1 DL/UL TRP with corresponding reference timing. Hence, the gradual timing adjustment requirements shall be applicable only to TRP with both UL and DL.

- The gradual timing adjustment requirements are only applicable to TRP with both UL and DL

On the open issues –

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

Further discuss the two proposals and down-select one proposal in the next meeting:

  - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement.

  - Proposal 3 (Xiaomi, Nokia):

    - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP.

    - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

    - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay.

    - Further refine the wording for P3 in the WF, if needed

For UL TCI state switching in R17, the components of delay are MAC CE decoding timing, PL-RS measurement for PL-RS not maintained, L1-RSRP measurement time for unknown TCI state. For SRS based TCI state switch would only be based on MAC-CE decoding time. For UL only TRP, it would be PL-offset to be applied to reference PL from DL TRP. There is no known/unknown condition for SRS beam.

For SRS based TCI state switch for UL only TRP, the delay would only be MAC-CE decoding time.

Also, the feasibility to test SRS based TCI state switch without a DL beam is unclear and were not defined sine R16 UL spatial relation info switch.

Feasibility to test SRS based TCI state switch is unknown.

Based on the above observations, we propose not to introduce any requirements for SRS based UL TCI state switch.

- Do not define TI state switching requirements based on SRS.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

UE Reporting enhancement for CJT calibration

- CJT calibration reporting is aperiodically triggered. UE could be expected to make a one-shot measurement based on the trigger.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

Based on results from RAN4#114 the accuracy improvement is very small 1/100ths of CP with 1 vs 3 samples.

Impact of such small accuracy improvement on overall system performance is unclear.

- Do not define core requirements for CJT calibration reporting enhancement for delay/frequency/phase offset reporting based on multiple samples.

Asymmetric DL sTRP/UL mTRP

Scenario 1, 2 only concerns PL-offset configuration and not reference timing.

RAN1#118bis agreement on reference timing is applicable to both scenario 1 and scenario 2.

The DL timing reference is based on DL/Joint TCI state configured for TRP receiving on DL.

- Discuss DL timing reference for UL only TRP for scenario 2 after receiving reply from RAN1.

- The gradual timing adjustment requirements are only applicable to TRP with both UL and DL

For SRS based TCI state switch for UL only TRP, the delay would only be MAC-CE decoding time.

Feasibility to test SRS based TCI state switch is unknown.

- Do not define TCI state switching requirements based on SRS.

# Reference

- R4-2502596, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.