3GPP TSG-RAN4 Meeting #114bis		R4-2503553

# Wuhan, China 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	7.19.3

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements and testing framework for AI/ML based beam management.

RAN4 made the following agreements during the last few meeting [7][8][9][14]

Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point. Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

## 2.1 Selection of Beam Prediction Metrics

3GPP focused on the following prediction metrics in [3]

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

RAN4 agreed to the following during the last meeting [14]:

Issue 2-1: Metrics/KPIs for beam prediction Agreement If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Last meeting’s agreement shows that RAN4 is considering both option 2 and 3 regarding beam prediction accuracy. Technically, RAN4 can define performance requirements for both option 2 and option 3. However, option 3 provides more flexibility than option 2 for RAN4 to define a metric. Option 2’s drawback lies in the fact that option 2 (top-1 (%) version of option 2) does not distinguish between following two scenarios:

- UE’s top-1 predicted beam is adjacent to, but not same as, the top-1 strongest beam

- UE’s top-1 predicted beam is far from the top-1 strongest beam.

This drawback of option 2 is addressed in option 3. UE can pass option 3 if the maximum RSRP among UE’s top-K predicted beams lies within X dB of the RSRP of the strongest beam. Hence, option 3 allows UE to pass a test if UE’s top-1 predicted beam is adjacent to the top-1 strongest beam.

RAN1 and RAN2 have not yet decided the maximum number of beams that UE can convey in its inferred reporting. The maximum number of measured beams that TE can report is also upper bounded by legacy spec. If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

Observation 1: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE may not be able to pass option 2 focused test.

- Note: The maximum value of K depends on the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Option 3 is a more refined version of option 2. It does not suffer from the constraints of option 2 and it retains the same usefulness of option 2. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam. RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 2: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Proposal 1: RAN4 defines beam prediction accuracy requirement for option 3

## 2.2 TCI State Status of Predicted Tx Beam

RAN4 agreed to the following during a couple of meetings ago.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

We focus on the requirement of UE’s RX beam knowledge during TCI state switch from two perspectives.

- To meet performance requirement of AI-ML based beam prediction

- Legacy spec requirement

RAN4 has agreed to define RSRP accuracy as a metric for AI-ML based beam prediction. For both temporal and spatial beam prediction, UE must know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. If UE does not know its RX beam corresponding to the gNB Tx beam, UE’s predicted L1-RSRP RSRP will be drastically different from the actual L1-RSRP.

Observation 3: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

RAN4 has also agreed to define beam prediction accuracy as a metric for AI-ML based beam prediction. UE has to predict the top beams of setA based on what UE would see at its baseband to pass beam prediction metrics. To compare between gNB beams, UE needs to have some idea of the Rx gain associated with them. Hence, UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 4: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Legacy spec defines CSI-RS L1-RSRP measurement period in the following way:

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where -… - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. - For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set. - For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

Observation: Legacy spec requires UE to know the RX beam of CSI-RS if it is configured without “repetition = ON” and if it is QCLed with SSB.

Figure 1: Spatial Beam Prediction

3GPP is focusing on two different scenarios for spatial beam prediction. They are shown in Figure 1 and mentioned below:

- Scenario 1:Wide-to-narrow beam prediction

- Scenario 2: Narrow-to-narrow beam prediction

Scenario 1 requires UE to predict properties of set A CSI-RS resources based on set B SSB measurement. The setA CSI-RS resources whose properties that UE needs to predict are QCLed with respect to type D with the setB SSBs that UE has measured. Hence, UE should have knowledge of, at least, the wide RX beam that it can use to receive the setB beams.

Scenario 2 requires UE to predict properties of a larger set of CSI-RS resources (setA) based on the measurement of a smaller set of CSI-RS resources (setB). Although both setB and setA have CSI-RS resources, any RAN4 test focusing on scenario 2 will also allow transmission of SSBs. If the CSI-RS resources of setA are not configured with “repetition = ON”, they are expected to be QCLed w.r.t. type D with SSB resources that UE can measure.

Observation 5: In “wide-to-narrow” beam prediction, the setA CSI-RS resources whose properties that UE needs to predict are QCLed wrt type D with the setB SSBs that UE has measured. In order to receive a CSI-RS of setA beams after TCI state switch, UE can use the wide RX beam that it needs to measure the SSB that is QCLed with the CSI-RS.

Observation 6: In “narrow-to-narrow” beam prediction, if SSB resources are configured during the test, UE can use the RX beams corresponding to the SSBs as the RX beam for the setA CSI-RS resources if following conditions are satisfied:

- CSI-RS resources are configured without “repetition = ON” and QCLed wrt type D with SSB that are present during the tests.

Proposal 2: After TCI state switches to a setA beam, spec mandates UE to “know” the RX beam of that setA beam in, at least, the following scenario:

- setA CSI-RS resources are configured without “repetition = ON”, SSB resources are configured during the tests and setA CSI-RS resources are QCLed wrt type D with SSBs

- FFS: UE’s RX beam knowledge in other scenarios.

## 2.3 Simulation Results and Error Modeling

### 2.3.2 Simulation Framework

RAN4 agreed to the following simulation framework during the last meeting [14].

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in TR 38.843 Table 6.3.1-1. - Use subset samples of the ideal dataset generated from the 1st step for training models. - Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference and to evaluate the performance metric without considering measurement errors. Note: UE distribution is 100% outdoor - Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors. - Use the Simulation assumptions for RF error defined in Clause 3 to generate RF errors. - Add the measurement errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be: - Case 1: Only baseband errors are included - Case 2: Only RF errors are included - Case 3: Both baseband errors and RF errors are included - Use the dataset with measurement errors for inference and to evaluate the performance metric with considering measurement errors.

The ultimate goal of this simulation framework is to investigate the impact of measurement error on beam prediction metrics. The consideration of measurement error can appear in three forms.

- Case 1:

  - Training data: No error

  - Testing data: Includes measurement errors

  - Ground truth for training and testing: No error

- Case 2:

  - Training data: Includes measurement error

  - Testing data: Includes measurement error

  - Ground truth for training and testing: No error

- Case 3:

  - Training data: Includes measurement error

  - Testing data: Includes measurement error

  - Ground truth for training and testing: Includes measurement error

RAN4 agreed to focus on case 1 during the last meeting. However, if testing data includes measurement error, UE should train its model based on a dataset that includes measurement error, too. Hence, RAN4 should investigate training with measurement error, too.

In a real FR2 AI-ML BM test, ground truth will be decided based on UE’s report. RAN4 already agreed to this concept in the context of ground truth for predicted RSRP in [14]

- Agreement - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Baseband error will decrease at high SNR. However, RF error may not decrease at high SNR because UE’s Rx AGC settings might be calibrated for a particular SNR value.

Observation 7 Baseband error will reduce at high SNR.

Observation 8: RF error may not decrease at high SNR because UE’s Rx AGC settings might be calibrated for a particular SNR value.

Since ground truth will be based on UE’s report and UE’s reported RSRP will include RF error that may not decrease with SNR, it would make sense to investigate where the ground truth during training and testing include measurement error.

Observation 9: Ground truth will be based on UE’s report and UE’s reported RSRP will include RF error that may not decrease with SNR.

Proposal 3: RAN4 investigates performance where training data includes measurement error.

Proposal 4: RAN4 investigates performance where ground truth during training and testing include measurement error.

### 2.3.2 Baseband Error Modelling

We have generated baseband error results at -3 dB with both AWGN and the TDL-C channel assumptions of [11]. We show the distribution of baseband error results in Figure [3] and [4].

Figure 3: Baseband absolute error distribution of SSB L1-RSRP at AWGN (-3 dB SNR, curve fitted with zero mean normal distribution and sigma = 0.6 dB)

Figure 4: Baseband absolute error distribution of SSB L1-RSRP (TDL-C, -3 dB SNR, curve fitted with normal distribution and mean = -0.12 dB and sigma = 0.85 dB)

Observation 10: At -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-C: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

### 2.3.3 RF Error Modelling

[14] mentioned the following regarding RF error.

The impact of RF errors should be considered. Following assumptions can be used to generate RF errors. - Truncated Gaussian distribution under ±4dB RF error (μ=0, σ=2) is used - Dependence for each TX beam - Option 1: Independent for each TX beam - Option 2: Dependent to the strongest TX beam - RF error of each SSB is dependent with respect to RF error of strongest SSB. - Assume strongest SSB’s RF error is +X dB in one instance. - Other SSBs’ RF error will be conditioned with respect to +X dB. - For example, if RF error were uniform, other SSBs’ RF errors will be uniform in [X-Y, X+Y] dB range where X depends on company.

Legacy RAN4 spec has introduced relative L1-RSRP accuracy tests. This implies that the total measurement error, modelled as the summation of baseband and RF error, of a particular SSB is correlated with that of the strongest SSB. Simulation results from previous section suggest that baseband errors can be reasonably modelled as independent and identically distributed Gaussian random variables. This clearly implies that the RF measurement error of a particular SSB is correlated with that of the strongest SSB. However, it is not straightforward to model the exact dependency of RF measurement error of other SSBs on that of the strongest SSB. Hence, it would be reasonable to focus on option 1, i.e., independent RF error for each TX beam, to generate initial simulation results for alignment.

Observation 11: The existence of relative L1-RSRP accuracy tests suggest that the total measurement error, modelled as the summation of baseband and RF error, of a particular SSB is correlated with that of the strongest SSB. This implies that the RF measurement error of a particular SSB is correlated with that of the strongest SSB.

Observation 12: It is not straightforward to model the exact dependency of RF measurement error of other SSBs on that of the strongest SSB.

Observation 13:  It would be reasonable to use option 1 of RF error modeling, i.e., independent RF error for each TX beam, to generate initial simulation results for alignment among companies.

### 2.3.4 Simulation Results

# Conclusions

Observation 1: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE may not be able to pass option 2 focused test.

- Note: The maximum value of K depends on the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Observation 2: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 3: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

Observation 4: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 5: In “wide-to-narrow” beam prediction, the setA CSI-RS resources whose properties that UE needs to predict are QCLed wrt type D with the setB SSBs that UE has measured. In order to receive a CSI-RS of setA beams after TCI state switch, UE can use the wide RX beam that it needs to measure the SSB that is QCLed with the CSI-RS.

Observation 6: In “narrow-to-narrow” beam prediction, if SSB resources are configured during the test, UE can use the RX beams corresponding to the SSBs as the RX beam for the setA CSI-RS resources if following conditions are satisfied:

- CSI-RS resources are configured without “repetition = ON” and QCLed wrt type D with SSB that are present during the tests.

Observation 7 Baseband error will reduce at high SNR.

Observation 8: RF error may not decrease at high SNR because UE’s Rx AGC settings might be calibrated for a particular SNR value.

Observation 9: Ground truth will be based on UE’s report and UE’s reported RSRP will include RF error that may not decrease with SNR.

Observation 10: At -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-C: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

Observation 11: The existence of relative L1-RSRP accuracy tests suggest that the total measurement error, modelled as the summation of baseband and RF error, of a particular SSB is correlated with that of the strongest SSB. This implies that the RF measurement error of a particular SSB is correlated with that of the strongest SSB.

Observation 12: It is not straightforward to model the exact dependency of RF measurement error of other SSBs on that of the strongest SSB.

Observation 13:  It would be reasonable to use option 1 of RF error modeling, i.e., independent RF error for each TX beam, to generate initial simulation results for alignment among companies.

Proposal 1: RAN4 defines beam prediction accuracy requirement for option 3.

Proposal 2: After TCI state switches to a setA beam, spec mandates UE to “know” the RX beam of that setA beam in, at least, the following scenario:

- setA CSI-RS resources are configured without “repetition = ON”, SSB resources are configured during the test and setA CSI-RS resources are QCLed wrt type D with SSBs

- FFS: UE’s RX beam knowledge in other scenarios.

Proposal 3: RAN4 investigates performance where training data includes measurement error.

Proposal 4: RAN4 investigates performance where ground truth during training and testing include measurement error.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

[14] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114