3GPP TSG-RAN WG4 Meeting #114bis	R4-2503554

Wuhan, China, 7 – 11 April, 2025

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.16.2

Document for:	Discussion

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements (FS_NR_demod_SCM) resulted in the agreed WF [1] at RAN4#114.

This contribution addresses the open issues captured in the last WF [1] and the discussions surrounding it. Specific emphasis is placed on the following topics:

- CDL candidate implementation alignment

  - Down selection between A1/A2/A3 table variants.

  - Choice of AAV options (1Y vs. 3).

  - Aligning the simulation summary delivery.

  - Alignment criteria.

- Comparison setups

  - Performance Comparison of Methodologies vs. Channel Properties Comparison.

In this bis meeting we see it as the most important task to facilitate alignment of the CDL SCM candidate implementations, which is the basis for moving to comparisons that match between contributors in the next non-bis meeting.

In the following we only marginally extend our fundamental observations regarding spatial channel models (e.g., TDL, TDL nonLow, SCM, CDL, AAV) and use cases, as we focus on advancing the study. We instead refer to our earlier contributions, especially R4-2418043, for these fundamental observations:

| Tdoc | Title | Meeting |
| --- | --- | --- |
| R4-2402277 | Discussion on 8Rx general demodulation aspects and spatial channel models | RAN4#110 |
| R4-2411300 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#112 |
| R4-2415282 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#112-bis |
| R4-2418043 | Discussion on Spatial Channel Model for Demodulation Performance Requirements | RAN4#113 |

Common parameters for Spatial Channel Modelling evaluations

Most common parameters that are shared and needed for all evaluations in this study (including alignment, comparison, and all SCM candidates), have been agreed in the previous meetings.	
Only the fixed PMI selection procedure remains as a known fundamental open point, but we also want to preemptively wrap up the discussions about SNR and EPRE in the following.

RAN4 SCM SNR definition

All parts of the SCM SNR definition have been previously agreed as [1]:

#### SNR Definition – ‘long term average’ Agreement: Normalization factor is constant over the duration of the simulation

With prior agreements in [2]

#### SNR Definition with CDL Agreement: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain]. The normalisation factor will be aligned and can be included in TR 38.753. Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

In our understanding there is no further discussion needed on the topic of RAN4 SNR definition.	
The legacy RAN4 SNR definition is directly applicable to requirements and simulations with SCMs. Where the legacy SNR is understood by us as (see TS 38.101-4, sections 4.4 ff):

- Average per RE.

- Average per receiver branch.

- Receiver side and pre-equalizer.

- Measured without precoder on the BB transmit ports (but including AAV).

- Long term average over all REs of a requirement/simulation.

I.e., the receive SNR definition normalizes away any AAV gains (and precoding array gains, just leaving the signal processing gains from SDM diversity).

The legacy RAN4 SNR definition is directly applicable to requirements and simulations with SCM.

CSI-RS EPRE

There was a discussion at RAN4#114, during which the intent behind the CSI-RS EPRE configuration was agreed as [1]:

#### Regarding EPRE Ratio: Agreement: Average EPRE of PDSCH data is equal to the average EPRE of CSI-RS, no power boosting is allowed for CSI-RS with respect to the number of CDM-groups in an OFDM symbol.

This agreement seems sufficient to complete the study, as all CSI-RS configurations shall follow the understanding, independently on different readings of 38.101-4 section Table C.3.1-1.

The CSI-RS EPRE configuration intention has been aligned and agreed. The study can be completed without further discussion on the understanding of the configuration in legacy requirements.

PDSCH fixed PMI selection

In our earlier contributions, a non-random precoder choice was found to be useful to capture the impact of typical MIMO effects (e.g., quality difference between MIMO layers and midterm stable spatial preferences) on the receiver implementation performance (e.g., regularization strategies, spatial distinction via covariance matrix estimation or other means).	
Any non-random precoding strategy combined with a spatial channel model can serve to model these effects, but for requirement and test simplicity it may be desirable to have a procedure to arrive at a comparable fixed precoding choice.

In RAN4#114 a number of options was discussed concerning how a fixed PMI could be chosen [1]:

#### PDSCH PMI Selection Way Forward: Random PMI already agreed. Regarding potential fixed PMI: - Option 1: Random PMI from sub-set - Option 2: Collectively chosen fixed PMI - Option 3: Independently chosen fixed PMI Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

We are a proponent of option 3 (“independently chosen fixed PMI”), as we reason it is the easiest method to align.

With option 2 (“collectively chosen fixed PMI”) the exact implementation of BB port to SCM AEs may cause differences in PMI to spatial transmission characteristics, which becomes another source of error for alignment. E.g., while two SCM implementations may be equivalent from a performance point of view, they may result in different spatial characteristics of the same PMI.	
As such fixing the PMI value collectively will make the alignment more complicated that necessary.

Concerning option 1 (“random PMI from sub-set”), this seems to be a mixture of option 2 and 3. I.e., select a subset of PMIs that are seen in all implementations to be aligned with the spatial properties of each implementation, and then select randomly amongst them so on average the fixed PMI (set) works for all implementations.	
The performance of this approach seems hard to predict and this approach will require additional work to select the sub-set.

Hence, we propose

RAN4 to allow for independently chosen fixed PMI.

However, we also want to propose how to independently chose the fixed PMI across multiple implementations.

It is feasible to make a fixed PMI choice based on the most frequently chosen “PMI_follow” under any given channel model that exhibits midterm stable large-scale parameters. This is achieved, using the most common PMI from a histogram of signaled/chosen follow PMIs (see section 3 of [3]) at reasonably high relative throughput operation points.

Please find hereunder the PMI_follow histogram for CDL 827 (A1 option) + AAV 3, as an example of the approach we used to derive the fixed precoder in our results collection contributions:

827 CDLC UMa – AAV 3 SNR = 22dB TP CW0 = 97% TP CW1 = 92%

Figure 1: 827 CDLC UMa – AAV 3 follow PMI histogram.

With this approach PDSCH performance is comparable between implementation, without disclosing or mandating a specific PMI that might not fit the expectations of the receiver algorithm.

RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations at about [95]% TPUT operating point.

The selection of the SNR operating point for the SNR histogram should not be chosen to high (i.e., for 100% TPUT) as follow PMI selection algorithms may perform erratically, if many PMI achieve 100% TPUT. Similarity, a SNR operating point that is too low, may obscure the fixed PMI choice that match mid term stable large scale channel directions.	
A value of 95% (or 90%) was observed to deliver stable results.

CDL based methodology/model

In this section we try to agree on a common CDL based methodology; typically called 38.753 CDL. A lot of progress has been made in the last meetings, but some aspects are still to be selected among several options.

38.753 channel model parameter tables

Directions of arrival and departure scaling and rotation

In [1] it was discussed how to derive the AS_model and how to use the direction of arrival/departure rotation feature of the TR 38.753 angle scaling formulation:

#### Angles of Departure and Angles of Arrival Way Forward: For all 4 direction combinations (AoA, AoD, azimuth, zenith) the following parameters need to be configured. Angle spread scaling: ASdesired Angle rotation: μϕ,desired as a starting point. For CDL Option A1: Based on 38.827 For CDL Option A2, for each (AoA, AoD) and θ,ϕ, the parameters in the formula are the following: ASmodel is computed according to TR 38.901, Annex A.1, Equation A-1 μϕ,model is computed according to TR 38.901, Annex A.2, Equation A-2 For the purposes of simulation alignment: μϕ,desired = μϕ,model ASdesired ASdesired, ASD; ASdesired, ASA; ASdesired, ZSD; ASdesired, ZSA. CDL Option A1; 25.7620; 74.1138; 4.8978; 18.2050. CDL Option A2; 25.7620; 74.1138; 4.8978; 18.2050. CDL Option A3; 25.7620; 74.1138; 4.8978; 18.2050. As a starting point: cASA,ASD,ZSD,ZSA according to the table below Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 2; 15; 3; 7; 7.

Rotation μϕ,desired

Concerning rotation, we agree to use the alignment assumption of μϕ,desired=μϕ,model for all SU MIMO cases.	
For MU MIMO cases this rotation scaling is one opportunity to create spatially consistent channel realization for more than one UE and the topic should be re-considered, when MU MIMO is discussed.

When considering SU MIMO setup, use mu_desired = mu_model for the direction of arrival/departure rotation feature of 38.753 CDL.

Model angular spread ASmodel

Concerning AS_model, most companies may be ready to align on using calculated AS_model(_calc) for TR 38.753 CDL variants re-derived from 38.901, and to use the values given in TR 38.827 for TR 38.753 CDL variants based on TR 38.827.	
The calculation method is be based on TR 38.901, Annex A.1 and Annex A.2; see also reference formulas further down this section.

For CDL option A1 use AS_model as in 38.827, for options A2 and A3 use AS_model as calculated using TR 38.901, Annex A.1 and CDL link level base tables from section 7.7.1.

Note: Using the AS formulas for TR 38.901 directly on TR 38.901 tables, does not result in exactly the same values for AS_model as given in TR 38.827.	
In TR 38.827 we find the following explication concerning table calculations, which may explain the difference. Our TR 38.901 based approaches do not account for the “deconstructed” midpaths, while TR 38.827 calculations did:

“Each CDL parameter table of contains two sets of three rows, i.e., three clusters, with exactly same angular parameters. […]. For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). […]”

The difference in AS_model derivation in A2/A3 to the values captured in TR 38.827 may be due to the consideration of midpaths in the TR 38.827 parameter tables calculation.

Model angular spread ASdesired

Concerning AS_desired we agree with the WF from the last meeting to use the values in TR 38.827.

Concerning AS_desired use the values in 38.825 (25.7620, 74.1138, 4.8978, 18.2050).

For reference:

Reproduction of calculation of angular spread and mean angle from TR 38.901:

A.2 Calculation of angular spread Based on the circular standard deviation in directional statistics, the following expression for the angular spread AS in radians is proposed (A-1) where is the power for the mth subpath of the nth path and is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

A.2 Calculation of mean angle The power weighted mean angle is given by (A-2) where is the power for the mth subpath of the nth path and is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

Furthermore, the cluster-wise rms azimuth spread of arrival angles (c_ASA, etc.) for the table can then be derived as

cASA,scaled=ASdesiredASmodelcASA,model

And finally, the mean scaled angular per cluster (example for Phi_AoA)

ϕn,AOA,scaled=1Mm=0Mϕn,m,AOA,scaled

Albeit that the last step is not necessary as the αm are symmetric.

Channel model candidate tables

Following the previous section on per cluster angle scaling and rotation, we arrive at a number of TR 38.573 channel model parameters tables, pertaining to the different options of:

- Option A1: with Table 7.2.1-8 in TR 38.827.

- Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation (and without quantization).

- Option A3: based on Table 7.7.1-3 in TR 38.901 with truncation and quantization.	
The truncation method is described in [1] and makes the table dependent on the specific AAV with which it will be used with.

These options follow [1]:

#### TDL and CDL Methodologies Way Forward: Options: CDL from TR 38.753 will include randomness reduction for the purpose of the SI - CDL (TR 38.753) - Option A1 : with Table 7.2.1-8 in TR 38.827 - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis. - […]

In our understanding the three options result in the following UMa CDLC channel model parameter tables.

Note that the table for option A3 is currently a reproduction of the result shared by QC and HW, and still subject to confirmation. But in the meantime, we used it for our evaluations.

Option A1

Table 1: Option A1 - UMa CDLC channel model parameters (equals Table 7.2.1-8 in TR 38.827)

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

Option A2

Table 2: Option A2 - UMa CDLC channel model parameters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -38.222 | -96.938 | 96.764 | 98.141 |
| 2 | 76.6135 | -1.25 | -21.829 | 118.89 | 98.451 | 71.056 |
| 3 | 80.9935 | -3.4684 | -21.829 | 118.89 | 98.451 | 71.056 |
| 4 | 85.0085 | -5.2294 | -21.829 | 118.89 | 98.451 | 71.056 |
| 5 | 79.424 | -2.5215 | -34.158 | -124.42 | 100.86 | 67.561 |
| 6 | 232.359 | 0 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 7 | 235.352 | -2.2185 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 8 | 239.44 | -3.9794 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 9 | 240.316 | -7.4215 | 44.223 | 51.882 | 106.4 | 62.843 |
| 10 | 289.6275 | -7.1215 | -50.55 | 63.395 | 94.476 | 56.552 |
| 11 | 299.7745 | -10.7215 | 49.113 | -42.068 | 107.48 | 69.832 |
| 12 | 340.764 | -11.1215 | -73.004 | 43.065 | 92.308 | 50.785 |
| 13 | 448.4025 | -5.1215 | -44.214 | 65.055 | 104.59 | 103.38 |
| 14 | 477.5295 | -6.8215 | -50.413 | -63.435 | 105.2 | 50.086 |
| 15 | 792.196 | -8.7215 | -60.193 | 78.954 | 91.706 | 51.659 |
| 16 | 989.3325 | -13.2215 | 64.61 | 26.262 | 105.2 | 121.03 |
| 17 | 1554.4985 | -13.9215 | 62.199 | -22.591 | 93.994 | 53.931 |
| 18 | 1679.1095 | -13.9215 | 55.036 | -1.6394 | 91.826 | 61.619 |
| 19 | 2003.923 | -15.8215 | -76.31 | -19.791 | 90.742 | 37.505 |
| 20 | 2046.8105 | -17.1215 | 57.378 | 4.4803 | 108.21 | 53.057 |
| 21 | 2301.8725 | -16.0215 | 58.136 | 0.22765 | 91.706 | 35.757 |
| 22 | 2422.651 | -15.7215 | 67.296 | -4.8548 | 91.585 | 52.882 |
| 23 | 2570.5855 | -21.6215 | 76.181 | -28.296 | 106.4 | 46.417 |
| 24 | 3158.0895 | -22.8215 | -91.394 | 29.27 | 109.53 | 44.669 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3775 | 15.558 | 3.6133 | 12.232 | 7 |  |

Option A3

Table 2: Option A3 – This is table used for our current simulations, but subject to confirmation.
Valid only for AAV 3 and AAV 1Y, which share very similar spatially filtered cluster power characteristics.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 225 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 240 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 250 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 235 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 685 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 690 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 705 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 705 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 850 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 1315 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 12 | 1405 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.4 | 15.6 | 3.6 | 12.2 | 7 |  |

We observe that the cluster-wise spreads in the A3 truncated table, are the same as for the A1 and A2 “24 cluster” model, which is in line with current truncation procedure as described in [1] Annex B.

However, this implies that the final RMS AS of A3 has changed away from the desired value after truncation (as the AS_model_calc changes after truncation) and is no longer be in line with TR 38.901 UMa expectations. Even considering that all clusters have the same AS in all directions, they have different mean angles, which impacts the RMS AS derivation.

When we reduce the cluster set (e.g., by removing weaker clusters), the distribution of angles changes and we are altering the angular spreads. Thus, AS_model changes because it depends on both the variance of the angles and their mean (both included in Phi_n,m).

Hence, we propose

RAN4 to discuss the A3 truncation procedure to clarify, if there needs to be an additional step to recalculate c_ASx after truncation.

Channel model delay resolution/system BW

In the future 5GA, 6G, and beyond, it will be required to extend the system bandwidth, especially for MU-MIMO case this is already the case for 5G. We can expect the required system bandwidth for the requirement and test system for MU-MIMO can be 500MHz for FR1, and 1GHz for FR2. Therefore, it is reasonable to extend the delay resolution of CDL model to 2ns for FR1 (500MHz BW) and to 1ns (1GHz BW) for FR2.

For the CDL model does not support large bandwidth (500MHz/1GHz), it is possible to check channel emulation issues by plotting the frequency correlation function (FCF) of such CDL model. In the example below, Figure 2 shows an incorrect FCF of the channel model which the correlation (Y-axis) bouncing back with very high spikes as frequency distance become larger (X-axis). Figure 3 shows an example of a correct FCF of a channel model supporting large bandwidth. The FCF gradually become smaller and smaller as the frequency distance become larger.

Figure 2: Example of wrong FCF bouncing back at the tails for a channel model does not support large bandwidth (indicative only, not simulation or measurement).

Figure 3: Example of a correct FCF of channel model supporting larger bandwidth (indicative only).

Hence, we propose:

Use a delay resolution of CDL model to 2ns for FR1 and to 1ns for FR2, so that the CDL model can support requirement/system bandwidths of 500MHz BW in FR1 and 1GHz BW in FR2 for both SU-MIMO and MU-MIMO use cases.

We suggestion a modification in the Annex B: Cluster Reduction for CDL Angular and Delay profiles (Informative) in the WF [1] related to Step 8 to extend the delay resolution to 2ns for FR1 and 1ns for FR2.

Modify Step 8 in truncation procedure as follows: Apply the quantization to the delay resolution of 2 ns (for FR1) and [1] ns for FR2. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

Channel model parameters selection

In this section we will make our case for selecting the final TR 38.753 CDL channel model among the options A1/A2/A3, which were re-derived and presented in the prior section.	
To provide a basis for our reasoning we will re-combine our simulation results delivered in our companion tdoc [4] to compare across the channel options (instead of across test cases and features).

PDSCH 8T8R8L

Figure 1: PDSCH 8T8R8L: A1 vs. A2 vs. A3-QC, rand PMI, AAV 1Y (+AAV3), 10/100Hz.

We observe

In 8T8R8L and under AAV 1Y, A1 and A2 perform similar, with A2 having a slight advantage due to higher diversity. A3 scales similarly to A1/A2 for low SNR but saturates at higher SNR due to loss of channel MIMO diversity due to truncation. Differences become more pronounced with increased mobility. AAV3 has less inter-CW gap, i.e., less spatial effects.

PDSCH 4T4R4L

Figure 1: PDSCH 4T4R4L: A1 vs. A2 vs. A3-QC, rand PMI, AAV 1Y (+AAV3), 10/100Hz.

We observe

In 4T4R4L under AAV 1Y, A1 and A2 perform virtually the same. A3 scales similarly to A1/A2 for low SNR but deviates slightly at higher SNR due to loss of channel MIMO diversity due to truncation. Differences become more pronounced with increased mobility. AAV3 performs similarly.

PMI 8T4R8L

Figure 1: PMI 8T4R4L: A1 vs. A2 vs. A3-QC, follow eTypeII PMI, AAV 1Y 8Tx (+32Tx), 10/100Hz.

We observe

In eTypeII follow PMI 8T4R4L with AAV 1Y, we see A1 and A2 performing virtually the same. Using AAV 1Y 32 Tx exhibits the expected performance gain. A3 is scaling similarly to A1/A2 but at lower SNR operating points. Though this behaviour is not systematic, e.g., in 2 layers, A3 has higher SNR than A1/A2.

MU PDSCH 4T4R2+2L

Figure 1: MU PDSCH 4T4R2+2L: A2 vs. A3-QC, random typeI PMI, AAV 1Y, linear IRC vs. eIRC.

We observe

In MU PDSCH 4T4R2+2L under AAV 1Y with linear IRC vs. eIRC, we see that A2 barely exhibits a gap between IRC and eIRC performance, while A3 has a substantial gap. This is due to higher diversity/better channel condition in A2, which “naturally decorrelates/statistically orthogonalizes” the two UEs even with SU linear receivers, which is what we expect from deployment. A3 is lacking some diversity and thus receivers need to implement inter-UE IRC to perform optimally.

Conclusion

From our observations it is clear that

The choice between A1/A2 is does not depend on performance considerations.

Choosing A3 with truncation to 12 clusters causes misalignment with typical MIMO effects in deployment, due to loss in diversity. It could be argued that the link level tables with their 2 groups of 3 clusters with same spatial properties, aggravate this loss of diversity artificially, when combined with the arbitrary choice to retain only 12 clusters.

Thus, we propose

RAN4 to retain only A2 as candidate option for CDL based SCM.

If A3 is to be considered further, the number of retained clusters shall be increased to 16 and cluster-wise AS scaling procedure is to be revisited.

Ray splitting application

In [2] the following agreement was made:

#### Subcluster Correlation Agreement: To avoid inter-cluster correlation, RAN4 companies should make sure they are performing ray-splitting for the sub-clusters as specified by TR 38.827 and TR 38.901 (Table 7.5-5). No further action required.

This ray splitting is to be applied at coefficient generation (step 11 of section 7.5) for the two strongest clusters:

For the two strongest clusters, say n = 1 and 2, rays are spread in delay to three sub-clusters (per cluster), with fixed delay offset. The delays of the sub-clusters are (7.5-26) where is cluster delay spread specified in Table 7.5-6. When intra-cluster delay spread is unspecified (i.e., N/A) the value 3.91 ns is used; it is noted that this value results in the legacy behaviour with 5 and 10 ns sub-cluster delays. Twenty rays of a cluster are mapped to sub-clusters as presented in Table 7.5-5 below. The corresponding offset angles are taken from Table 7.5-3 with mapping of Table 7.5-5. Table 7.5-5: Sub-cluster information for intra cluster delay spread clusters sub-cluster #; mapping to rays; Power; delay offset. 10/20; 0. 6/20; 1.28. 4/20; 2.56.

However, the link level approach for CDL (TR 38.901 section 7.7.1 and TR 38.827) create channel model parameter tables that split the strong clusters into 3 weak separate clusters, and then treat all clusters as weak clusters (i.e., without further sub-paths) in coefficient generation:

Under this approach strong clusters, arguably the predecessors to CDLC UMa clusters 2-4 and 6-8, should use different rays in each sub-cluster (where sub-cluster refers to cluster 2,3,4 for the first set and cluster 6,7,8 for the next set) as specified in Table 7.5-5 of TR 38.901.

However, it is standard practice in RAN1 to run CDL link-level simulation with the method described in TR 38.901 section 7.7.1. Hence, we don’t see a need to deviate from this approach in RAN4 link level simulations, and in particular we don’t want to consider changing the TR 901 CDL channel model basics, as this would require alignment with RAN1.

RAN1 is using weak cluster tables for coefficient generation in CDL link level evaluations, presumably following the guidance of treating all clusters as weak clusters.

The above also entrails that we have a differing understanding from Keysight [5] on the interpretation of the following text from TR 38.827 section 7.2:

Each CDL parameter table of contains two sets of three rows, i.e., three clusters, with exactly same angular parameters. This is harmful for the statistical properties of the models as they become non-WSS across the ensemble of model realizations. Instead of making the angular parameters non-equal by introducing small offsets to angles of the three rows, the problematic clusters are treated as midpaths as intended when the CDLs where drawn from statistical distributions which works across all frequency ranges. For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). This helps keeping the rms DS of the modified CDL to 1s.

While [6], interprets this as needing to re-apply ray-splitting to coefficient generation, we caution that this paragraph is only present in the explication section concerning how the channel model parameter tables were derived for TR 38.827 CDL. 	
In particular the channel coefficient generation part of TR 38.827 section 7.2 explicitly states:

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights defined in Equation 7.2-8.

Or, to summarize in an observation:

When consider TR 38.827 and TR 38.901 descriptions of link level CDL models, one needs to separate the steps of “channel model parameter modelling” and “channel coefficient creation”. In the latter, both reports ask for all clusters to be treated as “weaker clusters”, while in the former mid-paths for the strong clusters are baked into the parameter derivation.

Hence, we have a mismatch of prior agreements that we need to resolve:

Discuss the ray splitting approach for TR 38.753 CDL, with emphasis on strong vs. weak clusters and link level considerations.

Continue using the approach of TR 38.827 and TR 38.901 link level CDL and treat all clusters as weak clusters, i.e., do not apply ray-splitting in TR 38.753 CDL coefficient generation.

Note: It would also be instructive to test the performance impact of implementing ray-splitting vs. weak-cluster on actual performance.

Concerning the impact of treating 2x3 clusters as weak clusters with same angular properties, but not same power and delay, it is our understanding that this introduces some frequency domain non-stationarity due to samples correlated in Doppler domain. This means that the statistics/mean gains per RE change over the frequency domain.	
Since we decide the REs to use (i.e., center frequency and FDRA) in the simulation configuration, all implementations will exhibit the same statistical changes in the frequency domain, and this is independent of the initial phase values (“simulation seed”) selected for simulation runs.	
For MU cases such commonly aligned statistical changes over the frequency domain are a desirable effect, and the same is true for wideband cases in general. This is also in line with typical MMO effects in deployment.

Antenna Array Virtualization selection

From the last meeting, we still have two AAV options on the table [1]:

#### Antenna Array Virtualisation (for CDL options) Way Forward: Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality) Options: - Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only] - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports - Option 3: with AAV with the following configuration: - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Considering that one of the aspects of this study is to follow typical deployments to recreate typical MIMO spatial effects, we are partial to option 1Y, which has been requested by operators (and is modelled after deployment).	
Additionally, for some contributors in RAN4#114, AAV 1Y with 32Tx showed gains for eT2f over T1f in PMI simulations.

For all AAV options, we previously agreed in Maastricht to include the transmitter side “cos pattern” AE array factor. Hence, all AAVs including AAV3/noAAV, include a minimum level of analog beamforming effects. AAV 3 is, thus, not agnostic to implementation.

From our delivered simulation results, both AAV 1Y and AAV3 seem usable from a performance scaling perspective. Alos, both can support two rows of “ports” that enable eTypeII results, which are closer to RAN1 and deployment expectations. However, only 1Y is modelling typical MNO AAVs and is in line with RAN1 assumptions during codebook development evaluations.

Slight preference for RAN4 to choose Option 1Y, including 32Tx case for both alignment and comparison, to align with deployment and RAN1 assumptions. However, Option 3 is feasible.

Follow up on SNR normalization

During RAN4#114 it was discussed whether the CDL normalization method could be defined [1]:

#### SNR Normalisation with CDL Way Forward: Options: - Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis. - Option 2: Define a normalisation method - Option 2a: RAN4 to use following as normalization factor: C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2} Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.. - Option 2b: Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns) Here, amplitude normalization is separated into two factors - 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements - 1GBF : normalize with measured transmit beamforming (power) gain GBF.

As outlined in [7] section 3.2, it is neither possible to derive a normalization factor in closed from, nor is it useful, as possible arbitrary implementation choices in the SCM would render enforcement meaningless, or rather counterproductive.	
In any case, the RAN4 SNR definition at the receiver side will normalize any channel gains, if applied (e.g., measured) correctly.

Do not focus on alignment of any derived or measured normalization factors for CDL channel with different AAV options. Rather leave it up to simulation implementation to keep track of the long-term pre-EQ power measurements and adapt.

Note: In the simplest implementation of long term preEQ signal power measurement, the simulator is run twice, and the signal power/channel gain is measured in the first round, and then either the channel/signal is normalized in the second round, or the noise power is adapted as needed.

For information: We presented our channel gains in the last meeting as follows:

|  | 4x4 CDLC 363ns |  |  |  | 8x8 CDLC 363ns |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AAV | 1X | 1Y | 2 | 3 (noAAV) | 1X | 1Y | 2 | 3 |
| Avg RX branch | 1.13 | 2.31 | 0.50 | 1.50 | 1.56 | 2.04 | 0.52 | 2.55 |
| Each RX branch | 1.23 1.11 1.14 1.04 | 2.45 2.28 2.31 2.20 | 0.54 0.50 0.48 0.46 | 1.62 1.51 1.44 1.41 | 1.62 1.86 1.63 1.33 1.50 1.62 1.55 1.40 | 2.08 2.30 2.08 1.72 2.00 2.15 2.09 1.92 | 0.52 0.58 0.54 0.45 0.51 0.55 0.52 0.47 | 2.54 2.80 2.62 2.28 2.48 2.66 2.60 2.41 |
| Assumptions: - 1/MeNe AAV weight normalization for all AAVs. - Average over the whole simulation duration (10’000 slots). - Average over 100 random starting seeds (all seeds converge to same value eventually) |  |  |  |  |  |  |  |  |

Table 4: Long term average channel model gains (unchanged from last contribution).

However, attempts to align the numbers during the last meeting remained fruitless.

Extensions for MU-MIMO

To fully support MU-MIMO use case and channel scenarios, there are two modifications may be needed based on the current channel model.

The larger bandwidth

since the system bandwidth of MU-MIMO use case may up to a few times of current UE’s bandwidth, therefore, up to 500MHz bandwidth in FR1 and up to 1GHz bandwidth in FR2 the channel model, i.e., CDL models, should be supported.

This has been mentioned in previously to increase the resolution of the delay tab/bin of the CDL model to 2ns in FR 1 and 1 ns in FR2.

CDL models should support large system bandwidth up to 500MHz in FR1 and up to 1GHz in FR2. This is the same as increasing the delay resolution of the CDL model to 2ns in FR1 and 1ns in FR2, which is proposed previously.

Special consistency of the channel environment for more than one UE

In the channel scenario for MU-MIMO case, more than one UE is involved in the same cell or in two neighbor cells for the network. Therefore, the physical channel environment should be considered as special consistent.

It means that since those two UEs that are served as MU-MIMO cases, both UEs should see the same clusters from its own physical locations in the wireless environment. The key aspect in the channel models is that the cluster generated for both the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should be the same clusters. It is OK to has some variation on cluster power, angle, delay between those two CDL models.

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. It is OK to has some variation on cluster power, angle, delay between those two CDL models.

For the equation of generating the channel coefficients of fast fading model in TR 38.901 for MU-MIMO case can consider generating one channel model first, then generate the second channel model based on the modification of the first one by adjust the λ(f):

Similarly, for using the CDL tables, i.e., CDL-A/B/C/D/E in TR 38.901, to generate two CDL models for both UE1 and UE2, the second CDL model should be adjusted based on the first CDL model by power, delay and angles. However, due to lack of the description in the standard, then it is implementation agnostic.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on same clusters. One model can be generated first and the other model can be adjusted based on the first model.

Alignment of spatial channel model implementations

For this (bis) meeting we see it as the most important task to facilitate alignment of the CDL SCM candidate implementations, which will be the basis for moving to comparisons that match between contributors in the next (non-bis) meeting.

Experience from last meeting

Last meeting was the first time a substantial number of simulation results were brought for partially agreed alignment use cases. This allowed to have a first overview, for example, that the 4 Rx use case have clusters of companies that may already be aligned.

To borrow a table submitted to the plenary, but modifying it by removal of results that were not contributed to the simulation summary (i.e., have presumably preliminary character), removal of results from different AAVs, and adding further (case 3) results:

Table 4: Initial 4Rx alignment cases

| 38.753 CDL option A1 |  |  |
| --- | --- | --- |
| maximum TP, 4Tx4Rx 4layer, 30kmh, random precoder |  |  |
| Case 2 100Hz | Case 3, 100Hz (4T4R vs. 8T4R, but similar for random precoding and Rx SNR) |  |
|  | Operating point for 70% |  |
|  | AAV Option 1Y, (8,2,2,8,1) |  |
|  | Case 2 | Case 3 |
| Nokia | 20.0dB | 20.0dB |
| BT | 14.3dB | 17.5dB |
| Ericsson |  | 15dB |
| Samsung | 18.4dB |  |
| MediaTek | 17.8dB | 20dB |

In this table, we see a cluster of companies aligned around 18.5dB.	
In our understanding case 2 (4T4R) and case 3 (8T4R) should perform almost equal given random precoding and RAN4 receives side SNR definition. Indeed, Nokia’s results are almost identical, but not all companies observe this equivalence.

It can also be observed that delivery of whole TPUT curves, instead of just 30/70% TPUT points, is advantageous as it is not straightforward to check for spurious effects that may hinder alignment without full curves.

Delivery of alignment results as full TPUT curves is preferable to identify spurious effects that may obscure potential alignment.

Normalization choice impact

In the last meeting it was agreed that [1]:

#### SNR Definition – ‘long term average’ Agreement: Normalization factor is constant over the duration of the simulation

I.e., a single scalar normalization is to be used over the whole duration/for all REs of the simulation run.	
To illustrate that this is an important point to align implemnetationson, we have run two 4Rx simulations with both (a) full simulation duration normalization and (b) per slot normalization:

Figure 4: CDL normalization, full simulation vs. per slot.

We observe

When using per slot normalization, the 70% TPUT SNR operating point is about 1.5dB better for both 4T4R and 8T4R cases. Alignment is hard without aligned SNR averaging methods.

Alignment criteria and thresholds

In RAN4#114, two further constraints on the SCM implementation alignment criteria were brought forward [1]:

#### SNR Throughput Percentage measurements (for PDSCH) Way forward: At least 70% and 30% for alignment of PDSCH and interested companies can provide full curves if desired. Simulation results collection excel sheet format will be updated accordingly in the next meeting allowing to capture input SNR values for target % of the max throughput for PDSCH alignment. #### PMI for alignment of SCM Way Forward: First check alignment with PMI Random (type I)

We remind ourselves that the “alignment” step of this study is to align the SCM implementations between different contributors. The comparison between the SCM candidates and legacy models is a separate, but not independent, step that we agreed to call “comparison”.

PxSCH simulations

Looking back at Rel-15 (NR_newRAT), it was generally understood to check for alignment at the operating point of the SNR results delivered for requirement derivation.

For UE DMD the (ideal) results were immediately accepted as aligned, if the SNR points were within 2.5dB span and larger spans were treated on a case-by-case basis [8].

For BS DMD, the (ideal) results were declared aligned, if there were at least 3 contributions within 2dB (later often extended to 2.5dB) [9]. Outlier removal was/is automated.

Precoding was usually chosen to be random in PDSCH, and fixed identity in PUSCH requirement delivery.

We would like to propose a similar alignment criteria and thresholds to aim for in this study:

For SCM implementation alignment in PxSCH simulations, aim to have at least 3 contributions be within 2.5dB of SNR at the [70]% TPUT operating point, under random precoding. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

PMI simulations

For PMI results alignment, we could not find as clear-cut agreements in Rel-15 as were available for PxSCH results. However, we assert it being reasonable to use the same approach, under follow PMI, and further discussion on the operating point. Note that follow PMI selection implementation are not specified and bring in another element of uncertainty, hence requiring larger alignment thresholds.

The initially delivered Rel-15 results for PMI were of varying alignment degrees and depending on the number of RX branches presenting spans of up to 3.5dB; 3 contributors were taken as sufficient [10].

For SCM implementation alignment in PMI simulations, aim to have at least 3 contributions be within [3.5]dB of SNR at the [TBD]% TPUT operating point under [type1 and/or eType2] follow precoding in 4Rx conditions. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

We explicitly encourage outlier removal and discourage alignment on gamma values as this value showed a high degree of variability in Rel-15. Thus, we prefer to align on “follow TPUT”.

Note also that in [1] it was agreed to “First check alignment with PMI Random (type I)”, which is counter to our PMI proposal above.

Decision on implementation alignment

A decision can only be taken after collecting simulation results and giving ample opportunity for bug-fixes and sharing information (e.g., via full TPUT curve delivery).

Remaining configurations

In the last meeting all open alignment configurations were closed with exception of fixed precoder cases, that remained FFS. The following agreements were made for alignment in particular, while most agreements were made in the “common” section [1]:

#### Speed/Doppler Way Forward: […] For Alignment: TDL Channels use 10Hz CDL Channels use 3kph #### PMI for alignment of SCM Way Forward: First check alignment with PMI Random (type I)

In our understanding of prior discussions and agreements, the agreed SU simulation/implementation alignment cases are:

SU PDSCH

1) 8Tx8Rx: 8 Layer, MCS 13 on both codewords (Table 1), type I CB, Doppler 10Hz/3kph – PMI Choice (FFS Fixed, Random) – At least 30/70% TPUT

2) 4Tx4Rx: 4 Layer, MCS 13 (Table 1), type I CB, Doppler 10Hz/3kph – PMI Choice (FFS Fixed, Random) – At least 30/70% TPUT

SU PMI

3) 8Tx/[32Tx] 4Rx: 4 Layer (type I), Doppler 10Hz/3kph – Full Throughput Curves (First: PMI Random, later: PMI Follow)

4) 8Tx/[32Tx] 4Rx: 4 Layer (eType II), Doppler 10Hz/3kph – Full Throughput Curves (PMI Follow)

5) 8Tx/[32Tx] 4Rx: 2 Layer (type I), Doppler 10Hz/3kph – Full Throughput Curves (First: PMI Random, later: PMI Follow)

6) 8Tx/[32Tx] 4Rx: 2 Layer (eType II), Doppler 10Hz/3kph – Full Throughput Curves (PMI Follow)

MU PDSCH (SU taking priority over MU)

[7) 2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) , Doppler 10Hz/3kph.]

[8) 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE), Doppler 10Hz/3kph.]

Agree list of SU simulation/implementation alignment cases, as (1) to (6) from above, which are the last meeting’s simulation collection use cases 1 to 6 with 10Hz Doppler only, and PMI random first (follow PMI later) for SU PMI cases.

MU alignment

As discussed in the last meeting, we can come back to MU alignment, when SU alignment has progressed more [1]:

#### Cases for MU-MIMO Agreement: SU-MIMO to take priority over MU-MIMO. Simulation alignment spreadsheet will only include agreed alignment cases.

Comparison of spatial channel model candidate and legacy

Performance wise comparison

The question of metrics for performance comparison of test cases under different channel models has been brought up many times during the past meetings. Until now we did not formalize (nor did we need to formalize) such metrics, as SCM candidates were not stable enough to predict their behaviours.	
Our intent was to check all channel models in MIMO test cases, with respect to whether typical MIMO effects that are either observed in deployment or expected from MIMO systems, are well reflected in the simulation output.

In the last meeting it was also proposed to split “comparison” into “performance comparison” and “channel properties comparison”, which we think is a worthwhile step. The former analyzes the impact on potential demodulation performance figure, while the latter directly compares with channel property measurements and expectations contributed to prior meetings (e.g., [11] and [12]).

From [1]:

#### Performance Comparison Methodologies Way Forward: Companies welcome to bring views at next meeting #### Channel Properties Comparison Methodologies Way Forward: Companies welcome to bring views at next meeting.

In prior contributions (e.g. [7]) we have highlighted several trends that we attribute to expected typical MIMO effects and can be taken as a basis for performance comparison metrics; assuming said trends are replicated by other contributors.

To be more specific, expect to see at least most of the following typical MIMO effects in a demodulation channel model:

- SU PDSCH 2CW

  - SDM receiver performance differences per CW.

- SU PDSCH 1CW

  - Gains of fixed/follow precoder over random precoder.

  - SDM receiver performance (through covariance estimation) impacted by channel evolution rates.

- MU PDSCH

  - Similar performance of linear receivers with or without inter-user IRC.

  - Linear receivers perform worse, but not unreasonably so, than non-linear receivers.

- SU PMI

  - CSI report selection performance and implementation impacted by channel evolution rates.

  - Relative CB PMI selection performance difference impacted by number of layers.

- MU PMI

  - CSI report selection performance and implementation impacted by interference presence (through receiver SDM bias choice).

- All (also non-MIMO)

  - Typical SNR operating points.

Additional expectations for demodulation channel models outside of typical MIMO effects are:

- Performance trends shall be explainable from the channel model construction.

- Alignment with channel measurements, e.g., mid-term stable principal directions and angle diversity (subjective).

- Alignment with performance in deployment (subjective).

- Alignment with RAN1 feature development assumptions (subjective).

- Usefulness of model RAN4, e.g., re-use by AIML work, etc. (subjective).

Hence, we propose:

Performance comparison to be based on observed presence of expected typical MIMO effects (e.g., those listed above), explainability of performance trends, alignment with channel measurement, alignment with trends in deployment, and reuse potential in RAN4.

In general channel model performance comparison cannot be reduced to a simple numerical metric. Such numeric metric comparisons are only feasible, if a ground truth is present (e.g., good measurement results to align with).	
Comparison in our case refers to relatively evaluating, whether the channel model allows for detection and minimum performance bounding of baseband features, where the baseband feature performance should be sensitive to typical MIMO effects. If such effects are present or not should be visible in the performance scaling of the feature under test.

However, without a ground truth, there is a degree of subjectivity in the comparison of channel model candidates for RAN4 demod minimum performance requirements.	
We propose to select a number of use cases/feature configurations, where multiple proponents agree that spatial channel effects in deployment should have an impact on use case performance. Then deliver performance curves and evaluate as along the following examples:

PxSCH focused comparison examples

SU PDSCH 2CW:

Compare the per CW TPUT vs. SNR curves of an 8T/8R/8L PDSCH setup with random and follow precoding, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR scaling would be expected in deployment and how well the channel model candidates match expectations.

SU PDSCH 1CW:

Compare TPUT vs. SNR curves of an 4T4R4L PDSCH setup with random precoding and [3] & [30]kph mobility given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. mobility scaling would be expected in deployment and how well the channel model candidates match expectations.

MU PDSCH:

Compare TPUT vs. SNR curves of an 4T4R2+2L MU PDSCH setup with random (but not overlapping) precoding and linear receiver with and without inter-user IRC and R-ML receiver, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. receiver implementation scaling would be expected in deployment and how well the channel model candidates match expectations.

All PDSCH:

Compare SNR operating point levels of an 4T4R4L PDSCH setup given different channel model candidates by collecting feedback from the group on what operating points would be expected in deployment, as well as are feasible for test chambers, and how well the channel model candidates match expectations.

PMI and other comparison examples

Considering that there is a limit to how many use cases can be discussed at one meeting, we would leave further cases (esp., PMI related) to the next meeting.

Though we already note that using gamma values vs. SNR, for PMI cases, may not be a good choice as gamma was rather unstable and misaligned in Rel-15. Additionally, it was often seen to not reflect expected gains. Follow vs. follow PMI TPUT curves are likely a better choice.

Discuss additions to the performance comparison use cases, e.g., with PMI cases for several CBs and PDSCH extensions, in the next meetings.

Configuration re-use

We maintain our reservations about using gamma values, even though gamma was the only initial discussion in RAN4#114 [1]:

#### Metric for PMI tests comparison Way Forward: Options: - Option 1: Gamma value to random PMI configuration. - Other options are not preconcluded.

The above proposed comparison cases are aligned with the prior agreement of [1]:

#### Speed/Doppler Way Forward: For Comparison: TDL Channels use 10 and 100Hz CDL Channels use 3kph and 30kph

To reduce discussion and workload it is advisable to reuse non-channel related simulation configurations from the alignment cases as much as possible, albeit that they may need adaptation for some of the proposed alignment cases.

Use non-channel related simulation configurations from alignment cases, as baseline for comparison cases and adapt, if necessary.

Channel properties wise comparison

As was more deeply discussed in the prior section, during the last meeting it was proposed to split comparison into “performance comparison” and “channel properties comparison”. The latter can openly compare channel property measurements and expectations contributed to prior meetings (e.g., [11] and [12]) with SCM candidate observations.	
In our understanding channel properties refer mostly to physical properties -as well as- statistical properties of physical aspects, signal processing related quantities (e.g., SINR), and test relevant quantities (e.g., convergence time).

Hence, we would like to compare the channel candidates vs. each other vs. measurements at least with respect to

- DoD and DoA direction, powers, diversity, and stability

  - Can be shown via Bartlett/ESPRIT/etc. analysis.

- Post-EQ SINR histograms.

- Convergence times.

- Optionally, frequency coherence.

As with performance comparisons, there is a subjective element, and comparison is carried out by expressing expectations and subjective alignment with said expectations.

Compare channel properties in terms of at least DoD and DoA direction, powers, diversity, and stability, as well as post-EQ SINR histograms and Convergence times of channel candidates vs. each other vs. measurements.
This is done by collecting feedback from the group on expectations from deployment or measurements, as well as feedback on how well the channel model candidates match expectations.

Legacy channel configuration

First agreements on legacy channel configurations for comparison have been discussed in the last meeting [1]:

#### Comparison with Legacy TDL Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-10/100 with following correlation: - Option 1a: Low - Option 1b: XP Medium - Option 1c: XP High - Option 1d: Medium

We see it as important to especially cover the most commonly used legacy channel models in comparison, such as to illuminate potential blind spots in our current performance requirements. In particular, blind spots that could potentially be avoided with a reliable channel model whose default behaviours cover effects without additional effort in model tailoring/adaptation.

Additionally, the legacy XP and low configurations among given options are less impacted by the questionable ULA antenna correlation formulation (which scales unrealistically with number of antennas and does not consider phase), and both are rarely used in current performance requirements.

Comparison to cover the most commonly used legacy channel models, for example TDLC 300-10/100 with low and medium correlation. Alternatively, TDLA 30-10 med and high correlation.

CDL based SCM configuration

Follow the agreement from alignment and CDL based methodology, i.e., CDLC UMa among choice of option A1/A2/A3, with 10/30kph velocity.

We don’t currently see other CDL specific SCM configurations that we need to consider.

Aim to maintain the CDL based SCM configuration from alignment in comparison.

Conclusion

In this contribution we addressed the open issues captured in the last meeting’s WF and the questions raised in the discussions surrounding it, but specific emphasis is placed on the following CDL candidate implementation alignment (including down selection of variants and ways to improve facility alignment outcomes). Comparison setups and criteria have been discussed as well, but we see the work on alignment as most urgent in this meeting.

In summary, we made the following observations and proposals:

= Common parameters for Spatial Channel Modelling evaluations

== RAN4 SCM SNR definition

- The legacy RAN4 SNR definition is directly applicable to requirements and simulations with SCM.

== CSI-RS EPRE

The CSI-RS EPRE configuration intention has been aligned and agreed. The study can be completed without further discussion on the understanding of the configuration in legacy requirements.

== PDSCH fixed PMI selection

- RAN4 to allow for independently chosen fixed PMI.

- RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations at about [95]% TPUT operating point.

= CDL based methodology/model

== 38.753 channel model parameter tables

=== Directions of arrival and departure scaling and rotation

When considering SU MIMO setup, use mu_desired = mu_model for the direction of arrival/departure rotation feature of 38.753 CDL.

For CDL option A1 use AS_model as in 38.827, for options A2 and A3 use AS_model as calculated using TR 38.901, Annex A.1 and CDL link level base tables from section 7.7.1.

The difference in AS_model derivation in A2/A3 to the values captured in TR 38.827 may be due to the consideration of midpaths in the TR 38.827 parameter tables calculation.

Concerning AS_desired use the values in 38.825 (25.7620, 74.1138, 4.8978, 18.2050).

=== Channel model candidate tables

RAN4 to discuss the A3 truncation procedure to clarify, if there needs to be an additional step to recalculate c_ASx after truncation.

=== Channel model delay resolution/system BW

Use a delay resolution of CDL model to 2ns for FR1 and to 1ns for FR2, so that the CDL model can support requirement/system bandwidths of 500MHz BW in FR1 and 1GHz BW in FR2 for both SU-MIMO and MU-MIMO use cases.

Modify Step 8 in truncation procedure as follows: Apply the quantization to the delay resolution of 2 ns (for FR1) and [1] ns for FR2. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

=== Channel model parameters selection

In 8T8R8L and under AAV 1Y, A1 and A2 perform similar, with A2 having a slight advantage due to higher diversity. A3 scales similarly to A1/A2 for low SNR but saturates at higher SNR due to loss of channel MIMO diversity due to truncation. Differences become more pronounced with increased mobility. AAV3 has less inter-CW gap, i.e., less spatial effects.

In 4T4R4L under AAV 1Y, A1 and A2 perform virtually the same. A3 scales similarly to A1/A2 for low SNR but deviates slightly at higher SNR due to loss of channel MIMO diversity due to truncation. Differences become more pronounced with increased mobility. AAV3 performs similarly.

In eTypeII follow PMI 8T4R4L with AAV 1Y, we see A1 and A2 performing virtually the same. Using AAV 1Y 32 Tx exhibits the expected performance gain. A3 is scaling similarly to A1/A2 but at lower SNR operating points. Though this behaviour is not systematic, e.g., in 2 layers, A3 has higher SNR than A1/A2.

In MU PDSCH 4T4R2+2L under AAV 1Y with linear IRC vs. eIRC, we see that A2 barely exhibits a gap between IRC and eIRC performance, while A3 has a substantial gap. This is due to higher diversity/better channel condition in A2, which “naturally decorrelates/statistically orthogonalizes” the two UEs even with SU linear receivers, which is what we expect from deployment. A3 is lacking some diversity and thus receivers need to implement inter-UE IRC to perform optimally.

The choice between A1/A2 is does not depend on performance considerations.

Choosing A3 with truncation to 12 clusters causes misalignment with typical MIMO effects in deployment, due to loss in diversity. It could be argued that the link level tables with their 2 groups of 3 clusters with same spatial properties, aggravate this loss of diversity artificially, when combined with the arbitrary choice to retain only 12 clusters.

RAN4 to retain only A2 as candidate option for CDL based SCM.

If A3 is to be considered further, the number of retained clusters shall be increased to 16 and cluster-wise AS scaling procedure is to be revisited.

=== Ray splitting application

RAN1 is using weak cluster tables for coefficient generation in CDL link level evaluations, presumably following the guidance of treating all clusters as weak clusters.

When consider TR 38.827 and TR 38.901 descriptions of link level CDL models, one needs to separate the steps of “channel model parameter modelling” and “channel coefficient creation”. In the latter, both reports ask for all clusters to be treated as “weaker clusters”, while in the former mid-paths for the strong clusters are baked into the parameter derivation.

Discuss the ray splitting approach for TR 38.753 CDL, with emphasis on strong vs. weak clusters and link level considerations.

Continue using the approach of TR 38.827 and TR 38.901 link level CDL and treat all clusters as weak clusters, i.e., do not apply ray-splitting in TR 38.753 CDL coefficient generation.

== Antenna Array Virtualization selection

Slight preference for RAN4 to choose Option 1Y, including 32Tx case for both alignment and comparison, to align with deployment and RAN1 assumptions. However, Option 3 is feasible.

== Follow up on SNR normalization

Do not focus on alignment of any derived or measured normalization factors for CDL channel with different AAV options. Rather leave it up to simulation implementation to keep track of the long-term pre-EQ power measurements and adapt.

== Extensions for MU-MIMO

=== The larger bandwidth

CDL models should support large system bandwidth up to 500MHz in FR1 and up to 1GHz in FR2. This is the same as increasing the delay resolution of the CDL model to 2ns in FR1 and 1ns in FR2, which is proposed previously.

=== Special consistency of the channel environment for more than one UE

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. It is OK to has some variation on cluster power, angle, delay between those two CDL models.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on same clusters. One model can be generated first and the other model can be adjusted based on the first model.

= Alignment of spatial channel model implementations

== Experience from last meeting

Delivery of alignment results as full TPUT curves is preferable to identify spurious effects that may obscure potential alignment.

== Normalization choice impact

When using per slot normalization, the 70% TPUT SNR operating point is about 1.5dB better for both 4T4R and 8T4R cases. Alignment is hard without aligned SNR averaging methods.

== Alignment criteria and thresholds

For SCM implementation alignment in PxSCH simulations, aim to have at least 3 contributions be within 2.5dB of SNR at the [70]% TPUT operating point, under random precoding. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

For SCM implementation alignment in PMI simulations, aim to have at least 3 contributions be within [3.5]dB of SNR at the [TBD]% TPUT operating point under [type1 and/or eType2] follow precoding in 4Rx conditions. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

== Remaining configurations

Agree list of SU simulation/implementation alignment cases, as (1) to (6) from above, which are the last meeting’s simulation collection use cases 1 to 6 with 10Hz Doppler only, and PMI random first (follow PMI later) for SU PMI cases.

= Comparison of spatial channel model candidate and legacy

== Performance wise comparison

Performance comparison to be based on observed presence of expected typical MIMO effects (e.g., those listed above), explainability of performance trends, alignment with channel measurement, alignment with trends in deployment, and reuse potential in RAN4.

Compare the per CW TPUT vs. SNR curves of an 8T/8R/8L PDSCH setup with random and follow precoding, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare TPUT vs. SNR curves of an 4T4R4L PDSCH setup with random precoding and [3] & [30]kph mobility given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. mobility scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare TPUT vs. SNR curves of an 4T4R2+2L MU PDSCH setup with random (but not overlapping) precoding and linear receiver with and without inter-user IRC and R-ML receiver, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. receiver implementation scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare SNR operating point levels of an 4T4R4L PDSCH setup given different channel model candidates by collecting feedback from the group on what operating points would be expected in deployment, as well as are feasible for test chambers, and how well the channel model candidates match expectations.

Discuss additions to the performance comparison use cases, e.g., with PMI cases for several CBs and PDSCH extensions, in the next meetings.

Use non-channel related simulation configurations from alignment cases, as baseline for comparison cases and adapt, if necessary.

== Channel properties wise comparison

Compare channel properties in terms of at least DoD and DoA direction, powers, diversity, and stability, as well as post-EQ SINR histograms and Convergence times of channel candidates vs. each other vs. measurements.
This is done by collecting feedback from the group on expectations from deployment or measurements, as well as feedback on how well the channel model candidates match expectations.

== Legacy channel configuration

Comparison to cover the most commonly used legacy channel models, for example TDLC 300-10/100 with low and medium correlation. Alternatively, TDLA 30-10 med and high correlation.

== CDL based SCM configuration

Aim to maintain the CDL based SCM configuration from alignment in comparison.

- References

- R4-2502378, Way Forward for [114][322] NR_SCM, Moderator (Nokia), RAN4#114.

- R4-2419782, Way Forward for [113][320] NR_SCM, Moderator (Nokia), RAN4#113.

- R4-2418043, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#113.

- R4-2503555, Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#114-bis.

- R4-2418042, On Rel-19 Spatial channel model CDL Based Methodologies aspects, Keysight Technologies UK Ltd, RAN4#113.

- R4-2418042, On Rel-19 Spatial channel model CDL Based Methodologies aspects, Keysight Technologies UK Ltd, RAN4#113.

- R4-2500427, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#114.

- R4-1816488, Way forward on NR UE PDSCH demodulation requirements and General aspects, Intel Corporation, RAN4#89.

- R4-1904713, WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, RAN4#90-Bis.

- R4-1903383, Summary of FR1 TDD CSI simulation results, Samsung, RAN4#90-Bis.

- R4-2411557, Scenarios and Requirements for Spatial Channel Modelling, BT plc, RAN4#112.

- R4-2419338, Further observations on the properties of MIMO channels in typical deployments, BT plc, Ericsson, RAN4#113.