3GPP TSG-RAN4 Meeting #114Bis	R4-2503556

Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Procedures for fragmented carriers

Source:	Qualcomm Incorporated

Agenda item:	7.5.2

Release:	Rel-19

WI: 	FS_NR_FR1_DL_Frag_Carrier

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

Ran4 approved a WF [3] on DL Fragmented carriers study SI [1]. In discussion, method for supporting these fragmented carriers in SID has been narrower down to consider some form of RF chain switching. In this paper we discuss the UE behaviour for this feature and what information exchange is needed between the UE and network.

# 2. 	Discussion

## 2.1	Different UE States

WF [3] mentions “Triggering condition to enable ”One RF Rx chain”” as open topic despite the previous WF [2] made agreements on this issue. The agreement in [2] is “Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2”. Despite the lively discussions on this subject, a good common understanding of the UE and network behaviour has not been documented to an agreement level. Without common understanding of the intended behaviour, it is difficult to discussion requirements such as in-gap aggressor levels. Here we first propose to define states of the UE and then in the following we discuss the procedure for changing the state.

In Figure 1 we present the three possible states. There can be two different state 1’s depending on which CC is configured but that is irrelevant for now for the discussion here. For the purposes of understanding the discussion in the next section about the dynamic behaviour reader must understand that while UE is configured to receive a CC, it is able to send CSI reports which contain the impact of any aggressor to channel degradation due to aggressor. We discuss further this aspect in our companion paper [6]. It is also important to understand that while UE is in state 1, network can command UE to measure CC2 via normal measurement procedures and acquire the information of the impact of in-gap aggressor to the CC2 channel quality.

## 2.2	Switching between states

The legacy behaviour: Switching between State 1 and State 2 is what current UE behaviour an be assumed. After UE attaches to the network provided by CC1, network configures UE with the SCell in CC2 and then UE operates in CA for both configured CCs. Network typically does this after it has received information about power level and quality of the neighbouring cell via measurements. In order for UE to perform measurements, the normal operation of the UE is interrupted. As discussed in the previous section and in [6], the channel quality measurement contains the impact of the aggressor to the usability of the CC for this UE. Network would configure UE with this state only if UE has indicated to support such CA configuration.  Switching from state 2 to state 1 would happen in reverse order, after network decided to de-configure the CC2.

New behaviour under discussion in this SID is how to include the State 3 in the procedures and also handling the CC on other bands. The approach has been to enable some sort of measurement in the gap to determine the aggressor levels. This approach has numerous problems as we discuss in [6] and there is an easier way to enable this functionality.

Figure 1. Description of different UE states for fragmented carriers

Instead of requesting the UE to measure the gap, network can ask UE to measure the CSI, RSRP etc for the CC1 in single RX mode (State 3) and two RX mode (State 2) and then determine the actual impact of the possible in-gap aggressor to the usability of the CC for the UE in question in either RX chain mode. In this way the ambiguity created by the different UE implementation is ruled out and the procedure is robust for any kind of aggressor.

Proposal 1: Determination of the UE ability to operate in single RX mode under the in-gap aggressor is made using ordinary channel quality measurements performed on known cell by the UE in1RX mode receiving both CCs and 2Rx mode

The construct of the report should be discussed, whether UE reports state of the channel for both CCs separately in 1 RX mode and then 2Rx mode or does UE perform measurements in both modes and provides the difference in one report.

Observation: Report can be a delta report or separate report from two different modes.

This method is also flexible and provides a solution to the problem if UE only has one RX chain available for this band. UE can measure each CC with 1Rx in separate occasions and in this way construct the equivalent 2Rx measurement information.

The procedures are transparent to the network. Network can instruct the UE to measure with one RX or with two RXs depending on how it wants the UE to keep the RX chains in other bands.

The discussion on the triggering condition is not needed and it is up to the network to decide if channel quality is sufficient for UE to operate in State 3 for the fragmented carriers. The determination if the RX chain is better used in other bands is also simple since the report is of same format than the channel quality reports in other bands.

To solidify the transfer of understanding, we provide a state diagram of this proposal in the Figure 2.

Figure 2. State diagram for switching between the states

This may not be fully conclusive and it is recognised that some state transitions need measurement procedures that may take time. However, also measurement of the aggressors in the gap with 2Rx will need gaps and reconfigurations in the UE. Only process that is simpler is the measurement of the in-gap aggressor when UE is in single RX mode but then the interpretation and use of this measurement result is unclear, or possibly even useless, as we discuss in [6]. Discussion for this procedure should be studied further.

Proposal 2: RAN4 to study efficiently if the above described procedure compared to the previously presented one

# Conclusion

We discussed

Proposal 1: Determination of the UE ability to operate in single RX mode under the in-gap aggressor is made using ordinary channel quality measurements performed on known cell by the UE in1RX mode receiving both CCs and 2Rx mode

Observation: Report can be a delta report or separate report from two different modes.

Proposal 2: RAN4 to study efficiently if the above described procedure compared to the previously presented one

# Reference

[1]	RP-243002, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek Inc., 3GPP TSG RAN Meeting #106, Madrid, Spain, Dec 09-12, 2024

[2]	R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-2502873, “WF on fragmented DL”, MediaTek, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[4]	R4-2502925, “TP for TR38.755”,  MediaTek Inc., Nokia, 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2502949, “Impacts on UE RF requirements and DL performance impacts”, MediaTek Inc. 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2503557, “UE performance in one RX mode”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025