3GPP TSG RAN WG4 Meeting #114-bis		 R4-2503558

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.5.3

Source: 		Qualcomm Incorporated

Title: 	TP for TR38.755: Company C evaluation results

Document for:	Approval

# 1. Introduction

In RAN4#114 a TP [2] with system simulation results was approved for TR [1]. In this paper we provide our view and simulations results for the wanted and aggressor RSRP.

# 2. Discussion

## Proposed TP for TR38.755

<Start of proposed TP>

### 6	Study on power spectral density difference between carriers of co-located adjacent channel operators

[ unchanged text omitted]

Evaluation from company B:

Referring to [14], some system level simulation results which follows coexistence frame work considering large-scale parameters were also provided. The environment and configuration assumptions are as following bullets:

- Co-site case

- Same BW/Power/ANT # of 2 operators at n25 (1.9GHz)

    - 10 MHz(wanted)+10 MHz(unwanted) BW

- Care interference from the same site.

- Log RSRP difference between 2 operators à CDF

The detailed parameters are listed in the table below:

Table 6-1: Simulation parameters of PSD difference from UE reception point of view

| Parameter | setting |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | UMa |
| ISD | 200m |
| Number of operators | 2 |
| Number of sites | 7/operator, each with 3 sectors |
| Number of UEs | 10/sectors |
| Carrier frequency | 1.9GHz |
| Channel BW | 10MHz |
| Antenna pattern | AAS is assumed (same as that in coexistence study) |
| BS antenna configuration | 32 ANT# : (M,N,P) = (4,4,2) 128 ANT# : (M,N,P) = (8,8,2) |
| Note: The simulation follows coexistence framework, which considers large-scale parameters only. |  |

Figure 6-3: Antenna patterns for the simulation

Figure 6-4: Simulated PSD difference at UE receiving antenna

From simulation result, it can be seen there’s no “interference level larger than wanted CC case” if the BW of both wanted CCs and unwanted CC(=Wgap) are same. The maximum ratio of unwanted CC level to wanted CC level is 0dB in this case. Other different BW/Wgap configurations can be scalable estimated that existing requirements at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence. On the other hand, collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases. And even applying the new UE capability that RF parts can support higher order CA configurations with more CCs, the baseband processing capability may need to extend to handle higher order CA combos including more DL carriers as well as more MIMO-layer processing capability.

Evaluation from company C:

This evaluation is based on system level simulation procedure long established in 3GPP, according to the assumptions listed in this section. The result metric provided is the CDF of the difference in RSRP between the serving operator BS and the strongest interfering BS from the other operator. We have varied some large scale deployment parameters (Antenna Panel, Network Shift).

NW shift of 50% and 100% are computed respectively using ISD*sqrt(3)/3/2 and ISD*sqrt(3)/3.

Table 6-2: System Level Simulation Parameters for Company C evaluation

| Parameter | Value | Unity of Measure |
| --- | --- | --- |
| Number of Operators | 2 | - |
| Deployment Scenario | Dense Urban | - |
| Channel Model | UMa (38.901) | - |
| Building Entry Loss Model | P.2109 | - |
| Frequency | 1.9 | GHz |
| Channel Bandwidth | 10 | MHz |
| ISD | 750 | meters |
| Antenna Model | - Sector Antenna, - 32Tx AAS (4,4,2) - 128Tx AAS (8,8,2) | - |
| Active UEs per sector | 1 | - |
| Number of sites/sectors | 19 sites with 3 sectors each | - |
| Network shift across different operators | - Co-site - 50% - 100% | - |
| Sector orientation across different operators | Same | - |

Figure 6-5: Difference between Own BS RSRP and Strongest Interfering BS RSRP [ISD = 750m]

The Figure 6-5 is in agreement with the Figure 6-4 but more broader view is taken.

# 7	Study on RF requirements for the inter-operator co-located BS scenario

It has been agreed baseline UE architecture is one Rx RF chain from single antenna to the end of ADC with a single down-converter. The evaluation assumptions were also agreed as summarized and re-organized bellow[4][5]:

- Example bands and CBW/BCS assumptions:

  - n2/n25, n3, n7, n66, n41, n26

  - Prioritize the frequency bands of ≤2700MHz

  - The example band(s) should already have NC DLCA specified

<End of proposed TP>

Reference:

[1] R4-2504105 draft TR 38.755 FR1 Fragmented carriers_rm_rev1

[2] R4-2502925, “TP for TR38.755”,  MediaTek Inc., Nokia, 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025