3GPP TSG-RAN WG4 Meeting #114bis	 R4-2503566

# Wuhan, China, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | <CR#> | rev | <Rev#> | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TR 38.863 to capture agreement for n252 introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Sband-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Baseline/running CR to capture agreement for Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing agreements for n252 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements on definition of NTN band n252 are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 7.2, 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged section omitted>

# 2	Reference

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-213691: "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)".

[3]	ITU-R Radio Regulations, 2020 Edition

[4]	ECC Decision 06(09): "Designation of the bands 1980-2010 MHz and 2170-2200 MHz for use by systems in the Mobile-Satellite Service including those supplemented by a Complementary Ground Component (CGC)", Approved 01 December 2006, Amended 05 September 2007,

[5]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[6]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN)".

[7]	3GPP RP-152284: "Revised Work Item: Narrowband IoT ".

[8]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[9]	3GPP TR 45.820: "Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)".

[10]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[11]	Report ITU-R M.2292: "Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses".

[12]	3GPP RP-200559: "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz) ".

[13]	ITU-R Annex 4.4 to Document 5D/716-E.

[14]	3GPP TR 38.921: "Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz".

[15]	3GPP TR 36.802: "Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception".

[16]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[17]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[18]	3GPP R4-2111460: "On the Rx Parameters and Rx Testing Setup for NTN gNB".

[19]	3GPP R4-2108099: "WF on [312] NTN_Solutions_Part1".

[20]	FCC Online Table of Frequency Allocations, 47 C.F.R § 2.106, February 1, 2021.

[21]	FCC Order 03-15, Report and Order and Notice of Proposed Rule Making, January 29, 2003.

[22]	FCC Order 20-48, Order and Authorization, April 19, 2020.

[23]	FCC Code of Federal Regulations.

[24]	SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020.

[25]	3GPP TS 38.181: "NR; Satellite Node conformance testing".

[26]	3GPP TS 38.108: "NR; Satellite Node radio transmission and reception".

[27]	3GPP R4-213618: "Moderator’s summary of discussion [94e-41-R17-NRNTN-RAN4Spec] ".

[28]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain".

[29]	Recommendation ITU-R SM.1541-6: "Unwanted emissions in the out-of-band domain".

[30]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain".

[31]	3GPP TR 38.817-02: "General aspects for Base Station (BS) Radio Frequency (RF) for NR".

[32]	3GPP TR 38.817-01: "	General aspects for User Equipment (UE) Radio Frequency (RF) for NR".

[33]	FCC 16-89, REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING

[34]	FCC 20-159, REPORT AND ORDER

[35]	FCC 20-166, SECOND REPORT AND ORDER IN IB DOCKET NO. 17-95 AND REPORT AND ORDER IN IB DOCKET NO. 18-315 AND FURTHER NOTICE OF PROPOSED RULEMAKING

[36]	ECC Decision(05)01, The use of the band 27.5-29.5 GHz by the Fixed Service and uncoordinated Earth stations of the Fixed-Satellite Service (Earth-to-space)

[37]	ECC Decision(13)01,  The harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

[xx]	3GPP TR 36.811, “Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America (Release 10)”, v10.0.0

<Unchanged section omitted>

## 5.2	Regulatory aspects for NTN satellites

### 5.2.1	NTN Satellite band: UL 1980–2010 MHz / DL 2170–2200 MHz

<Change 1>

### 5.2.4	NTN Satellite band: UL 2000–2020 MHz / DL 2180–2200 MHz

Table 5.2.4-1 shows the service allocation for the frequency range in the ITU-R Radio Regulations [3] and Table 5.2.4-2 shows the the FCC frequency allocation table for the frequency range [20].  From Table 5.2.4-1 and Table 5.2.4-2, there is some deviation in how MSS S-band is allocated in region 2 versus in all other regions. Per ITU, the Region 2 MSS S-Band had been allocated to UL 1980-2025 MHZ and DL 2160-2200 MHz. In the case of North America region (US, Canada, Mexico) the MSS band is allocated to UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz which is not exactly aligned to n256 definition, i.e., UL 1980 MHz – 2010 MHz, DL 2170 MHz – 2200 MHz. The 3GPP NTN  S-band n252 (UL 2000 MHz -2020 MHz and DL 2180 MHz -2200) is defined for consistency across North America and Region 2 as a whole.

Table 5.2.4-1: Allocation of 2000-2020 MHz and 2180–2200 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 5.388: The frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-15)* (see also Resolution 223 (Rev.WRC-15)*). 5.389A: The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. 9.11A and to the provisions of Resolution 716 (Rev.WRC-2000)** 5.389B: The use of the frequency band 1 980-1 990 MHz by the mobile-satellite service shall not cause harmful interference to or constrain the development of the fixed and mobile services in Argentina, Brazil, Canada, Chile, Ecuador, the United States, Honduras, Jamaica, Mexico, Paraguay, Peru, Suriname, Trinidad and Tobago, Uruguay and Venezuela. 5.389F: In Algeria, Cape Verde, Egypt, Iran (Islamic Republic of), Mali, Syrian Arab Republic and Tunisia, the use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service shall neither cause harmful interference to the fixed and mobile services, nor hamper the development of those services prior to 1 January 2005, nor shall the former service request protection from the latter services. |  |  |

Table 5.2.4-2: FCC frequency allocation for 2000-2020 MHz and 2180–2200 MHz

The following are the applicable FCC regulatory requirements:

#### 47 CFR § 25.202

According to paragraph (a)(4)(ii), the following frequencies are available for use by the 2 GHz Mobile-Satellite Service:

- 2000-2020 MHz: User-to-Satellite Link;

- 2180-2200 MHz: Satellite-to-User Link.

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

- In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)  In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

#### 47 CFR § 25.216

According to paragraph (e) on limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service, the e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

It should be noted that for the TN Band 23, which is studied and documented in [xx], the FCC regulation part 25.252 on ATC (Ancillary Terrestrial Component) for satellite service was referred to derive the RF requirements. However, since n252 is defined as an NTN band, ATC related requirements are not applicable for this band.

The following figure 5.2.4-1 shows an overview of the NR TN bands adjacent to the NTN n252 band.

Figure 5.2.4-1: Adjacent TN bands to NTN band n252 in North America

As shown in figure 5.2.4-1, the n252 is adjacent to NR bands n25 DL and n2 DL, and is overlap with n70 DL and n66 DL. The n25 DL and n2 DL protections in terms of UE-to-UE coexistence perspective is described in section 7.4.2.4.1.  For UE-to-UE coexistence between n252 and n66, and between n252 and n70 is described in section 7.4.2.4.1.3.

## 5.3	Regulatory aspects for HAPS

<Unchanged section omitted>

<Change 2>

## 7.2	Common issues for satellite access node and NTN UE

### 7.2.1	General

This section aims to present the agreed definitions for the NTN operating bands introduced in Rel-17 and subsequent releases. The possible configurations in terms of channel bandwidth, SCS, channel and synchronization raster for NTN operations are introduced as well.

### 7.2.2	Operating bands

The following bands in Table 7.2.2-1 are exemplary NTN satellite bands agreed in Release 17 (n256, n255) and NTN bands defined in Rel-19 (n252). Regarding the band numbering for NTN satellite bands, it is agreed to start from the largest band number in FR1 range for NTN satellite bands which fully within FR1 frequency ranges to better differentiate NTN satellite band and TN bands and ensure contiguous band number allocation for NTN satellite bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.2-1: NTN satellite bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

### 7.2.3	Channel bandwidth, SCS and spectral utilization

The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[16] and TS 38.101-1[17] can be reused for NTN satellite system.

The supported channel bandwidths per operating band should be defined based on NTN operator input. The details are given in Table 7.2.3-1.

Table 7.2.3-1: Channel bandwidths and SCS per NTN satellite operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| n256 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| n255 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| n252 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

### 7.2.4	Channel raster and sync raster

#### 7.2.4.1	Channel raster

In order to be compatible with current design of sync raster below 3 GHz, the ARFCN parameters below 3000 MHz will be reused and 100 kHz was chosen as the channel raster for NR NTN satellite bands below 3 GHz.

The applicable ARFCNs per operating and for satellite access node are defined in table 7.2.4.1-1.

Table 7.2.4.1-1: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

For NTN bands with 100 kHz channel raster, 10 kHz enhanced channel raster are mandatory supported. See [xx: 38.101-5] for details.

#### 7.2.4.2	Sync raster

The applicable SS raster entries per operating band are defined in Table 7.2.4.2-1

Table 7.2.4.2-1: Applicable SS raster entries per operating band

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |

<Unchanged section omitted>

## 7.4	NTN UE requirements

### 7.4.1	General

In general, NTN UE products are assumed to meet TN UE requirements as specified in TS 38.101-1[17]. The structure and content of the NTN UE specification TS 38.101-5 should be based on TS 38.101-1[17]. The development of TS 38.101-5 should follow the set of conditions that were agreed at the RAN#94e meeting in RP-213618[27].

### 7.4.2	UE Transmission characteristics for satellite access

#### 7.4.2.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Release 17 for satellite access.

#### 7.4.2.2	Conducted transmitter characteristics

<Unchanged section omitted>

#### 7.4.2.3	Radiated transmitter characteristics for Ka band VSAT

<Unchanged section omitted>

<Change 4>

#### 7.4.2.4	Transmitter requirements for NTN Satellite band: UL 2000–2020 MHz / DL 2180–2200 MHz

##### 7.4.2.4.1	Maximum Output Power

For this release, Table 7.4.2.4.1-1 defines the supported maximum output power for band n252.

Table 7.4.2.4.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n252 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

##### 7.4.2.4.1	Additional emission requirements

###### 7.4.2.4.1.1	Additional spurious emission requirements to protect GNSS frequency

As described in section 5.2.4, n252 shall protect the aeronautical radionavigation-satellite service in the frequency range of 1559 MHz and 1610 MHz. The following additional spurious emission requirement in Table 7.4.2.4.1.1-1 is defined. The detail of definition of NS value for this additional spurious emission requirement and the mapping to NS signalling for n252 is defined in [xx: TS 38.101-5]

Table 7.4.2.4.1.1-1: Additional spurious emission requirement to protect GNSS frequency

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

###### 7.4.2.4.1.2	UE-to-UE coexistence spurious emission requirements to protect n2 and n25

To protect n2 and n25, the following two different additional spurious emission requirements are defined, each is associated to an NS value, and a corresponding A-MPR.

  - -30 dBm/MHz (NS_07N)

  - -40 dBm/MHz (NS_06N)

This requirement applies based on coordination between operators and subject to regional/national regulation. From UE perspective, UE applies this requirement based on the received NS value.

The power reduction impact to different bandwidth of n252 is described in 7.4.2.4.2.

See [xx: TS38.101-5] for the detail A-MPR table definition and the normative note for the requirement applicability.

###### 7.4.2.4.1.3	UE-to-UE coexistence spurious emission requirements for n70 and n66

The following spurious emission requirement for UE-to-UE coexistence applies for n70 and n66.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n252 | NR Band n70 | FDL_low | - | FDL_high | NA | NA | 1 |
|  | NR Band n66 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: The co-existence between n252 and band 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

##### 7.4.2.4.2	A-MPR

###### 7.4.2.4.2.1	A-MPR and NS value

Two NS values and their associated A-MPR are defined as described in the following Table 7.4.2.4.2.1-1.

Table 7.4.2.4.2.1-1: NS value and associated A-MPR for n252

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_06N | 7.4.2.4.2.2 | n2521 | 5, 10, 15, 20 |  | 7.4.2.4.2.4 |
| NS_07N | 7.4.2.4.2.3 | n2521 | 5, 10, 15, 20 |  | 7.4.2.4.2.5 |
| NOTE 1: The NS_06N and NS_07N is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

###### 7.4.2.4.2.2	Requirement for NS_06N

Table 7.4.2.4.2.2-1: Additional requirements for "NS_06N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  |

###### 7.4.2.4.2.3	Requirement for NS_07N

Table 7.4.2.4.2.2-1: Additional requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz |  |

###### 7.4.2.4.2.4	A-MPR for NS_06N

To Be Completed

###### 7.4.2.4.2.5	A-MPR for NS_07N

To Be Completed

##### 7.4.2.4.3	Tx-Rx frequency separation

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n252 | 165 – 195 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

##### 7.4.2.4.4	Asymmetric channel bandwidth configurations

All the asymmetric channel bandwidth configurations in Table 7.4.2.4.4-1 are supported with BSC 0.

Table 7.4.2.4.4-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |

### 7.4.3	UE Receiver characteristics for satellite access

#### 7.4.3.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

#### 7.4.3.2	Conducted receiver characteristics

<Unchanged sections omitted>

#### 7.4.3.3	Radiated receiver characteristics for Ka band VSAT

<Unchanged sections omitted>

<Change 5>

#### 7.4.3.4	Receiver requirements for NTN Satellite band: UL 2000–2020 MHz / DL 2180–2200 MHz

##### 7.4.3.4.1	Reference Sensitivity

To Be Completed

##### 7.4.3.4.2	Out-of-band blocking

To Be Completed

### 7.4.4	UE antenna characteristics for satellite access