# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503570

# Wuhan, China, 7 – 11 April, 2025

Agenda Item:	7.25.2.2

Source: 	Apple

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

RAN4 requirements for adaptation of common signal/channel transmission was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are still some open issues as captured in [1].

In this contribution we provide further discussion on the open issues. After discussion, corresponding proposals are provided.

# Discussion

### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – general

Agreement:

Deactivated SCell measurement:

- Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements.

  - At least for the case when the SSB periodicity is adapted to be shorter than before

- Option 2: Same as legacy

- Other options are not precluded.

The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

We continue supporting option 2. Scenario here is similar with legacy. Regardless of SMTC periodicity, UE always follow measCycleSCell for RRM measurement on deactivated SCC:

Observation 1: neither SMTC periodicity nor SSB periodicity appears in requirement of RRM measurement on deactivated SCC in legacy.

Observation 2: regardless of actual SSB periodicity, UE always follow measCycleSCell for RRM measurement on deactivated SCC from measurement period point of view.

Besides, RAN2 approved an LS to RAN4 in Feb. meeting [2]:

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Our interpretation of the first bullet is that after SSB adaptation, UE will follow additional SMTC configuration for RRM measurement on SCell. The additional SMTC configuration tells UE where to measure the SCell. If the SCell is a deactivated SCell, UE would still follow measCycleSCell for RRM measurement, thereby existing requirement still apply. If the SCell is an active SCell, we only need to clarify that SMTC is from the additional SMTC configuration.

Proposal 1: existing requirements of RRM measurement on deactivated SCC also for SSB periodicity adaptation.

Proposal 2: existing requirements of RRM measurement on active serving cell also apply for SSB periodicity adaptation. Only the SMTC may need to be clarify to accommodate the additional SMTC configuration.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for adaptation of common signal/channel transmission. After discussion, the following conclusions are provided:

Observation 1: neither SMTC periodicity nor SSB periodicity appears in requirement of RRM measurement on deactivated SCC in legacy.

Observation 2: regardless of actual SSB periodicity, UE always follow measCycleSCell for RRM measurement on deactivated SCC from measurement period point of view.

Proposal 1: existing requirements of RRM measurement on deactivated SCC also for SSB periodicity adaptation.

Proposal 2: existing requirements of RRM measurement on active serving cell also apply for SSB periodicity adaptation. Only the SMTC may need to be clarify to accommodate the additional SMTC configuration.

# Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- R2-2501484, Reply LS on SSB adaptation, Apple