# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503572

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	7.27.2.1

Source: 	Apple

Title:	Discussion on event triggered L1 measurement reporting

Document for:	Discussion

# Introduction

L1 event triggered measurement reporting has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

We understand the concern and motivation from vivo regarding the above side conditions from UE side. However, we don’t think we should simply add those conditions since it may result in too much constraint to network implementation. For instance, if TCI switch happens during L1 measurement period, i.e. UE needs to compare the target beam with a different serving beam, UE just needs to perform another round of measurement on the new beam as minimum requirements. Therefore,  it is unnecessary to say that “No indicate TCI change during the evaluation period”. Regarding configuration change, it is similar as legacy to some extent. Configuration can only be changed via RRC. After RRC reconfiguration UE needs to follow the updated configuration to perform measurement, including both L1 and L3.

Proposal 1: seems no need to capture assumptions listed under issue 2-1 into core requirements.

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement:

- Except for periodic MR:

  - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

  - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

  - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

The fundamental issue here is to make sure that both serving cell and neighbor cell are considered in events wherein UE needs to compare measurement results of the two cells, like Event LTM3. That technically makes sense. The event can still be triggered if UE realizes target cell RSRP doesn’t not change but serving cell RSRP somehow drops. Although we didn’t use this approach in legacy L3 measurement, we are open to optimize the similar procedure in LTM. Considering that we usually define minimum requirement in RRM spec, we propose to use option 1:

Proposal 2: Event triggered reporting delay for L1 reporting:

- Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

#### Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///)

- Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

- Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia)

- Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK)

- Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

#### Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options:

- Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC)

- Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

#### Issue 2-6: how to define requirements for reporting criteria per measurement category

Candidate options:

- Proposal 1: Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM. (vivo)

We are fine to define a new UE capability for Event Triggering and Reporting Criteria for LTM. However, it is a bit unclear at current stage that if this is really necessary if RAN4 eventually need to define new capabilities regarding support of CSI-RS for L1-RSRP event triggered reporting.

Proposal 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM.

#### Issue 2-7: whether and how to coordinate with R19 MIMO

Candidate options:

- Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC)

Procedure wise, we don’t think we need to discuss this in this work item. Although they are similar in some extent, they are still separate features. New events introduced by RAN1/2 for these two work items are also different. whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage.

Proposal 4: whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage.

One another issue we would like to discuss is about the case that after UE realizes event is met (T0) and before report is actually sent (T1), UE has chance to update measurement result, shall UE report the result that triggers report or report the latest result (after T0 and before T1). In legacy L3, based on RAN2 design UE needs to include serving cell measurement results into report once measurement of neighbor cells triggers event report. Therefore, even if UE has chance to make another measurement and the measurement turns out to be below entering condition, UE can still send the report with the latest result although it may fall below entering condition. It won’t cause any confusion to network. However, for LTM, so far there is no such design in RAN2. We prefer to leave this to RAN2. If companies really want to address this in RAN4, we can leave it to UE implementation.

Proposal 5: after event is triggered and before the report is actually sent, if UE has chance to make another measurement on target cell, it is up to UE whether to report the result that triggers the report or the latest result.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: seems no need to capture assumptions listed under issue 2-1 into core requirements.

Proposal 2: Event triggered reporting delay for L1 reporting:

- Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

Proposal 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM.

Proposal 4: whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage.

Proposal 5: after event is triggered and before the report is actually sent, if UE has chance to make another measurement on target cell, it is up to UE whether to report the result that triggers the report or the latest result.

# Reference

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple