3GPP TSG-RAN WG4 Meeting # 114bis	R4- 2503578

# Wuhan, Hubei, China, 07th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM impacts of UE-to-UE CLI handling

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the some remain open issues as below on RRM impacts of UE-to-UE CLI handle described in WF [1] and more detail in last meeting’s moderator summary [2].

- Issue 1-1-2: Rx beam

- Issue 1-1-9: Requirement applicability

- Issue 1-1-10: Spec structure

- Issue 1-2-1: Side conditions

- Issue 1-3-1: Measurement timing for Method#3

Discussion

Sub-topic 1-1: General issues

Issue 1-1-2 Rx beam - Proposal 1a (LGE): - RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered. - Proposal 1b (ZTE, CATT): - directly refer to RAN1 spec instead of repeating all details in RAN spec.

Regarding the Rx beam configuration/determination for L1 based UE-to-UE CLI measurement, RAN4 need to include full sentences which agreed in RAN1. Between the two proposals above, directly referring to RAN1 spec looks like more simple and easier to manage when the contents are updated. So, we also support proposal 1b.

Proposal 1: directly referring to RAN1 spec looks like more simple and easier to manage when the contents are updated. So, we also support proposal 1b.

Issue 1-1-9: Requirement applicability - Proposal 1 (LGE): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within DL sub-band - Proposal 2 (Nokia, CATT, HW, MTK, QC): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP - Proposal 3 (ZTE): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED: - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP. - For CLI-RSSI resource - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP. - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration. - No gap is needed for UE-to-UE CLI measurement

RAN1 agreed to support the Method#3 which is CLI-RSSI measurement resource is configured within UL sub-band. So, the proposal 1 should be updated as below:

  - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

    - SRS-RSRP measurement resource is fully confined within BW of UL sub-band

    - CLI-RSSI measurement resource is configured within active BWP

We are also fine to reuse the legacy applicability like proposal 2.

Proposal 2: We support the updated proposal 1 as below or reusing the legacy applicability like proposal 2:

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within active BWP

Issue 1-1-10: Spec structure - Proposal 1 (LGE): - RAN4 need to discuss the spec. structure for CLI measurement of the SBFD. - Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements) - Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

RAN4 needs to consider the spec. structure. Currently the CLI measurement spec. structure is as below:

9.7 Cross Link Interference measurements 9.7.1 Introduction 9.7.2 SRS-RSRP measurements 9.7.3 CLI-RSSI measurements 9.7.4 Scheduling availability of UE during CLI measurements 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements <- can be one candidate option.

The measurement items L1 CLI-RSSI/CLI-SRS-RSRP are introduced in Rel-19. So, we think new clause under 9.7 Cross Link Interference measurements can be considered. For example, 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements. In this case the contents of 9.7.1 should be updated to include the L1 CLI-RSSI/CLI-SRS-RSRP measurements. Or new clause 9.7X also can be considered for CLI measurement for SBFD. RAN4 need to start discussion of the spec. structure for CLI measurement of the SBFD.

Proposal 3: RAN4 need to discuss the spec. structure for CLI measurement of the SBFD.

- Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements)

- Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-1: Side conditions - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition.

In the last RAN4#114 meeting, RAN4 decided to further discuss timing offset side condition for accuracy requirements in the performance part, including whether to consider inter-cell scenarios also for defining the side condition based on the reply LS [4] from RAN1 as below:

1 Overall description When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Related to the CLI information exchange among gNBs, RAN3 sent LS [3] to RAN2 in RAN3#127 meeting as below:

1 Overall description RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1. RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, and also between gNB-DU and gNB-CU in case of split architecture. RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration information, which should be based on parameters provided by RAN1. With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1.

From the above LS, RAN3 already started to work on the SBFD information exchange among gNBs for CLI mitigation, and requested to RAN2 to work on the IE design of SBFD time and frequency configuration information. So, RAN4 also need to review whether the inter-cell scenario should be discussed at core part stage.

Proposal 4: RAN4 also need to review again whether the inter-cell scenario should be discussed at core part stage.

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-1: Measurement timing for Method #3 - Proposal 1 (ZTE, HW): - Same as that for L1-SRS-RSRP measurement within UL SB - Proposal 2 (E///): - Base on DL reference timing, same as that for L1-CLI-RSSI measurement within DL SB

According to the agreed measurement methods from RAN1, the additional applicability for L1 based UE-to-UE CLI measurement is necessary.

The measurement timing for Method#3 can be based on DL reference timing which is same as the L1-CLI-RSSI measurement within DL SB. Because the L1-CLI-RSSI measurement within UL SB is not necessary to align with UL timing like L1-SRS-RSRP measurement. In case of L1-SRS-RSRP measurement, the timing error can impact the measurement accuracy performance. But L1-CLI-RSSI measurement is not impacted by timing error and the interesting period of the L1-CLI-RSSI within UL SB measurement also rather DL timing area. So, we support proposal 2.

Proposal 5: We support proposal 2. Base on DL reference timing, same as that for L1-CLI-RSSI measurement within DL SB

Summary

In this contribution, we provide our views on the remain open issues on RRM impacts of UE-to-UE CLI handle described in WF [1] and more detail in last meeting’s moderator summary [2]. Our proposals are as below.

Proposal 1: directly referring to RAN1 spec looks like more simple and easier to manage when the contents are updated. So, we also support proposal 1b.

Proposal 2: We support the updated proposal 1 as below or reusing the legacy applicability like proposal 2:

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within active BWP

Proposal 3: RAN4 need to discuss the spec. structure for CLI measurement of the SBFD.

- Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements)

- Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

Proposal 4: RAN4 also need to review again whether the inter-cell scenario should be discussed at core part stage.

Proposal 5: We support proposal 2. Base on DL reference timing, same as that for L1-CLI-RSSI measurement within DL SB

Reference

[1] R4-2502597, “WF on RRM requirements for NR_duplex_evo”, Huawei

[2] R4-2500536, “Topic summary for [114][222] NR_duplex_evo”, Moderator (Huawei)

[3] R3-250888, “SBFD information exchange among gNBs for CLI mitigation”, Source RAN3, To RAN2, CC RAN1

[4] R1-2410890, “Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation”, Source RAN1, To RAN4