3GPP TSG-RAN WG4 Meeting # 114bis	R4- 2503582

# Wuhan, Hubei, China, 07th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for RRM measurement prediction

Agenda item:	7.21.2.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility as below based on last RAN4 meeting’s moderator summary [2] and WF [1].

- Issue 2-1-1a Relative RSRP Prediction Accuracy

- Issue 2-1-2 Ground truth definition for RSRP accuracy

Discussion

- Topic #2: RAN4 requirements

Sub-topic 2-1: General and Scope

### Issue 2-1-1a: Relative RSRP Prediction Accuracy

During last RAN4#114 meeting the relative RSRP prediction accuracy issue was discussed and made an agreement as below.

Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

According to the agreed definition, the relative RSRP is between beam1 and beam2 or between cell1 and cell2. So, if the relative RSRP is between cell1 and cell2, then the beams from each cell are can be different also.

Proposal 1: Beam 1 and Beam 2 can be from same cell or different cells.

The ground truth of RSRP can get by reporting from UE in FR2. But, in FR1 case, we need to consider the test environment also. If the test environment is conducted test and AWGN channel for FR1, then the ideal RSRP can be used for predicted relative RSRP accuracy. Beacause ideal RSRP is more accurate than ground truth RSRP, so for the more reliable evaluation of the predicted relative RSRP accuracy ideal RSRP should be used in FR1 when the conducted test and AWGN channel environment. If the FR1 test environment is OTA or fading channel, then FR1 also can get the ground truth of RSRP by reporting from UE like FR2.

Proposal 2: For FR2 the ground truth can be get by reporting. And for FR1, it depends on test environment. If the test environment is conducted test and AWGN channel, then use the ideal value. Else the ground truth can be get by reporting.

The measured RSRP accuracy can be different from predicted RSRP accuracy. So, the two metrics for relative RSRP accuracy should be in the same group. So, for the predicted relative RSRP accuracy, the reported RSRP of cell2/beam2 also should be predicted samples.

Proposal 3: The reported RSRP of cell2/beam2 should be predicted samples.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy

Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

If the conducted test is considered with AWGN channel in FR1, then our preference is Alt 1. But if OTA test is considered or fading channel is considered, then our preference is Alt 2.

When Alt2 is adopted, it should be considered how to avoid the UE cheat in the test. One of the methods to avoid the UE cheat in the test is as follows.

If TE has an environment set for the test in advance, and before proceeding with the test against UE, it receives measurement reporting from UE on all the elements in the environment set. Then the TE can know the correct answers for each environment in the set, and then randomly selects the environment in the set to perform the test. UE will not be able to cheating because it does not know which environment has been selected.

Proposal 4: If the conducted test is considered with AWGN channel in FR1, then our preference is Alt 1. But if OTA test is considered or fading channel is considered, then our preference is Alt 2.

Proposal 5: When Alt2 is adopted, it should be considered how to avoid the UE cheat in the test as follow.

- If TE has an environment set for the test in advance, and before proceeding with the test against UE, it receives measurement reporting from UE on all the elements in the environment set. Then the TE can know the correct answers for each environment in the set, and then randomly selects the environment in the set to perform the test. UE will not be able to cheating because it does not know which environment has been selected.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: Beam 1 and Beam 2 can be from same cell or different cells.

Proposal 2: For FR2 the ground truth can be get by reporting, and for FR1, it depends on test environment. If the test environment is conducted test and AWGN channel, then use the ideal value. Else the ground truth can be get by reporting.

Proposal 3: The reported RSRP of cell2/beam2 should be predicted samples.

Proposal 4: If the conducted test is considered with AWGN channel in FR1, then our preference is Alt 1. But if OTA test is considered or fading channel is considered, then our preference is Alt 2.

Proposal 5: When Alt2 is adopted, it should be considered how to avoid the UE cheat in the test as follow.

- If TE has an environment set for the test in advance, and before proceeding with the test against UE, it receives measurement reporting from UE on all the elements in the environment set. Then the TE can know the correct answers for each environment in the set, and then randomly selects the environment in the set to perform the test. UE will not be able to cheating because it does not know which environment has been selected.

Reference

[1] R4-2502594, “WF on AI/ML Mobility,” Moderater (Nokia)

[2] R4-2500533, “Topic summary for [114][219] FS_NR_AIML_Mob,” Moderator (Nokia)