3GPP TSG-RAN WG4 Meeting #114bis	 					     							R4-2503585

Wuhan, China, 7-11 April 2025

Agenda item:			7.27.2.2

Source:	Apple

Title:	Further discussion on CSI-RS based L1-RSRP measurement

Document for:	Discussion

# Introduction

During the past meetings, there have been extensive discussions on CSI-RS based L1 measurement for NR mobility enhancements WI [1-3]. WF in [3] captured all the conclusions and open issues for CSI-RS based L1 measurement in RAN4#113 meeting.

This contribution will give further analysis on the open issues and present our proposals.

# Discussion

## Measurable or detectable

Issue 4-1-1: Whether to use detectable or measurable?

Tentative agreement:

  - Use measurable and specify the side condition for RS being measurable.

The following is concluded from [3]

As discussed in our paper [x], the word “detectable” seems used for SSB because there are clear identification requirements for SSB, and it is possible to perform cell search and detect SSB without prior knowledge. With this understanding detectable seems to be used for unknow cell identification. While measurable is used for cell signal measurement in connected mode or after a cell is detected. Also, after further check the requirement applicability for CSI-RS based L3 measurement, it also seems “measurable” is more suitable here to follow similar logic as L3 CSI-RS based measurement. E.g. The CSI-RS from the target cell configured for L1 measurement should remain measurable.

Proposal 1: confirm the tentative agreement “Use measurable and specify the side condition for RS being measurable”.

## Side condition for CSI-RS based L1 measurement

The following remains open for further discussion from [3]

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

Candidate options:

  - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia)

  - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

#### Proposal 2: it is proposed to use detectable and legacy side condition of SINR=-3dB.

## Measurement procedure

The following is concluded from RAN4#113.

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

After discussion in RAN4#114bis meeting, the below options are captured in [Y]

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

- Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO)

- Option B: Always keep step 2 for FR2-1 (MTK, HW)

- Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

There has been extensive discussion on whether to keep SSB based L1 measurement (step 2) as the condition for CSI-RS based L1 measurement in FR2. The UE needs to perform Rx beam sweeping on CSI-RS resource for neighbour cells without prior SSB based L1 measurement as the basis, which will make the measurement delay for neighbour cell quite long (N times scaling). In this regard, we think it make sense to keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. On the other hand, there is a interest and support to make this step as optional to allow more capable UE to skip this step and save the measurement time, e.g. a UE with Channel based beam training for L1 measurement may not need step 2. In this case, specification support shall be considered to optimize the measurement procedure for such UE with capability reporting. Hence, we think the aforementioned option A should be confirmed. Also the capability interpretation in [4] should be considered. E.g.

- For a UE not supporting the capability,

  - Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are not in the CSI-RS resource set with ‘repetition = on.’

- For a UE supporting the capability,

  - Step 2 can be skipped, and if the Step 2 is skipped:

    - The CSI-RS resources in Step 3 should be QCL’ed with the SSB in Step 1.

    - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

    - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period and the beam sweeping factor can be based on the existing UE capability maxNumberRxBeam or a new additional UE capability.

Proposal 3: Whether step 2 can be skipped or not can be based on UE capability reporting.

  - For a UE not supporting the capability, step 2 is necessary.

  - For a UE supporting the capability, step 2 can be skipped, and if the Step 2 is skipped.

## Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

The following were concluded from RAN4#113 meeting and there is no further progress in RAN4#114bis meeting.

<Agreement>

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and

    - It is applied for SCS = 60KHz

  - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

Regarding the [], we think it’s ok to remove it. Since within active BWP, no matter how many CSI-RS resources are configured the SCS and CP will all be the same. It is reasonable to use the current wording for the first 2 bullets.

Regarding the 3rd bullet on centre frequency, one problem for intra-frequency/inter-frequency definition of CSI-RS based L1 measurement is how to determine the reference centre since multiple CSI-RS resources may be configured for both serving cell and neighbour cell. We think the same logic as CA/DC can be reused here, e.g. a UE could have multiple intra-frequency neighbour cells as long as the centre of the CSI-RS resource to be measured is the same as any of the CSI-RS resources within active BWP.

Proposal 4: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

## Measurement on CSI-RS with repetition ON/OFF

The following was concluded from RAN4#114 meeting.

Candidate options:

  - Option 1: (CATT, CT, vivo)

In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. 
In FR2, RAN4 will define the requirements for the following case: 
CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

  - Option 2: (Huawei)

Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

  - Option 3: (Nokia)

Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

  - Option 4: (Ericsson, Apple, Nokia)

RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

For L1 measurement with repetition OFF, beam training on CSI-RS resources can be avoided and the L1-RSRP measurement is only performed after UE has done SSB based L1-RSRP measurement or UE reported the capability that it is capable of skipping SSB based L1-RSRP measurement. For L1 measurement with repetition ON, there seems no clear conclusion in RAN1 whether CSI-RS with repletion ON can be supported for L1 measurement for mobility. We think RAN4 can prioritize L1 measurement with repetition OFF for RAN4 requirement discussion at this stage and wait for RAN1 agreement on L1 measurement with repetition ON.

Proposal 5: RAN4 prioritize L1 measurement with repetition OFF for RAN4 requirement discussion at this stage and wait for RAN1 agreement on L1 measurement with repetition ON.

## CSI-RS resource type

The following options are open for further study.

#### Issue 4-8: CSI-RS resource type

Candidate options:

  - Option 1: (Apple, OPPO, Nokia, vivo)

RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision.

  - Option 2: (CMCC, Ericsson)
Semi-persistent CSI-RS resource for LTM candidate cells is supported.

  - Option 3: (CATT)

RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

Besides periodic CSI-RS, there seems also interests to cover aperiodic and semi-persistent CSI-RS when specifying requirements. RAN1 has concluded semi-persistent CSI-RS.

Proposal 6: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

## Supported measurement type & RTD assumption

<Agreement>

- Option 1: CMCC, ZTE, CATT, China Telecom

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- Define requirements for RTD > CP, as optional UE feature. FFS details

- Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No Ran4 requirements for RTD > CP.

- Option 3:

- Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

- Define requirements for support RTD > CP, as optional UE feature. FFS details

- Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise

- Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

- No Ran4 requirements for RTD > CP.

The following was concluded for further discussion.

There has been analysis on the benefits and complexity to support measurement with GAP and RTD>CP in the contributions from companies. These options were motivated by how to trade-off between measurement gap and RTD >CP assumption since UE vendors prefer to skip both while operators think the feature should be compete as minimum. We are fine to use option 2 as compromise to move forward.

Proposal 7: Option 2 is adopted as compromise. E.g.

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No Ran4 requirements for RTD > CP.

<Candidate solutions>

- Option 1: In Rel-19 LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement. (CATT)

- Option 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer. (CATT)

- Option 3: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements. (CATT)

- Option 4: RAN4 to further discuss type of MG for the CSI-RS based L1-RSRP measurements. (Ericsson)

Measurement gap related

RAN4 to further consider whether to consider the case that GAP is needed at first in section 2.7. If needed, we think option 1 is reasonable, e.g. in Rel-19, LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement at minimum. We are also open for further discussion on other GAP type, e.g. concurrent GAP and/or preconfigured MG.

Proposal 8: If GAP is needed, then consider type 1 GAP as minimum. Other GAP types are FFS.

## Definition of “inside/outside active BWP

#### Issue 4-5: Definition of “inside/outside active BWP”

Candidate options:

  - Option 1: (Apple)

The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP.

  - Option 2: (CATT)

RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

  - Option 3: (vivo)

From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

In the last meeting, we propose that the CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP and it is outside of active BWP otherwise. After further check 133 spec, it seems we could relax a little bit. The current requirement for CSI-RS based L3 measurement apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. It was also agreed to reuse the same CSI-RS configuration for L1 measurement and we assume same accuracy requirement is applicable. In this sense, it is reasonable to assume that as long as 48RBs are within active BWP, the L1 measurement requirement can be satisfied. Hence, it is proposed that a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Proposal 9: A CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

#### Issue 4-11: Definition of frequency layer

Candidate options:

  - Option 1: (Apple)

Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

  - Option 2: (MTK)

If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

  - Option 3: (Huawei)

If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer.

  - Option 4: (Nokia)
Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity)

  - Option 5: (Ericsson)

Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

Definition of frequency layer

First of all, we think it’s ok to focus on the same SCS and CP types. E.g. do not define RRM requirements for the case where target CSI-RS to be measured are with different SCS and CP type. But we still need to discuss the case when centre frequency is different and the case when centre frequency is the same but not fully confined within active BWP. We propose to focus on supported intra-frequency/inter-frequency definition, measurement type and RTD assumption to be supported before we start this discussion.

If both measurement with GAP and RTD>CP are not considered, all the measurement can be done with active BWP, maybe we don’t need frequency layer definition. For measurement period and measurement capaibilty definition, we can introduce number of cells or number of resources…

Proposal 10: Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition, measurement type and RTD assumption are concluded.

#### Issue 4-12: Whether to consider CSI-RS based L1-SINR measurement

Candidate options:

  - Option 1: (CATT, Apple)

RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

  - Option 2: (ZTE, CT)

Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

Whether to consider CSI-RS based L1-SINR measurement

After checking with RAN1 colleagues, When RAN1 concludes with a text like, “there is no consensus in RAN1,” it means that the feature is not supported by RAN1 rather than further discussion is needed. Therefore, RAN4 should not discuss this matter any more as RAN1 intended not to support it.

Proposal 11: RAN4 does not need to develop RRM requirements for CSI-RS based L1-SINR measurement.

## CSI-RS accuracy requirements

#### Issue 4-14: CSI-RS accuracy requirements

Candidate options:

- Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (Apple, ZTE)

Proposal 12: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

## LS to RAN1

LS to RAN1 on intra-frequency/inter-frequency definition is needed if proposal 5 is agreed.

Proposal 13: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

-	It is applied for SCS = 60KHz

-	the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# Conclusion

This contribution proposed our consideration on open issue for CSI-RS based L1 measurement. The following proposals are concluded.

Proposal 1: confirm the tentative agreement “Use measurable and specify the side condition for RS being measurable”.

#### Proposal 2: it is proposed to use detectable and legacy side condition of SINR=-3dB.

Proposal 3: Whether step 2 can be skipped or not can be based on UE capability reporting.

  - For a UE not supporting the capability, step 2 is necessary.

  - For a UE supporting the capability, step 2 can be skipped, and if the Step 2 is skipped.

Proposal 4: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

Proposal 5: RAN4 prioritize L1 measurement with repetition OFF for RAN4 requirement discussion at this stage and wait for RAN1 agreement on L1 measurement with repetition ON.

Proposal 6: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

Proposal 7: Option 2 is adopted as compromise. E.g.

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No Ran4 requirements for RTD > CP.

Proposal 8: If GAP is needed, then consider type 1 GAP as minimum. Other GAP types are FFS.

Proposal 9: A CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Proposal 10: Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

Proposal 11: RAN4 does not need to develop RRM requirements for CSI-RS based L1-SINR measurement.

Proposal 12: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

Proposal 13: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

-	It is applied for SCS = 60KHz

-	the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# Reference

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102

[2] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis

[3] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, RAN4#113

[4] R4-2502222, (NR_Mob_Ph4-Core) CSI-RS based L1 measurement for LTM, Qualcomm