3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503596

Wuhan, China, 7th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand for Rel-19 NES

Agenda item:	7.25.2.3

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized … - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of SIB1 for Rel-19 NES.

Discussion

In previous meeting, RAN4 agreed to discuss potential RRM impact for OD-SIB1 as follows[2]:

Sub-topic 2-1 RRM requirements impacts for OD-SIB1 Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

Paging interruption requirement

In previous meeting, some companies raised an issue regarding RRM impact for paging interruption due to on-demand SIB1 (OD-SIB1). The argument of proponent company is that additional paging interruption needs to be added, because more delay happens to be occured when UL WUS triggers OD-SIB1.

In 4.2.2.4 in TS38.133, interruption requirement is captured as below

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

According to current interruption requirement, the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms, where the term TSI-NR is the total required time for receiving all the relevant system information data, which can be interpreted as it includes OD-SIB1 procedure. As stated in the spec, actual time required for gather all the system information refers to TS38.331. Hence, the component or total time of TSI-NR is related to RAN2, not RAN4. Other than TSI-NR, additional interruption time may not be necessary.

Proposal 1: Regarding paging interruption requirement, no RAN4 impact is identified

- Detail of TSI-NR is upto RAN2

Te requirement

Based on the agreement in RAN1/2 for OD-SIB1 prcedure, UL-WUS configuration is configured from Cell A, and the UE can transmits UL-WUS to NES Cell in order to receive OD-SIB1 from NES Cell. Regarding TA of UL-WUS, the parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Techinically, no additional specification effort is needed for Te requirement. The reason is as follows:

- Reference cell

  - Since NES cell transmits SSB, downlink timing can be provided

- NTA

  - Since the physical channel of UL-WUS is PRACH, it is natural to set NTA as 0

- NTA_offset

  - “n-TimingAdvanceOffset” is included in the UL-WUS configuration

As analyed above, technical issue is not identified from our perspective. However, if some companies want to update specification wording to avoid mis-interpretation or confusion, we are fine to update the wording.

Observation 1: Current Te requirement can be applied to OD-SIB1 procedure

Proposal 2: Fine to update wording of specification to avoid confusion

Conclusion

In this contribution, we provide views on RRM requireiments for OD-SIB1, and we propose

Proposal 1: Regarding paging interruption requirement, no RAN4 impact is identified

- Detail of TSI-NR is upto RAN2

Observation 1: Current Te requirement can be applied to OD-SIB1 procedure

Proposal 2: Fine to update wording of Te requirement to avoid confusion

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei