3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503598

Wuhan, China, 7th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of UE-initiated/event-driven beam management

Agenda item:	7.22.3.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 MIMO operation [1], following objective was captured:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we provide our views on RRM core requirements for UE-initiated/event-driven beam reproting

Discussion

Define measurement period in Event triggered measurement reporting delay

In the previous meeting, measurement period for reporting delay has been discussed, and following has been agreed:

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not preclude

Regarding M, it is related to parameter timeRestrictionForChannelMeasurement. In general, this parameter is used for providing report using the latest measurements, which is different scenario from UE-initiated/event-driven beam reporting. Therefore, we think M=1 can be used for measurement period in Event triggered measurement reporting delay.

Regarding the case of different periodicity of RS for new and current beam, we prefer Option A. When comparing current beam and new beam, periodicity of the RSs configured for the current beam and new beam can be different. Although RAN1 defiend that the periodicity of current beam and new beam are same in the same CC, still different periodicity can be configured, if current beam and new beam are in different CC. In order to compare the RSs with different periodicity, measurement periodicity needs to be the longer periodicity as baseline. In consequence, both current and new beam should be evaluated in the measurement period for comparison. In order to measure both RSs, the UE requires maximum periodicity of the two RSs. Option B seems to take into account one of the RSs based on the measurement location after the event.

Proposal 1: Define measurement period as follows:

- M=1

- TSSB = Max{TSSB_current, TSSB_new}

Reporting delay requirements

Issue 1-4: Components of Event triggered measurement reporting delay? <Way forward > - Option 1: (Apple, vivo, Samsung, E///, MTK) - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel - FFS on whether other components are needed. - FFS the definition of Tfirst UL channel - Option 2: (CMCC, Xiaomi) - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. - Option 3: (Nokia) - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

The one of main open issue is specifing reporting delay requirement. For the delay requirement RAN4 defined the start point as reusing L3 event triggered reporting delay requirement, which is “an event that will trigger a measurement report”. If Event-2 is applied, the event means comparing L1-RSRP of current and new beams. In pervious meeting, RAN4 agreed to measure both the current beam and new beam after the starting point.

Once the event is triggered, the UE reports the result of L1-RSRSP at the available UL resource, which depends on the configuration. Thus, we think UL uncertainty needs to be included in the delay requirement, because First PUCCH should  be transmitted within event triggered measurement reporting delay. Similar approach was already used in current requirement, e.g. clause 8.5.9 requirements for beam failure recovery in SCell. The definition of Tfirst UL channel can be the first available UL channel after the measurement period.

Proposal 2: RAN4 to specify reporting delay requirement as follows:

- Reporting delay = Measurement time + UL_uncertainty

  - UE should transmit First PUCCH within the reporting delay

  - UL_uncertainty is the first available UL channel after the measurement period

Figure 1 Reporting delay requirement

Time window based or cover both no time window based and with time window based

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > - Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M - Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS - Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. - Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

According to RAN1 agreement, Event 2 can be no-time-window-based, or time-window-based with configurable M. For measurement before event triggering, it is reasonable to count multiple instance to avoid ping-pong effect. On the other hand, during measurement period for evaluation, where the event is already triggered, the reason is not clear to fulfil multiple counts. What UE needs to do is to evaluate the current and new beam, and confirm, which requires a no-time-window-based measurement period. So we think specifying only no-time-window based is enough.

Proposal 3: Specify only no-time window based requirment

Whether to update L1 report which is measured in L1 report?

Issue 1-7: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time? <Way forward > - Option 1: (Xiaomi) - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. - Option 2: (LGE, Nokia) - Need to wait RAN1 progress - Option 3: (vivo) - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

When the event is triggered, UE is required to report at the available UL resource. Given the fact that there can be one or more measurement samples can be acquired before reporting, it is not clear whether L1 reporting can be updated based on the further measurements. We think after the event is triggered, further measurements are not required, and no need to update the reporting. In our understanding, it is discussed in RAN1, so it can be further discussed based on RAN1 progress.

Proposal 4: Whether to update L1 report which is measured in L1 report needs to wait further RAN1 progress

Whether to specify new unified TCI switching delay requirements?

<Way forward > - Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. - Option 2: (Samsung) - No unknown case is applied if the TCI states switching is based on UEIBM. - Option 3: (ZTE) - Need to decide whether to define L1-RSRP requirements for unknown cell Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

In the last RAN1#119 meeting, reducing beam application latency after a UE-initiated/event-driven beam reporting was discussed. The issue is to whether to reduce activated TCI state switching delay by removing additional SSB reception (Tfirst-SSB) for reducing the unified TCI state beam application time. In our understanding, Tfirst-SSB is for AGC tunning before receiving PDSCH/PDCCH, if target TCI state is not in the active TCI state list. However, we are not sure whether RAN1 consider those perspective. We think RAN4 needs to discuss whether active TCI state swiching delay can be reduced or not.

Proposal 5: For unified TCI switching delay requirements, RAN4 needs to wait further RAN1 progress

Conclusion

In this contribution, we provide views on RRM requireiments for UE-initiated/event-driven beam reproting, and we propose

Proposal 1: Define measurement period as follows:

- M=1

- TSSB = Max{TSSB_current, TSSB_new}

Proposal 2: RAN4 to specify reporting delay requirement as follows:

- Reporting delay = Measurement time + UL_uncertainty

  - UE should transmit First PUCCH within the reporting delay

  - UL_uncertainty is the first available UL channel after the measurement period

- Proposal 3: Specify only no-time window based requirment

Proposal 4: Whether to update L1 report which is measured in L1 report needs to wait further RAN1 progress

Proposal 5: For unified TCI switching delay requirements, RAN4 needs to wait further RAN1 progress

Reference

[1] RP-242394, “WID revision: NR MIMO Phase 5,” Samsung