3GPP TSG-RAN4 Meeting #114bis	R4-2503599

Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	UE RF requirements of LB carrier aggregation

Agenda item:	7.7.2

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirement for low band CA based on WD [1].

Discussion

The followings were approved in WF [1].

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release Issue 2: Switching period values Summary of company proposals (for information): 5us: 3 (Spreadtrum, Huawei, Ericsson) 13us, assuming no LO retuning: 2 (Ericsson, Huawei) 35us: 6 (Huawei, Qualcomm, ZTE, CATT, Spreadtrum, Xiaomi) 70us: 1 (Samsung) 100us: 2 (Apple, Murata) 140us: 9 (Apple, Qualcomm, ZTE, CATT, MediaTek, Spreadtrum, Samsung, Xiaomi, Murata) 140 + FFS us: 1 (Skyworks) 200us: 1 (Google) 210us: 4 (Qualcomm, ZTE, Samsung, Xiaomi). - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded Issue 5: Switching period location - Discuss the switching period location based on the following options: - Option 1: Switching period location is entirely up to RAN1 design - Option 2: Switching period location is defined relative to the slot boundary - Other options are not precluded Issue 6: UE reference architecture - Companies are encouraged to contribute proposals on the UE reference architecture as a basis for developing RF requirements for the feature Issue 7: Test methodology Companies are encouraged to contribute proposals related to the testability of the time mask requirement

Figure 2-1 shows the switching between {case1, case 2} with example band combination : CA_5A-n29A in [2].

Figure 2-1: switching between {case1 , case 2} : CA_5A-n29A

As seen in Figure 2-1, the carrier switching from Tx/Rx in n5 to Rx in n29 needs to be considered newly compared with the following existing UL switching in NR inter-band CA.

Existing UL switching in NR inter-band CA.

- 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

  - 6.3A.3.3.1	General

  - 6.3A.3.3.2	Time mask for switching between two uplink carriers

  - 6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

  - 6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

  - 6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors

  - 6.3A.3.3.6	Time mask for switching across up to four uplink bands

  - 6.3A.3.3.6a	Additional requirements for three-band switching with dual TAG

In the existing uplink switching for inter-band CA, the following UE capability, RRC signalling, and time mask requirement can be considered as starting point for carrier switching.

- UE capability

  - ULTxSwitchingBandPair

    - bandIndexUL1, bandIndexUL2

    - uplinkTxSwitchingPeriod

    - uplinkTxSwithing-DL-Interruption

- RRC signaling

  - UplinkTxSwitching

    - uplinkTxSwitchingPeriodLocation

    - uplinkTxSwitchingCarrier

- Time mask requirement, e.g., 6.3A.3.3.2

Issue 2: Switching period values

On top of the agreement (35us, and 140us), other value needs to be further discussed.

For 13us, it was proposed by assuming no LO retuning. It seems be not a general case. And, according to [2], only SCS 15kHz for both carriers is considered. In this case, one symbol duration including CP is 71.35 us. Both switching periods of 35us and 13 us are smaller than the symbol duration, so it may occur at least one symbol interruption. On the other hand, the switching period of 140us may occur at least two symbol interruptions.

Therefore, if benefit is not significant with the switching period of 13us, it is preferred to keep the current agreement (35us and 140us).

Proposal 1: Not consider the switching period of 13us if the benefit is not significant.

Issue 3: Switching period capability

It needs to be defined similar as the existing UE capability on ULTxSwichingBandPair.

Proposal 2: Define the switching period capability per BC (BandPair).

Issue 4: Time mask for switching

It needs to be defined similar as the existing time mask for UE UplinkTxSwitching. Option 1 is preferred among 3 Options.

Proposal 3: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)  (Option 1).

Issue 5: Switching period location

It needs to be defined similar as the existing switching period location for UE UplinkTxSwitching. Option 2 is preferred among 2 Options.

Proposal 4: Switching period location is defined relative to the slot boundary (Option 2)

Figure 2.2 shows one example on time mask considering switching period location.

(a): Switching period location in n5 DL carrier

(b): Switching period location in n29 SDL carrier

Figure 2-2: Time mask for carrier switching between {case1 , case 2} : CA_5A-n29A

Proposal 5: Consider Figure 2.2 as a starting point for carrier switching time mask.

Conclusion

In this contribution, we proposed our view on UE RF requirement for low band CA.

Proposal 1: Not consider the switching period of 13us if the benefit is not significant.

Proposal 2: Define the switching period capability per BC (BandPair).

Proposal 3: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)  (Option 1)

Proposal 4: Switching period location is defined relative to the slot boundary (Option 2)

Proposal 5: Consider Figure 2.2 as a starting point for carrier switching time mask.

- References

[1] R4-2503016, “WF on low-low band switching”, Apple

[2] RP-243315, “New WID on low band carrier aggregation via switching”, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE