3GPP TSG-RAN WG4 Meeting # 114bis	                                              R4-2503605

Wuhan, CN, April 7 – 11, 2025

Agenda Item:      7.27.2.3

Source:	MediaTek Inc.

Title:	Discussion on conditional LTM

Document for:	Discussion

- Introduction

In this contribution, we provide our views on conditional LTM.

- Discussion

2.1 Conditional cell switch delay and interruption

Last meeting, RAN4 reached the following progress for conditional cell switch delay and interruption requirements.

Issue 1-3-3: The components of conditional LTM delay < Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

In our understanding, the delay and interruption requirements for CHO are defined as Fig. 1. The interruption time includes Tprocessing, time for T/F tracking and UL sync. In LTM, early T/F tracking is introduced to reduce both the cell switch delay and interruption. In our understanding, if UE supports early T/F tracking, it is also possible to perform T/F tracking before Tprocessing. As well known, the time introduced by T/F tracking is mainly spent on waiting for the SSB occasion. The waiting time also contributes to the interruption. If UE performs T/F tracking before Tprocessing, the delay is not reduced, but the interruption will be reduced as shown in Fig. 2. In C-LTM, source cell does not know when UE will start to switch to the target cell, so during the cell switch procedure, source cell may keep scheduling the UE. The shorter the interruption is, the better.

Fig.1

Fig. 2

For C-LTM, we propose the delay and interruption as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS and TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU, where

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional cell switch command until a condition exists at the measurement reference point which will trigger the conditional cell switch,

- Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled.

  - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

  - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

  - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1.

  - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15.

- TLTM-RRC-processing  can be up to 10ms.

- TLTM-processing is time for UE processing. Tprocessing can be up to 20 ms.

- TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

- Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

- TRS is time for fine time tracking and acquiring full timing information of the target cell.

  - Theoretically, TRS is  the time needed to get ready to receive SSB on target cell plus the SSB duration. For SSB to be measured within MG, RF retuning time is needed. The SSB duration is less than 1 slot. Most of the components in HO/cell switch delay are defined in the unit of ms. We understand that the time needed for intra-frequency case can be shorter. To make the spec simpler, we prefer to define a unified value of TRS for all the scenarios. Here we propose TRS as 2ms.

- Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

Here we include “TRS + TRS-proc” into interruption time due to there will be interruption on source cell to receive SSB on target cell, e.g.,

- Inter-f: UE needs to retune to another frequency

- Intra-f in FR2: Rx beam may be different from source cell

- Intra-f in FR1 with mixed numerology and UE incapable of mixed numerology

As the optimization of UE performing T/F tracking before Tprocessing has reduced the average interruption time significantly, we prefer to define a unified requirements for all the scenarios for the sake of spec simplification and readability.

Proposal 1: For C-LTM, conditional cell switch delay and interruption can be defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS and TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU, where

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional cell switch command until a condition exists at the measurement reference point which will trigger the conditional cell switch,

- Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled.

  - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

  - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

  - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1.

  - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15.

- TLTM-RRC-processing  can be up to 10ms.

- TLTM-processing is time for UE processing. Tprocessing can be up to 20 ms.

- TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

- Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

- TRS is time for fine time tracking and acquiring full timing information of the target cell. TRS can be up to 2ms.

- Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

2.2 UE based TA measurement

Last meeting, RAN4 discussed whether to define requirements for UE based TA measurement. The mechanism is to derive TA of neighbor cell based on TA of serving cell and RTD between serving cell and neighbor cell as shown in Fig.2. Considering that serving cell and neighbor cell are not ideally synchronized in the field, there will be some synchronization error (denoted as y). Then TA2=TA1+2*RTD = TA1+2*(y+TP2-TP1).

Fig. 2

The errors, for each component should be considered, i.e.,

- TA1

Consider the errors: UL to DL error + TA resolution error + TA adjustment accuracy

- TP2:

Consider DL timing error

- TP1:

Consider DL timing error

The total error is UL to DL error + TA resolution error + TA adjustment accuracy + 2*(y+2* DL timing error) = UL to DL error +DL timing error + TA resolution error + TA adjustment accuracy +2y +3* DL timing error = Te + TA resolution error + TA adjustment accuracy +2y +3* DL timing error. To make sure the degradation at gNB reception is acceptable, the total error should be less than 1/2 CP, which is used as a criterion to define UE based TA measurement accuracy requirements in FR2 HST. Then it can be derived the synchronous requirement on serving cell and neighbor cell under different numerologies as below:

| SCS of SSB (kHz) | SCS of UL signals (kHz) | 1/2 CP of UL signals (64*Tc) | Te | TA resolution error 16*64*Tc/2u | TA adjustment accuracy 38.133 Table 7.3.2.2-1 | DL timing error 1/(2*BW) | y (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 71.05 | 12*64*Tc | 16*64*Tc | 256 Tc | 139ns | 427.1 |
| 30 | 30 | 35.55 | 8*64*Tc | 8*64*Tc | 256 Tc | 69.4ns | 149.0 |
| 120 | 120 | 8.9 | 3.5*64*Tc | 2*64*Tc | 32 Tc | 17.4ns | 21.1 |

After calculation, it can be observed that only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

As the condition to use UE based TA measurement is too tight for almost all the cases, we prefer not to define the related requirements.

In addition, we don’t think the accuracy requirements defined for one shot large UL timing adjustment for FR2 HST can be applied directly, where at least two shots of T/F tracking or TRS based T/F tracking on the target TRP is needed. If to define the requirements, at least the following should be discussed:

- accuracy requirements of the estimated TA at UE side

- synchronization requirements between source cell and target cell

- Timeline to perform UE based TA measurement

  - We don’t think UE can start to perform UE based TA measurement at RRC configuration just following the signalling designed in R18 LTM to meet acceptable accuracy.

- The impact on cell switch delay requirements

  - Similar as the discussion on the conditions T/F tracking can be skipped during cell switch delay, the condition that TA measured at UE is valid should be discussed. If the measured TA is not valid, UE behaviour should be discussed further, e.g., measure TA during cell switch delay or fall back to RACH-based cell switch.

Considering the above work to do and there are only three meetings left, we propose not to define the requirements for UE based TA measurement.

Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19.

- Conclusion

In this contribution, we provide our views for conditional LTM and have the following proposals and observation.

Proposal 1: For C-LTM, conditional cell switch delay and interruption can be defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS and TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU, where

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional cell switch command until a condition exists at the measurement reference point which will trigger the conditional cell switch,

- Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled.

  - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

  - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

  - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1.

  - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15.

- TLTM-RRC-processing  can be up to 10ms.

- TLTM-processing is time for UE processing. Tprocessing can be up to 20 ms.

- TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

- Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

- TRS is time for fine time tracking and acquiring full timing information of the target cell. TRS can be up to 2ms.

- Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19.